3GPP TSG-RAN WG4 Meeting #88               	R4-1809955

Gothenburg, Sweden, 20 Aug – 24 Aug 2018

Agenda item:	7.13.2.4

Source:	Samsung

Title:	Initial simulation results for NR PRACH

Document for:	Discussion

# 1	Introduction

During RAN4 AH07 meeting, the NR BS demodulation performance requirements are further discussed on the general issue and performance requirements test. Some general issue about NR PRACH performance requirements are captured and agreed in the WF [1] as follow:

- There are no test cases for restricted sets in Rel-15

- Preamble Format (For Rel-15)

  - For long sequence, requirements are defined for

- Format 0

- FFS for Format 1 and Format 3

  - For short sequence, the following formats will be considered, but further down selection is needed

- A1, A2, A3,B4,C0 and C2

- SCS

  - 15 KHz

  - 30 KHz

  - 60 KHz (FR2)

  - 120 KHz

- Apply the performance requirements based on BS declare

- Simulation Assumption

  - SCS and Bandwidth

- 15kHz, 10MHz,

- 30kHz,40MHz

- 60kHz(FR2), 100MHz

- 120kHz, 100MHz

  - Channel

- AWGN,

  - Antenna Configuration

- 1TX, 2Rx

  - Ncs

- Format 0: Ncs =[13] and v=22 (for simulation alignment)

- Short sequence: FFS  Ncs, v=[0]

In order to facilitate simulation alignment of NR BS PRACH performance between each company, some test cases are introduced for initial alignment. In this contribution, the initial simulation results are provided for alignment.

# 2	Simulations

In this section, based on our preferred, the initial simulation results for NR PRACH are provided as shown in Table 2. For FR1, the simulation results of Format 0, A1 and C2, are provided, For FR2, the simulation results of Format A1 and C2 are provided.

As for the values of Ncs’s logical sequence index, and v, we reuse the value in LTE for format 0. For format C2 and A1, we prefer no cyclic shift for testing. As for the value of logical sequence index and v, we reuse the value in LTE for format4. Generally, the simulation assumption is summarized in Table 1 and Table 2 for FR1 and FR2, respectively.

Table 1: Simulation assumption for NR PRACH Format0, Format A1 and Format C2 in FR1

| Parameters | Value for Format 0 | Value for Format A1 | Value for Format C2 |
| --- | --- | --- | --- |
| Bandwidth | 10MHz, 40MHz | 10MHz, 40Mhz | 10MHz, 40Mhz |
| SCS | 1.25KHz | 15KHz, 30KHz | 15KHz, 30KHz |
| Rx | 2 | 2 | 2 |
| Propagation channel | AWGN | AWGN | AWGN |
| Logical sequence index | 22 | 0 | 0 |
| Preamble index | 32 | 0 | 0 |
| Ncs | 13 | 13 | 0 |
| Set type | Unrestricted | Unrestricted | Unrestricted |

Table 2: Simulation assumption for NR PRACH Format 0 in FR1 and Format C2 in FR2

| Parameters | Value for Format A1 | Value for Format C2 |
| --- | --- | --- |
| Bandwidth | 100MHz | 100MHz |
| SCS | 60KHz, 120KHz | 60KHz, 120KHz |
| Rx | 2 | 2 |
| Propagation channel | AWGN | AWGN |
| Logical sequence index | 0 | 0 |
| Preamble index | 0 | 0 |
| Ncs | 0 | 0 |
| Set type | Unrestricted | Unrestricted |

As for test metric, we follow the legacy way defined in LTE. Both the false alarm probability and missed detection probability are given as the performance requirement for NR PRACH. The initial simulation results can be referred as Table 3. The performance curve can be referred in appendix.

Table3: Simulation results for NR PRACH format 0 in FR1 and Format C2 in FR2 (to be updated)

| Format | SCS&BW | Missed detection probability (<=1%) |
| --- | --- | --- |
| Format 0 | 1.25KHz, 10MHz | -16.44 |
| Format 0 | 1.25KHz, 40MHz | -16.48 |
| Format A1 | 15KHz, 10MHz | --10.96 |
| Format C2 | 15KHz, 10MHz | -13.02 |
| Format A1 | 30KHz, 40MH | -10.98 |
| Format C2 | 30KHz, 40MHz | -13.04 |
| Format A1 | 60KHz, 100MHz |  |
| Format C2 | 60KHz, 100MHz |  |
| Format A1 | 120KHz, 100MHz |  |
| Format C2 | 120KHz, 100MHz |  |

For this table, we can observe that the performance requirement has about 2dB margin compared with the performance requirement LTE format 0 defined in 36.104. While for the result of format C2, it is similar with LTE format 4 with 8Rx, where sequence length of format 4 is 139. In terms of energy, C2 consist of four preambles with 2Rx, can be comparable with LTE format 4 with 8Rx. Since current only AWGN is considered, there is no frequency offset, the performance with different SCS and BW in same format should be similar.

# 3	Conclusion

In this contrition, based on our preferred, the initial simulation results of NR PRACH are provided for alignment.

# References

R4-1809369, WF on NR PRACH demodulation requirement, Ericsson

# Appendix

- Figure 1:  Performance of NR PRACH format 0 for FR1

- Figure 1:  Performance of NR PRACH format C2 for FR1

- Figure 2:  Performance of NR PRACH format A1 for FR1