3GPP TSG-RAN WG4 #88 Meeting                                     R4-1809988

Gothenburg, Sweden, August 20-24, 2018

Agenda item:	7.13.2.3

Source: 	ZTE

Title: 	On NR BS PUCCH demodulation performance requirements

Document for:	Discussion

Introduction

During RAN4#AH-1807 meeting in July two WFs [1][2] relevant for NR BS PUCCH demodulation performance requirements and simulation assumptions for alignment purpose have been approved. This contribution presents the simulation results.

Initial simulation assumptions and results

Here are the simulation assumptions for FR1 conducted and FR2:

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | FR1 conducted | FR2 |  |
| Number of Tx | 1 | 1 |  |
| Number of Rx | 2, 4, 8 | 2 |  |
| Carrier frequency (GHz) | 4 | 30 |  |
| Propagation channel | AWGN | AWGN |  |
| Waveform | CP-OFDM | CP-OFDM |  |
| SCS@CBW | 30 kHz @ 40 MHz | 120 kHz @ 100 MHz |  |
| Modulation | QPSK | QPSK |  |
| Cyclic prefix | Normal | Normal |  |

- Initial simulation results for FR1 conducted

The initial simulation results are summarized in the table below.

Initial simulation results for FR1 conducted single user PUCCH format 0:

1Tx, 1 bit, 1 OFDM symbol, 1 PRB, Missed Ack probability < 1%, DTX to Ack probability <1%

| Number of Tx | Number of Rx | Cyclic Prefix |  | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 kHz@40 MHz |  |  |  |
| 1 | 2 | Normal |  | AWGN |  |  | 2.2 (DTX on) -1.6 (DTX off) |  |  |  |
|  | 4 | Normal |  | AWGN |  |  | -0.4 (DTX on) -3.5 (DTX off) |  |  |  |
|  | 8 | Normal | AWGN |  |  |  | -2.8 (DTX on) -5.5 (DTX off) |  |  |  |

Initial simulation results for FR1 conducted single user PUCCH format 0:

1Tx, 1 bit, 2 OFDM symbol, 1 PRB, Missed Ack probability < 1%, DTX to Ack probability <1%

| Number of Tx | Number of Rx | Cyclic Prefix |  | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 kHz@40 MHz |  |  |  |
| 1 | 2 | Normal |  | AWGN |  |  | -0.2 (DTX on) -3.2 (DTX off) |  |  |  |
|  | 4 | Normal |  | AWGN |  |  | -2.8 (DTX on) -5.5 (DTX off) |  |  |  |
|  | 8 | Normal | AWGN |  |  |  | -4.8 (DTX on) -7.4 (DTX off) |  |  |  |

Simulation results for FR1 conducted single user PUCCH format 2:

1Tx, 4 bits, 1 OFDM symbol, 4 PRBs, Missed Ack probability < 1%, DTX to Ack probability <1%

| Number of Tx | Number of Rx | Cyclic Prefix |  | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 kHz@40 MHz |  |  |  |
| 1 | 2 | Normal |  | AWGN |  |  | -1.7 (DTX on) -1.7 (DTX off) |  |  |  |
|  | 4 | Normal |  | AWGN |  |  | -3.6 (DTX on) -4.0 (DTX off) |  |  |  |
|  | 8 | Normal | AWGN |  |  |  | -5.6 (DTX on) -5.9 (DTX off) |  |  |  |

Simulation results for FR1 conducted single user PUCCH format 3:

1Tx, 16 bits, 14 OFDM symbol, 1 PRB, Missed Ack probability < 1%

| Number of Tx | Number of Rx | Cyclic Prefix |  | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 30 kHz@40 MHz |  |  |  |
| 1 | 2 | Normal |  | AWGN |  |  | -4.2 (With/without additional DMRS, DTX on) |  |  |  |
|  | 4 | Normal |  | AWGN |  |  | -6.4 (With/without additional DMRS, DTX on) |  |  |  |
|  | 8 | Normal | AWGN |  |  |  | -8.5 (With/without additional DMRS, DTX on) |  |  |  |

- Initial simulation results for FR2

The initial simulation results are summarized in the tables below.

Simulation results for FR2 single user PUCCH format 0:

1Tx, 1 bit, 1 OFDM symbol, 1 PRB, Missed Ack probability < 1%, DTX to Ack probability <1%

| Number of Tx | Number of Rx | Cyclic Prefix | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 120 kHz@100 MHz |  |  |  |
| 1 | 2 | Normal | AWGN |  |  | 2.5 (DTX on) -1.2 (DTX off) |  |  |  |

Simulation results for FR2 single user PUCCH format 2:

1Tx, 4 bits, 1 OFDM symbol, 4 PRBs, Missed Ack probability < 1%

| Number of Tx | Number of Rx | Cyclic Prefix | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 120 kHz@100 MHz |  |  |  |
| 1 | 2 | Normal | AWGN |  |  | -2.25 (DTX off) |  |  |  |

Conclusions

The simulation results are summarized in the tables.

# References

- R4-1809370 Way forward on General part of NR BS demodulation performance, July 2018.

- R4-1809556 Way forward on NR PUCCH demodulation performance in Rel-15, July 2018.