3GPP TSG-RAN WG4 #88 Meeting                                     R4-1809989

Gothenburg, Sweden, August 20-24, 2018

Agenda item:	7.13.2.4

Source: 	ZTE

Title: 	On NR BS PRACH performance requirements

Document for:	Discussion

Introduction

During RAN4#AH-1807 meeting in July two WFs [1][2] relevant for NR PRACH performance requirements and simulation assumptions for alignment purpose have been approved. This contribution presents the simulation results.

Simulation parameters and initial simulation results

- FR1 conducted

For AWGN channel and SCSs 15/30 kHz @ 40 MHz, the simulation results have been presented in July meeting in [3].

- FR2

Here are the simulation parameters for FR2:

| Parameter (FR2) | Value |
| --- | --- |
| Format/SCS/CBW combinations | Format A3: SCS 60 & 120 kHz Format B4: SCS 60 & 120 kHz Format C2: SCS 60 & 120 kHz |
| Tx | 1 |
| Rx | 2 |
| Channel condition | AWGN |
| Frequency offset | 0 |
| NCS | Format A3: Ncs = 0, v=0 Format B4: Ncs = 0, v=0 Format C2: Ncs = 0, v=0 |
| Phase noise | Not modelled |

The initial simulation results of SNR for missed detection probability < 1% for FR2 are summarized in the table below. The 1 dB difference between SCS 60 kHz and SCS 120 kHz is due to different down sampling rates.

Table 1: Simulation results for FR2: SNR (dB) for missed detection probability < 1%

| Number of TX | Number of RX | Propagation conditions | Frequency offset | SNR [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Format A3 (NCS=0, v=0) | Format B4 (NCS=0, v=0) | Format C2 (NCS=0, v=0) |  |
| 1 | 2 | AWGN | 0 |  | -14.1 (SCS 60) -13.1 (SCS 120) | -17.1 (SCS 60) -16.1 (SCS 120) | -12.3 (SCS 60) -11.2 (SCS 120) |  |

Conclusions

The FR2 simulation results are summarized in Table 1. The FR1 simulation results have been presented in July meeting in [3] for AWGN channel and SCSs 15/30 kHz @ 40 MHz.

# References

- R4-1809370 Way forward on General part of NR BS demodulation performance, July 2018.

- R4-1809369 Way forward on NR PRACH demodulation requirements, July 2018.

- R4-1808619 On NR BS PRACH performance requirements, ZTE, July 2018.