3GPP TSG-RAN WG4 Meeting #88	R4-1810024

# Gothenburg, Sweden, August 20 – 24 , 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on updating interruption requirement for BWP switching in TS38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | BWP switching on one serving cell will cause interruption in other serving cells in EN-DC or NR CA.Currently the corresponding interruption requirements are not comlpeted in EN-DC and are not introduced for NR CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the interruption requirement for BWP switching for EN-DC and introduce the interruption requirement for BWP switching for NR-CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirement for BWP switching is not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, 8.2.1.2.7, 8.2.2.1, new 8.2.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

8.2.1.1	Introduction

This section contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a downlink BWP and/or an uplink BWP is switching in PSCell or any SCell

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This section contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in [15].

----------------------NEXT CHANGE----------------------------

8.2.1.2.7	Interruption due to Active BWP switching Requirement

When UE receives a DCI indicating UE to switch its NR active BWP, the UE is allowed to cause interruption of up to X4 slots to other active serving cells if the UE is not capable of per-FR gap. Otherwise the UE is allowed to cause interruption of up to X4 slots to other active serving cells in the same FR range wherein the UE is performing BWP switching. X4 is defined in Table 8.2.1.2.7-1. The interruption is only allowed within the BWP switching delay as defined in clause 8.6.2 in TS38.133.

When a BWP timer  bwp-InactivityTimer expires, UE is allowed to cause interruption of up to X4 slots to other active serving cells if the UE is not capable of per-FR gap. Otherwise the UE is allowed to cause interruption of up to X4 slots to other active serving cells in the same FR range wherein the UE is performing BWP switching. X4 is defined in Table 8.2.1.2.7-1. The interruption is only allowed within the BWP switching delay as defined in clause 8.6.2 in TS38.133.

Table 8.2.1.2.7-1 : interruption length X4

|  | NR Slot length (ms) | Interruption length X4 slot |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

----------------------NEXT CHANGE----------------------------

8.2.2.1	Introduction

This section contains the requirements related to the interruptions on PCell and activated SCell if configured, when up to TBD SCells are configured, deconfigured, activated or deactivated, or when a downlink BWP and/or an uplink BWP is switching in PCell or any SCell.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

----------------------NEXT CHANGE----------------------------

8.2.2.2.5	Interruption due to Active BWP switching Requirement

The requirements in sub-clause 8.2.1.2.7 shall apply

----------------------END OF CHANGES----------------------------