3GPP TSG-RAN WG4 Meeting #88                     	R4-1810029

Gothenburg, Sweden, August 20th –24th, 2018

Agenda item:            7.13.1.5

Source:	MediaTek Inc.

Title:	Initial CSI simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation result of CSI

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of CSI performance. Table A.1, Table A.2, Table A.3 and Table A.4 (in Appendix) are the initial simulation assumption for CQI, PMI and RI respectively. Table A.5 contains other detailed parameter setting in the simulation

2.1 CQI

In this paper, we provide the simulation result of static CQI test.

Figure 2.1.1~Figure 2.1.4 are the simulation result (CQI report distribution) for Static CQI test 1 and test 2 with 2RX and 4RX. The SNR selection is 0dB for Test 1 and 6dB for Test 2. Table 2.1.1~Table 2.1.4 are the corresponding BLER at the test point for median CQI, median CQI+1 and median CQI-1

Observation 1: For static CQI, similar performance requirement as LTE can be applied with very stable test result

LTE requirement:

- Wideband CQI should be within the set {median CQI-1, median CQI, median CQI+1} for more than [90%] of the time

- The PDSCH BLER using the transport format indicated by median CQI-1 should be less than or equal to [0.1]. Furthermore, the PDSCH BLER using the transport format indicated by median CQI+1 should be greater than or equal to [0.1]

Figure 2.1.1 Static CQI – Test 1 2TX x 2RX Rank2 0dB

Table 2.1.1 Static CQI – Test 1 2RX BLER

| CQI | MCS | BLER |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 1 | 0 |
| 3 | 3 | 1 |

Figure 2.1.2 Static CQI – Test 1 2TX x 4RX Rank2 0dB

Table 2.1.2 Static CQI – Test 1 4RX BLER

| CQI | MCS | BLER |
| --- | --- | --- |
| 2 | 1 | 0 |
| 3 | 3 | 0 |
| 4 | 5 | 1 |

Figure 2.1.3 Static CQI – Test 2 2TX x 2RX Rank2 6dB

Table 2.1.3 Static CQI – Test 2 2RX BLER

| CQI | MCS | BLER |
| --- | --- | --- |
| 3 | 3 | 0 |
| 4 | 5 | 0 |
| 5 | 7 | 1 |

Figure 2.1.4 Static CQI – Test 2 2TX x 4RX Rank2 6dB

Table 2.1.4 Static CQI – Test 2 4RX BLER

| CQI | MCS | BLER |
| --- | --- | --- |
| 4 | 5 | 0 |
| 5 | 7 | 0 |
| 6 | 9 | 0.163 |

2.2 Wideband PMI

Figure 2.2.1~Figure 2.2.8 are the simulation result for PMI test 1 and test 2

Observation 2: For wideband PMI, similar performance requirement as LTE can be applied () with very stable test result

LTE requirement:

- 1-1.  is 60% of the maximum throughput obtained at using random precoding, and  the throughput measured at with precoders configured according to the UE reports.

- 1-2. is 70% of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

Observation 3: For wideband PMI, the difference between follow UE and random mode is larger in Rank1. The requirement for low rank can be higher than high rank

Figure 2.2.1 Wideband PMI – Test 1 8TX x 2RX Rank1

Figure 2.2.2 Wideband PMI – Test 1 8TX x 2RX Rank2

Figure 2.2.3 Wideband PMI – Test 1 8TX x 4RX Rank1

Figure 2.2.4 Wideband PMI – Test 1 8TX x 4RX Rank2

Figure 2.2.5 Wideband PMI – Test 2 8TX x 2RX Rank1

Figure 2.2.6 Wideband PMI – Test 2 8TX x 2RX Rank2

Figure 2.2.7 Wideband PMI – Test 2 8TX x 4RX Rank1

Figure 2.2.8 Wideband PMI – Test 2 8TX x 4RX Rank2

2.3 RI

TBD

# 3	Conclusions

In the contribution, we provide our simulation on Static CQI and wideband PMI. We have the following observations:

Observation 1: For static CQI, similar performance requirement as LTE can be applied with very stable test result

Observation 2: For wideband PMI, similar performance requirement as LTE can be applied with very stable test result

Observation 3: For wideband PMI, the difference between follow UE and random mode is larger in Rank1. The requirement for low rank can be higher than high rank

# 4	Reference

[1] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung

Appendix

Table A.1 Static COI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | FFS | FFS |
| Propagation channel | AWGN | AWGN | AWGN |
| Correlation and antenna configuration | 2Rx 2*2 with [1 j; 1 –j] 4Rx: 2*4 with [1 1 j j; 1 1 –j -j] | 2Rx 2*2 with [1 j; 1 –j] 4Rx: 2*4 with [1 1 j j; 1 1 –j -j] | 2Rx 2*2 with FFS |
| resourceConfigType | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset | [5/0] | [5/0] | [5/0] |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration | FFS | FFS | FFS |
| reportConfigType | periodic | periodic | periodic |
| Reporting periodicity and slot offset | FFS | FFS | FFS |
| Codebook Restriction | 010000 (fixed rank2) | 010000 (fixed rank2) | 010000 (fixed rank2) |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Table A.2 Fading COI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | FFS | FFS |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | FFS |
| Correlation and antenna configuration | 2Rx 2*2 4Rx: 2*4 [ULA High] | 2Rx 2*2 4Rx: 2*4 [ULA High] | 2Rx 2*2 FFS |
| resourceConfigType | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset | 5/0 | 5/0 | 5/0 |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration | FFS | FFS | FFS |
| reportConfigType | periodic | periodic | periodic |
| Reporting periodicity and slot offset | FFS | FFS | FFS |
| Codebook Restriction | 000001 | 000001 | 000001 |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Table A.3 Wideband PMI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | FFS | FFS |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | FFS |
| Correlation and antenna configuration | 8*2 8*4 [XP spatially High] | 8*2 8*4 [XP spatially High] | 2*2 FFS |
| resourceConfigType | aperiodic | aperiodic | aperiodic |
| CSI-RS/CSI-IM periodicity and slot offset | 5/0 | 5/0 | 5/0 |
| NZP CSI-RS ports | 8 port {3000…3007} | 8 port {3000…30007} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density | [CDM4,density 1 (row 9)] | [CDM4,density 1 (row 9)] | [FD-CDM2/density 1] |
| CSI-IM configuration | FFS | FFS | FFS |
| reportConfigType | aperiodic | aperiodic | aperiodic |
| Reporting periodicity and slot offset | FFS | FFS | FFS |
| (N1,N2) | (2,2) | (2,2) | N.A |
| (O1,O2) | (4,4) | (4,4) | N.A |
| CodebookMode | 1 | 1 | N.A |
| Codebook Restriction | FFS | FFS | FFS |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Rank | 1/2 | 1/2 | 1/2 |
| FRC | MCS4/MCS13 | MCS4/MCS13 | MCS4/MCS13 |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Table A.4 RI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 FDD) | Test 3 FR2 |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | FFS | FFS |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | FFS |
| Correlation and antenna configuration | 2Rx 2*2 4Rx: 4*4 [ULA Low ULA High] | 2Rx 2*2 4Rx: 4*4 [ULA Low ULA High] | 2Rx 2*2 FFS |
| resourceConfigType | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset | 5/0 | 5/0 | 5/0 |
| NZP CSI-RS ports | 2/4 | 2/4 | 2 |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density | 2 ports: [FD-CDM2/density 1] 4 ports: [FD-CDM2 density 1 (row 4)] | 2 ports: [FD-CDM2/density 1] 4 ports: [FD-CDM2 density 1 (row 4)] | [FD-CDM2/density 1] |
| CSI-IM configuration | FFS | FFS | FFS |
| reportConfigType | periodic | periodic | periodic |
| Reporting periodicity and slot offset | FFS | FFS | FFS |
| Codebook Restriction | FFS | FFS | FFS |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Table A.5 CSI simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| TDD UL/DL configurations | - FR1 TDD: 7D1S2U, S = 6D:4G:4U - FR2 TDD - Configuration 1: DDDSU, S=10D:2G:2U Configuration 2: DDSU, S=11D+3G |  |
| HARQ | No HARQ retransmission |  |