3GPP RAN WG4 Meeting #88	R4-1810065

Gothenburg, Sweden, August 20 – 24, 2018

Agenda item:	7.6.17.1

Source:	Apple Inc.

Title:	On EIS Spherical Coverage for FR2 UEs

Document for:	Approval

# Introduction

As the NR Work Item [1] approaches the Stage 3 completion target, the spherical coverage requirement for EIS in FR2 has not yet been finalized. This contribution examines the relevant background information and provides proposals to finalize the EIS spherical coverage requirement for PC3 UEs.

# Discussion

## Background

The peak EIS requirement for power class 3 UEs was agreed in RAN4 #86bis and the requirement captured in Section 7.3.2.3 of TS38.101-2 [2]. However, the EIS spherical coverage requirement has not yet been finalized by RAN4. Additionally, over the past few meetings, RAN4 has also discussed a beam correspondence requirement without a final outcome.

In previous RAN4 meetings, companies have addressed the EIS spherical coverage requirement and have argued that if a UE supports beam correspondence, EIS spherical coverage performance can be inferred from EIRP spherical CDF [3-4]. The discussions on EIS spherical coverage were therefore postponed till conclusion of beam correspondence and EIRP spherical coverage.

Discussions on the EIRP spherical coverage requirement for PC3 UEs concluded at RAN4#87 with agreement to define it at 50%-tile of EIRP CDF. The requirement is captured in Section 6.2.1.3 of TS38.101-2 [2]. In the same meeting, some companies proposed that the EIS spherical coverage requirement be defined analogously, i.e., the gap between 50%-tile of EIS CDF and minimum EIS is the same as the drop from the peak EIRP to the 50%-tile EIRP value [5].

In RAN#80, analysis in [6] identified a gap in applicability of the EIRP spherical coverage requirement to UEs supporting multiple FR2 bands as the requirement was defined for single band FR2 UEs. TSG RAN has recognized this gap by including the following objective in the exception sheet: “For PC3, spherical coverage requirements for UEs which support multiple FR2 bands” [7].

In this paper, we follow up the discussions on peak EIS and EIS spherical coverage and provide the technical justification to also extend the EIS spherical coverage requirements’ applicability to UEs supporting multiple FR2 bands.

## Analysis

As discussed in [3], EIS spherical coverage performance may be inferred from the EIRP CDF if the UE supports beam correspondence. Furthermore, direct testing of an EIS spherical performance requirement is time consuming as throughput vs. receive power needs to be measured across multiple test points.

The RF response of transmit and receive chains is inherently different [9]. However, for a UE that supports beam correspondence and the difference in transmit and receive responses is appropriately bounded, the EIRP spherical coverage provides a good approximation of the EIS spherical coverage performance. This eliminates the need for EIS spherical coverage testing. Consequently, we make the following observation and proposals.

Observation 1: EIS spherical coverage performance may be inferred from the EIRP CDF if a UE supports beam correspondence.

Proposal 1: A separate EIS spherical coverage requirement or test is not required for UEs supporting beam correspondence and meeting the EIRP spherical coverage requirement.

We note that if a UE does not support beam correspondence, RAN4 should decide if an EIS spherical coverage requirement needs to be defined.

Proposal 2: If a UE does not support beam correspondence, RAN4 should decide if an EIS spherical coverage requirement needs to be defined.

In the event that RAN4 decides to define a spherical EIS requirement, we propose that it be defined at the 50%-tile CDF value, analogous to the agreements reached for EIRP spherical coverage.

Proposal 3: Define the EIS spherical coverage requirement in FR2 at 50%-tile value of its CDF.

In defining the EIS spherical coverage requirement, we have identified additional parameters, such as form factor de-sense, extreme temperature conditions, and supporting multiple bands, that can impact the EIS as discussed below. These factors should also be considered in deriving the EIS requirement in the beam peak direction. However, since RAN4 has already reached agreement on beam peak EIS we do not suggest revisiting this value.

Observation 2: Parameters such as form factor de-sense, extreme temperature conditions, and supporting multiple bands can impact EIS.

Proposal 4: Consider impact of form factor de-sense and extreme temperature conditions in defining the spherical EIS requirement.

### Form Factor De-sense

As mentioned earlier, the requirement on beam peak EIS was agreed in RAN4#86bis. During the discussions, some companies expressed concern about the impact of form-factor de-sense on the beam peak EIS value [10]. The beam peak EIS requirement was however agreed without considering the impact of form factor de-sense [11]. Based on our analysis, we propose a 2dB relaxation to account for form factor de-sense.

Proposal 5: Incorporate a 2 dB relaxation for form factor de-sense in the EIS spherical coverage requirement.

### Extreme Thermal Condition

The UE in FR2 is required to work in extreme temperature conditions per TS 38.101-2 [2]. We note that the difference in noise floor between -10C and +55C is about 1 dB using the equation below.

We therefore suggest to consider 1 dB for noise floor increase to address the extreme temperature conditions.

Proposal 6: Consider a 1 dB relaxation in the EIS spherical coverage requirement to address the noise floor increase at extreme temperature conditions.

### Multi-band Support

Analysis in [6] identified a gap in the applicability of the EIRP spherical coverage requirement to UEs supporting multiple FR2 bands as the requirement is only applicable to UEs which support single FR2 band. Addressing this gap, TSG RAN included the following objective in the exception sheet: “For PC3, spherical coverage requirements for UEs which support multiple FR2 bands” [7].

We provided a comprehensive analysis describing how to extend the peak EIRP and EIRP spherical coverage requirements’ applicability to UEs supporting multiple FR2 bands in [12]. In this section, we summarize the discussions from [12] and explain how the EIS spherical coverage requirement can be similarly extended to UEs supporting multiple bands in FR2.

For reference, Figure 1 below illustrates the 3GPP NR FR2 bands in the scope of Rel-15 work.

Figure 1: 3GPP NR FR2 Bands (Rel-15)

Observation 3: Receive performance of UEs which support multiple bands in FR2 is expected to be optimized over the entire supported range of frequencies.

The impact of joint optimization across both 28 GHz and 39 GHz frequency ranges on the peak EIS and EIS at 50%-tile CCDF can be quantified by considering two sets of spherical coverage CDF simulations:  one set assumes the UE supports all FR2 bands (n257, n258, n260, and n261; labeled Model 6), while another set assumes the UE supports a single FR2 band (labeled Model 4a and 4b).  Model 6 consists of two array designs:  one design covers the frequency range from 24 to 30 GHz, and another co-located design covers the frequency range from 37 to 40 GHz. Additional detail about the simulation assumptions is provided in [12].

The single-band result is taken as the baseline and is compared to the multi-band result; differences in the performance of peak EIS and EIS at 50%-tile CCDF metrics are quantified in the analysis. Figure 2 below illustrates the results for 28 GHz:

a)

b)

Figure 2: Multi-band analysis for 28 GHz

We note that the complementary CDF (CCDF) is used to quantify the EIS spherical coverage metric for readability.

Figure 2a illustrates the EIS CCDF for the single band reference (Model 4a, black curve) and EIS CCDF curves for three frequency points of the multi-band design (Model 6, dotted magenta, black, and blue curves).

Observation 4: EIS CCDFs (Figure 2a) quantify the multi-band impact on the peak EIS requirement as a 2.1 dB increment for band n257, 2.1 dB increment for band n258, and 2.0 dB increment for band n261.

Observation 5: EIS levels at the 50%-tile CCDF (Figure 2b) quantify the multi-band impact on the spherical coverage requirement as a 1.9 dB increment for band n257, 2.0 dB increment for band n258, and 1.2 dB increment for band n261.

Figure 3 below illustrates the results for 39 GHz:

a)

b)

Figure 3: Multi-band analysis for 39 GHz

Observation 6: EIS CCDFs (Figure 3a) quantify the multi-band impact on the peak EIS requirement as a 1.5 dB increment for band n260.

Observation 7: EIS levels at the 50%-tile CCDF (Figure 3b) quantify the multi-band impact on the spherical coverage requirement as a 1.8 dB increment for band n260.

### Analysis of Two-band Performance

In an effort to improve the understanding of the performance impact of UEs supporting both NR bands n261 and n260 vs. single band UEs, the antenna designs using the same simulation setup as [12] were optimized for the single band (n261 or n260) and dual band (n261 and n260) configurations. The dual-band design (Model 5) consists of two array designs:  one design covers the frequency range from 27 to 29 GHz (band n261), and another co-located design covers the frequency range from 37 to 40 GHz.

Simulations have shown that the low band array (Model 5, frequency in the range of 27 to 29 GHz) performance is not significantly impacted by optimizations for high band support when compared to Model 4a in band n261.  Any differences observed were negligible.

Observation 8: For a device which optimizes just bands n260 and n261, it is reasonable not to have peak EIS increment from the single-band requirement for both of them.

Observation 9: For a device which optimizes just bands n260 and n261, no impact on the EIS spherical coverage performance in band n261 is observed.

Figure 4 below illustrates the difference in the EIS 50%-tile CCDF between the dual band (Model 5) and single band (Model 4b) models.

Figure 4: Single band vs dual band results for NR band n260

Observation 10: For a device which optimizes just bands n260 and n261, the impact of dual band antenna array optimization over band n260 ranges from 0.8 to 1.6 dB.  Since the spherical coverage requirement is a minimum requirement for the UE, a relaxation of 1.6 dB of the single band spherical coverage requirement is needed for this band.

### Summary of EIS Spherical Coverage Requirement

Based on the technical analysis provided in this paper, we propose relaxations for the beam peak EIS and EIS spherical coverage requirement, labeled ΔTEMBP and ΔTEMBS, respectively, for PC3 UEs supporting multiple FR2 bands.

Observation 11: Supporting multi-band in FR2 requires relaxation for peak EIS, and EIS spherical coverage requirements for PC3 UEs.

In this paper, we explained the impact of form-factor de-sense, extreme temperature conditions on peak EIS and EIS spherical coverage for single and multi-band UEs. We however suggested to only consider impact of form-factor de-sense and extreme temperature conditions on the EIS spherical coverage. We also proposed to define the spherical coverage requirement at the 50%-tile of the EIS CCDF. Table 1 shows the proposes EIS spherical coverage limit considering relaxations due to form-factor de-sense and extreme temperature conditions.

Table 1: EIS Peak and Spherical Coverage for Single Band UEs

| Supported bands | Applicable to band | Form Factor De-sense (dB) | Extreme Temperature Condition Support (dB) | Peak EIS limit at 50 MHz BW (dBm) | EIS Spherical coverage limit at 50 MHz BW (dBm) |
| --- | --- | --- | --- | --- | --- |
| n257 | n257 | 2.0 | 1.0 | -88.3 | -74.4 |
| n258 | n258 |  |  | -88.3 | -74.4 |
| n260 | n260 |  |  | -85.7 | -70.1 |
| n261 | n261 |  |  | -88.3 | -74.4 |

Proposal 7: The EIS spherical coverage for single band UEs, as summarized in Table 1, is defined in the draft CR accompanying this contribution [13].

We suggested a combined relaxation including the impacts of form-factor de-sense, extreme temperature conditions and multi-band support at the 50%-tile of EIS CCDF. These relaxations are indexed by the bands which the device supports and are applicable in a band-specific manner, as shown in Table 2 below.

Table 2: Summary of the multi-band framework for EIS peak and spherical coverage

| Supported bands | Applicable to band | Multi-band relaxation at 50%-tile (dB) | Multi-band relaxation at peak (dB) | Peak EIS limit at 50 MHz BW (dBm) | EIS Spherical coverage limit at 50 MHz BW (dBm) |
| --- | --- | --- | --- | --- | --- |
| n260, n261 | n260 | 1.6 | 0 | -85.7 | -68.5 |
| n260, n261 | n261 | 0.0 | 0.0 | -88.3 | -74.4 |
| n257, n258, n260, n261 | n257 | 1.9 | 2.1 | -86.2 | -72.5 |
| n257, n258, n260, n261 | n258 | 2.0 | 2.1 | -86.2 | -72.4 |
| n257, n258, n260, n261 | n260 | 1.8 | 1.5 | -84.2 | -68.3 |
| n257, n258, n260, n261 | n261 | 1.2 | 2.0 | -86.3 | -73.2 |

Proposal 8: The multi-band framework for EIS spherical coverage, as summarized in Table 2, is defined in the draft CR accompanying this contribution [13].

# Proposals (for approval)

In this paper, we explained the impact of form-factor de-sense, extreme temperature conditions, and multi-band support on EIS peak and spherical coverage. We also proposed to define the spherical coverage requirement at 50%-tile of the EIS CDF. We suggest to apply the combined relaxations including the impacts of form-factor de-sense, extreme temperature conditions and multi-band support at 50%-tile of EIS CDF making the gap to the peak EIS larger compared to that of EIRP spherical coverage.

Based on the simulation results, the following observations and proposal are made:

Observation 1: EIS spherical coverage performance may be inferred from the EIRP CDF if a UE supports beam correspondence.

Observation 2: Parameters such as form factor de-sense, extreme temperature conditions, and supporting multiple bands can impact EIS.

Observation 3: Receive performance of UEs which support multiple bands in FR2 is expected to be optimized over the entire supported range of frequencies.

Observation 4: EIS CCDFs (Figure 2a) quantify the multi-band impact on the peak EIS requirement as a 2.1 dB increment for band n257, 2.1 dB increment for band n258, and 2.0 dB increment for band n261.

Observation 5: EIS levels at the 50%-tile CCDF (Figure 2b) quantify the multi-band impact on the spherical coverage requirement as a 1.9 dB increment for band n257, 2.0 dB increment for band n258, and 1.2 dB increment for band n261.

Observation 6: EIS CCDFs (Figure 3a) quantify the multi-band impact on the peak EIS requirement as a 1.5 dB increment for band n260.

Observation 7: EIS levels at the 50%-tile CCDF (Figure 3b) quantify the multi-band impact on the spherical coverage requirement as a 1.8 dB increment for band n260.

Observation 8: For a device which optimizes just bands n260 and n261, it is reasonable not to have peak EIS increment from the single-band requirement for both of them.

Observation 9: For a device which optimizes just bands n260 and n261, no impact on the EIS spherical coverage performance in band n261 is observed.

Observation 10: For a device which optimizes just bands n260 and n261, the impact of dual band antenna array optimization over band n260 ranges from 0.8 to 1.6 dB.  Since the spherical coverage requirement is a minimum requirement for the UE, a relaxation of 1.6 dB of the single band spherical coverage requirement is needed for this band.

Observation 11: Supporting multi-band in FR2 requires relaxation for peak EIS, and EIS spherical coverage requirements for PC3 UEs.

Proposal 1: A separate EIS spherical coverage requirement or test is not required for UEs supporting beam correspondence and meeting the EIRP spherical coverage requirement.

Proposal 2: If a UE does not support beam correspondence, RAN4 should decide if an EIS spherical coverage requirement needs to be defined.

Proposal 3: Define the EIS spherical coverage requirement in FR2 at 50%-tile value of its CDF.

Proposal 4: Consider impact of form factor de-sense and extreme temperature conditions in defining the spherical EIS requirement.

Proposal 5: Incorporate a 2 dB relaxation for form factor de-sense in the EIS spherical coverage requirement.

Proposal 6: Consider a 1 dB relaxation in the EIS spherical coverage requirement to address the noise floor increase at extreme temperature conditions.

Proposal 7: The EIS spherical coverage for single band UEs, as summarized in Table 1, is defined in the draft CR accompanying this contribution [13].

Proposal 8: The multi-band framework for EIS spherical coverage, as summarized in Table 1, is defined in the draft CR accompanying this contribution [13].

# References

- RP-172834, “New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #78, December 2017

- TS38.101-2 v15.2.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, June 2018

- R4-1710897, “EIS versus EIRP spherical coverage”, MediaTek, 3GPP RAN4 #84bis, October 2017

- R4-1710607, “mmWave EIS requirement”, Huawei, 3GPP RAN4 #84bis, October 2017

- R4-1808198,	 “WF for spherical coverage for FR2”, Qualcomm Inc., 3GPP RAN4 #87, May 2018

- RP-180933, “Extending FR2 spherical coverage requirement to multi-band UEs,” Apple Inc., 3GPP RAN #80, June 2018

- RP-181473, “Rel-15 Work Item Exception for New Radio Access Technology – Core,” NTT DOCOMO, 3GPP RAN #80, June 2018

- R4-1707824, “Discussion on mmWave EIRP/EIS test”, Huawei, 3GPP RAN4 #84, August 2017

- R4-1805322,	“On UE Beam Correspondence and EIS spherical coverage”, Sony, 3GPP RAN4 #86bis, April 2018

- R4-1804125, “FR2 peak EIS requirement for handheld UE”, Intel Corporation, Apple, 3GPP RAN4 #86bis, April 2018

- R4-1804589, “ MMW UE EIS“, Qualcomm Inc., 3GPP RAN4 #86bis, April 2018

- R4-1809786, “Extending FR2 power class requirements to multi-band UEs”, Apple Inc., 3GPP RAN4 #88, August 2018

- R4-1810066, “Draft CR to TS 38.101-2: Introducing EIS spherical coverage requirements for PC3”, Apple Inc., 3GPP RAN4 #88, August 2018