3GPP TSG RAN WG4 Meeting # 88                                            	R4-1810099

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.13.1.3

Source: 	AT&T

Title: 	On NR PDSCH Performance

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved.  In the previous RAN4 meeting (RAN4#86bis), it was decided to define UE demodulation requirements for PDSCH, PDCCH [1] in FR1 and FR2.

In this contribution, we provide our views on PDSCH demodulation requirements and list the simulation assumptions for reference.

# Open Issues in NR PDSCH Demodulation Requirements

In the previous meeting, some key parameters such as the number of receiver ports, number of layers have been agreed for PDSCH demodulation requirement.  However, there are many open issues for NR as the transmission schemed used for NR is different compared to LTE.  In this section, we discuss about the open issues in PDSCH and the demodulation requirements.

- DMRS Configuration:  Unlike LTE, NR defines different DMRS configurations for various scenarios.

Table 1 Possible DMRS Configurations in NR

| DMRS Type | Number of front loaded symbols | Maximum number of additional symbols |
| --- | --- | --- |
| Type 1 | Single | 1, 2, and 3 |
|  | Two | 1 |
| Type 2 | Single | 1, 2 and 3 |
|  | Two | 1 |

As shown in Table 1, NR defines two types of DMRS configurations where DMRS type 1 has more RE density per RB than Type 2.  Each Type can be single symbol front-loaded or two symbol front-loaded. In addition, NR provides an option to configure additional DMRS symbols. The maximum number of additional DMRS symbols depends on the number of OFDM symbols per slot. For 14 OFDM symbols per slot case, up to 3 additional DMRS symbols are possible for single symbol front loaded DMRS.  For two symbol front-loaded DMRS, up to 1 additional DMRS is possible for both Type 1 and Type 2.  Since we can’t define performance requirements for each case, we would like to minimize the simulation effort and standardization effort by eliminating some cases.

Figure 1 shows the spectral efficiency for single symbol and two symbol case for Type 1 DMRS configuration.  The simulation assumptions are given in Appendix. It can be observed that for this case, even though the channel estimation accuracy is improved there is no benefit of using two symbols for DMRS as they occupy more resources thereby reducing the number of resource elements needed for PDSCH. Hence we would like to eliminate two symbol cases for performance requirements.

Figure 1 Spectral efficiency comparison for Type 1 DMRS with Single Symbol and Two Symbols

Figure 2 shows the spectral efficiency performance with additional DMRS symbols. In this case, we didn’t see any benefit with additional DMRS symbols. However note that we assumed UE speed is equal to 3Kmph. The additional DMRS symbols are mainly used for high mobility scenarios.

Figure 2 Spectral efficiency comparison for Type 1 DMRS with Single Symbol front loaded and additional symbols

Figure 3 shows the performance of Type 2 configuration with single symbol and two symbols. In this case too, we observe small gain only at low SNR.  Hence we can skip two symbol case for defining performance requirement.

Figure 3 Spectral efficiency comparison for Type 2 DMRS with Single Symbol and Two Symbols

Figure 4 shows the performance of Type 2 configuration with single symbol front loaded and additional DMRS symbols. In this case, we observed gain due to additional DMRS symbol even with low UE mobility. Note that type 2 uses less number of REs per RB compared to Type 1. Hence we recommend to have performance requirement for both Type 1 and Type 2 with 1 additional DMRS symbol.

In addition, we would like to mention that RAN4 should define performance requirements with fixed number of layers. That is the performance depends on the CDM group. Hence to avoid confusion, RAN4 should define the CDM group for a given number of layers and port numbers.

Figure 4  Spectral efficiency comparison for Type 2 DMRS with Single Symbol front loaded and additional symbols

Hence we propose:

Proposal 1: Only single symbol front-loaded DMRS is considered for defining NR PDSCH demodulation performance for Type 1 and Type 2

Proposal 2: Up to 1 additional DMRS symbol is considered for defining NR PDSCH demodulation performance for Type 1 and Type 2

Proposal 3: For defining NR PDSCH demodulation performance, RAN4 should specify the CDM group and the number of ports

- MCS/TBS:   In the previous meeting it was decided to use QPSK/16QAM/64QAM/256QAM for FR1 and QPSK/16QAM/64QAM for FR2. However, it should be noted that channel coding scheme for NR is different compared to LTE. Two base graphs are used in NR. We would like to mention that we need test cases which cover both the base graphs.  Hence we propose

Proposal 4: For defining NR PDSCH demodulation performance RAN4 should consider cases which cover both LDPC base graphs

- Resource Allocation and Mini Slots:  As NR defines slots and mini slots, we would like to define performance requirements for both slots and mini slots case. Since the mini slot case can be of length 2, 4 or 7 symbols, ideally we need to define performance for each case. However to minimize the standardization effort at least performance is defined for 7 symbol mini slot for FR1.  For FR2, we would like to have requirements for 2 and 4 symbol mini slots. Note that Type B DMRS is used for mini slots. We expect the same DMRS configurations used for slots is applicable for mini slots.  Hence we propose

Proposal 5: For defining the NR PDSCH demodulation performance for mini slots, RAN4 should consider the case with 7 symbol mini slot for FR1 and 2 and 4 mini slot for FR2

- PTRS:   In NR, a new reference signal for phase compensation is defined. Since PTRS is mainly useful for FR2, we would like to define performance requirements for FR2 only and we don’t see any use case in FR1. Hence we propose

Proposal 6: For defining the NR PDSCH demodulation performance PTRS configuration is used only for FR2

- Transmission Schemes: Note that for PDSCH transmission even though NR defines transmission scheme 1, it is well understood in RAN1	 that NR BS can use transparent diversity scheme also called REG level precoder cycling. This is useful mainly for high mobility scenarios. From our previous experience in LTE that for high mobility scenarios, transmission scheme 1 incur losses due to outdated channel state information. Hence we would encourage RAN4 to discuss about setting up the performance requirements for transparent diversity or precoder cycling for NR PDSCH especially under high mobility scenarios. Hence we propose

Proposal 7: RAN4 should decide whether to define performance requirements for transparent diversity scheme in addition to transmission scheme 1

- Advanced receiver: In our view when transmission rank is greater than 2 significant gains can be achieved with advanced receiver such as ML/MAP. Hence we would like to define a test case with advanced receiver for 4x4 MIMO.

Proposal 8: For defining performance requirements for RAN4, ML/MAP receivers should be considered for transmission rank >2

# Conclusions

In this contribution we outlined our views on NR PDSCH demodulation requirements. Based on our observations we recommend

Proposal 1: Only single symbol front loaded DMRS is considered for defining NR PDSCH demodulation performance Type 1 and Type 2

Proposal 2: Up to 1 additional DMRS symbol is considered for defining NR PDSCH demodulation performance Type 1 and Type 2

Proposal 3: For defining NR PDSCH demodulation performance, RAN4 should specify the CDM group and the number of ports

Proposal 4: For defining NR PDSCH demodulation performance RAN4 should consider the cases which cover both LDPC base graphs

Proposal 5: For defining NR PDSCH demodulation performance for mini slots, RAN4 should consider the case with 7 symbol mini slot for FR1 and 2 and 4 mini slot for FR2

Proposal 6: For defining the NR PDSCH demodulation performance PTRS configuration is used only for FR2

Proposal 7: RAN4 should decide whether to define performance requirements for transparent diversity scheme in addition to transmission scheme 1

Proposal 8: For defining performance requirement RAN4, ML/MAP receivers should be considered for transmission rank >2

# References

[1]. R4-1805549, “Way forward on NR UE performance”, Intel Corporation, Ericsson, Samsung, NTT   DOCOMO, Qualcomm, LGE, Spirent

# Appendix

In this section we list the simulation assumptions for Figures 1-4.

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4.1 GHZ |
| Duplex | FDD/TDD |
| System Bandwidth | 50 MHz |
| Slot length | 1msec |
| Subcarrier spacing | Single numerology case: 15 KHz |
| LDPC iterations | Message passing with 50 iterations |
| FFT size | 4096 for 15 KHz subcarrier spacing |
| Channel model | TDL-A, 3 Kmph |
| Spectral shaping filter | Not modeled |
| Antenna configuration | 4X2 |
| Rank per UE | Fixed rank =1 |
| MCS | MCS 2 in TS 38.214 |
| DM-RS | Type 1and Type 2 with single/two symbol front loaded and additional symbols of 1, 2 |
| Channel estimation | MMSE |