3GPP TSG-RAN WG4 Meeting #88	R4-1810137

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	5.3.2.1

Source:	Ericsson

Title:	NB-IoT downlink channel quality determination and report

Document for:	Discussion

# 1	Introduction

RAN1 agreed to include a new mechanism for NB-IoT UE to determine and report the channel quality in Msg3 during the random access procedure [1].

- The downlink channel quality is denoted as the repetition number that the UE needs to decode hypothetical NPDCCH with BLER of 1% - FFS the details for this metric (at least including measure resources, measure duration, and the details for hypothetical NPDCCH, such as the format, the aggregation level)

RAN4#87 also discussed the MSG3-based based channel quality reporting and agreed with the following way forward [2].

- Actual values of candidate_x in MSG3 - For RRCConnectionReestablishmentRequest-NB, companies are encouraged to propose the actual values for {candidateRep_1, candidateRep_2, candidateRep_3} - Companies are encouraged to investigate the reporting three values - For other messages, the reported values are {1,2,4,8,16,32,64,128,256,512,1024,2048}. - From RAN4 perspective there could be two measurement periods UE can use to derive the NPDCCH repetition number to satisfy the hypothetical NPDCCH BLER of 1%: - T1: period used for NRSRP estimation for NPRACH CE level decision - T2: period from the beginning of MSG2 reception to the beginning of MSG3 transmission - It is up to UE implementation to use T1 and/or T2 to estimate the downlink channel quality as far as UE meets the accuracy requirement. - Accuracy requirement is defined to report the NPDCCH repetition number to satisfy the hypothetical NPDCCH BLER of 1% - The reporting accuracy will be verified with the corresponding test case. - RAN4 to specify the following requirements in TS36.133 core specification. FFS to how to specify. - The measurement period - The accuracy requirements of reported repetition numbers

This contribution discusses the open issues on the MSG3-based channel quality measurement and report.

# 2	Discussion

## 2.1	Reporting values in RRC Connection Reestablishment Request

RAN2 has specified the information element used for MSG3-based quality report as follows (TS36.331).

CQI-NPDCCH-Short-NB information element

-- ASN1START

CQI-NPDCCH-Short-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-1, candidateRep-2, candidateRep-3}

-- ASN1STOP

According to RAN2, there are only 2 bits (= 4 values) available for MSG3-based reporting. Since one value ‘0b00’ cannot be used because of the backward compatibility (no measurements), we can report 3 values on MSG3. It was also discussed in the last RAN4 meeting that some UE implementation prepares the MSG3 (e.g. RRC Connection Reestablishment Request) first then triggers the random access on NPRACH. This means some UE cannot update the RRC message on MSG3 after MSG2 is received and such a UE cannot report the actual NPDCCH repetition number used for MSG2. The possible repetition number UE can report is based on the maximum number of repetitions for NPDCCH common search space for RAR (Rmax) because UE should know Rmax from SI. According to our investigation [3], SINR estimation is not so accurate that the step size should be 6dB or more. We therefore propose to define the reporting value as: {Rmax/4, Rmax, more than Rmax}.

Proposal 1: RAN4 defines the MSG3-based reporting values as {‘Rmax/4’, ‘Rmax’, ‘More than Rmax’}, where Rmax is the maximum number of repetitions for NPDCCH common search space for RAR.

UE shall report:

- ‘Rmax/4’ if the hypothetical NPDCCH BLER with repetition Rmax/4 is less than 1%

- ‘Rmax’ if the hypothetical NPDCCH BLER with repetition Rmax is less than 1%

- ‘More than Rmax’ if the hypothetical NPDCCH BLER with repetition Rmax is 1% or more.

## 2.2	Requirements of channel quality reporting

RAN4 agreed that there could be two measurement periods UE can use to derive the NPDCCH repetition number to satisfy the hypothetical NPDCCH BLER of 1%:

- T1: period used for NRSRP estimation for NPRACH CE level decision

- T2: period from the beginning of MSG2 reception to the beginning of MSG3 transmission

RAN4 also agreed that the accuracy requirement is defined to report the NPDCCH repetition number to satisfy the hypothetical NPDCCH BLER of 1%.

We propose RAN4 will capture them in the TS36.133. We show TP below:

----------------------------------------------------- Beginning of TP ------------------------------------------------------------

#### 6.6.2.6	MSG3-based channel quality report

The requirements in this clause shall apply for UE supporting DL channel quality reporting for UE Category NB1 as defined in [7].

The DL channel quality provides the serving eNB with information about the minimum NPDCCH repetition level to satisfy the hypothetical NPDCCH block error rate of 1%. The reported NPDCCH repetition level shall be derived from the channel quality measured in the period T1 or T2 in the carrier where the random access response is transmitted, where

- T1 is the period before NPRACH transmission used for NRSRP measurement for enhanced coverage level estimation

- T2 is the period from the beginning of the random access response to the beginning of PUSCH format 1 for DL channel quality reporting.

The NPDCCH repetition level for CQI-NPDCCH-NB is chosen from the supported NPDCCH repetition levels [3] The report mapping is defined in 9.1.27.

### 9.1.27	MSG3-based Measurement Report Mapping

Table 9.1.27-1: Downlink channel quality measurement report mapping of CQI-NPDCCH-NB when the DL channel quality reporting is supported [7]

| Reported value | NPDCCH repetition level |
| --- | --- |
| candidateRep-A | 1 |
| candidateRep-B | 2 |
| candidateRep-C | 4 |
| candidateRep-D | 8 |
| candidateRep-E | 16 |
| candidateRep-F | 32 |
| candidateRep-G | 64 |
| candidateRep-H | 128 |
| candidateRep-I | 256 |
| candidateRep-J | 512 |
| candidateRep-K | 1024 |
| candidateRep-L | 2048 |

The NPDCCH repetition level for CQI-NPDCCH-Short-NB is chosen with regard to the signalled parameter Rmax, the maximum number of repetitions for NPDCCH common search space for random access response (npdcch-NumRepetitions-RA) in SIB2-NB. The report mapping is defined in Table 9.1.27-2. UE shall report candidateRep-1 when the hypothetical NPDCCH BLER with repetition Rmax/4 is less than 1%. UE shall report candidateRep-2 if the hypothetical NPDCCH BLER with repetition Rmax is less than 1%. UE shall report candidateRep-3 if the hypothetical NPDCCH BLER with repetition Rmax is 1% or more.

Table 9.1.27-2: Downlink channel quality measurement report mapping of CQI-NPDCCH-Short-NB when the DL channel quality reporting is supported [7]

| Reported value | NPDCCH repetition level |
| --- | --- |
| candidateRep-1 | Rmax/4 (Note 1) |
| candidateRep-2 | Rmax |
| candidateRep-3 | More than Rmax |
| Note 1: When Rmax is less than 4, set candidateRep-1 to 1. |  |

----------------------------------------------------- End of TP ------------------------------------------------------------

# 3	Conclusions

Proposal 1: RAN4 defines the MSG3-based reporting values as {‘Rmax/4’, ‘Rmax’, ‘More than Rmax’}, where Rmax is the maximum number of repetitions for NPDCCH common search space for RAR.

UE shall report:

- ‘Rmax/4’ if the hypothetical NPDCCH BLER with repetition Rmax/4 is less than 1%

- ‘Rmax’ if the hypothetical NPDCCH BLER with repetition Rmax is less than 1%

- ‘More than Rmax’ if the hypothetical NPDCCH BLER with repetition Rmax is 1% or more.

# References

- R1-1803163, “LS on NB-IoT Downlink Channel Quality Determination and Report”, RAN1.

- R4-1807938, “WF on NB-IoT downlink channel quality determination and report”, Ericsson.

- R4-1806754, “NB-IoT downlink channel quality determination and report”, Ericsson.