3GPP TSG-RAN WG4 Meeting #88	R4-1810145

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	6.10.5

Source:	Ericsson

Title:	Discussion on UE demodulation requirements for HRLLC

Document for:	Approval

# 1	Introduction

RAN plenary #80 almost completes the core part of WI on Ultra Reliable Low Latency Communication for LTE [1]. This contribution discusses the impact for UE demodulation requirement due to LTE HRLLC based on the summary [2].

# 2	Discussion

## 2.1	Semi-static CFI configuration

In LTE, UE first decodes PCFICH to know the number of OFDM symbols used for control channel. Since RAN1 identified CFI decoding performance is one of the bottlenecks for higher reliability operation, RAN1 agreed that the network configures the CFI semi-statically in order to avoid the UL/DL performance degradation due to the CFI decoding failure.

If the semi-static CFI is configured, UE does not need to decode CFI before PDCCH decoding. This does not impact to UE demodulation.

## 2.2	Blind PDSCH repetition

RAN1 introduce the repeated PDSCH transmission for the higher reliable operation. If the network configures the PDSCH repetition (a.k.a. blind PDSCH repetition), network can repeat the same PDSCH data, and it is signalled with DCI 1A or 7-1x (slot- or subslot-PDSCH). The supported repetition numbers (K) are 1, 2, 3, 4, or 6. During the repeated transmission, the network can use the different redundancy version (RV) per transmission, and the supported RV sequence options are {0, 2, 3, 1} or {0, 0, 0, 0}.

From the UE demodulation point of view, this feature is similar to PDSCH repetition used for eMTC. The main difference is the blind PDSCH is signalled from PDCCH or SPDCCH instead of MPDCCH, and the PRB allocation is not based on narrowband. However from the UE receiver algorithm point of view we don’t see any difference.

We therefore propose to introduce new PDSCH demodulation requirements for the blind PDSCH repetition by reusing the existing eMTC demodulation requirements. In Rel-14, RAN4 has introduced the non-BL UE demodulation requirements which uses 4 repetitions for 2Rx with RV sequence = {0, 2, 3, 1}, and these conditions are also supported in the blind PDSCH repetition.

Proposal 1: RAN4 will introduce new PDSCH demodulation requirements supporting the blind PDSCH repetition by reusing the existing eMTC demodulation requirements.

One difference is the PRB allocation method; the non-BL UE demodulation requirements assume 36PRB (PRB#1 to #18 and #31 to #48) to avoid the conflict with center PRB. If we keep the same number of PRBs, we may need to assume the contiguous PRB allocation such as PRB#0 - PRB#35.

Another issue is the TDD case. For eMTC, eNB configures the BL/CE DL subframs (fdd-DownlinkOrTddSubframeBitmapBR), where eNB can configure which subframes are used as DL transmission. With this configuration, the TDD demodulation requirements in RAN4 avoid the DL transmission in the special subframe to avoid different coding rate. However LTE URLLC cannot skip the DL transmission in special subframe if the PDCCH blind repetition is triggered. According to the eMTC demodulation requirements specified in TS36.101, TDD UL/DL configuration is set to 1 (DSUUD DSUUD), which has two consecutive DL subframes. To avoid the PDSCH transmission in special subframe, one possibility is to assume 2 repetitions for blind PDSCH repetition, and to configure UL/DL configuration 2 (DSUDD DSUDD) because we should avoid SF#0 and SF#5 because these subframes are used for the transmission of PSS/SSS/PBCH and SI.

Figure 1 shows the PDSCH simulation results with 2/4 repetitions with 36/50PRBs. The results with repetition 4 with 36PRB are the same condition as the existing eMTC requirements with TM2 EPA5 2x2 QPSK 1/3. It is observed from our simulation that:

- With repetition 4, changing from 36PRB to 50PRB requires additional 0.2dB to achieve 70% of the maximum throughput.

- With 36PRB, changing PDSCH repetition level from 4 to 2 requires additional 2.8dB to achieve 70% of the maximum throughput.

Proposal 2: For the blind PDSCH repetition requirements, reuse the existing Non-BL UE PDSCH demodulation requirements with TM2 EPA5 2x2, QPSK TBS=4392bits (8.11.1.1.3.2 Test 3 for FDD and 8.11.1.2.3.2 Test 3 for TDD).

Proposal 3: For FDD, reuse the same requirements as eMTC non-BL demodulation requirements.

Proposal 4: For TDD, set UL/DL configuration to 2, and schedule DCI on SF#3 and SF#8 avoid the DL transmission in special subframe. Repetition level is set to 2 and the requirements are reused by adding 3.0dB because the repetition level is changed from 4 to 2.

Figure 1	PDSCH with 2/4 repetitions.

## 2.3	Others

RAN2 introduced new features for higher reliability operation: PDCH packet duplication and granular time reference provision. PDCH packet duplication allows to transmit one PDCH packet on two independent transmission paths for diversity. Another feature is to provide a granular time reference value to a UE from an eNB (0.25us). Since these features are related to higher layer and thus no new demodulation requirements are needed.

# 3	Summary

Proposal 1: RAN4 will introduce new PDSCH demodulation requirements supporting the blind PDSCH repetition by reusing the existing eMTC demodulation requirements.

Proposal 2: For the blind PDSCH repetition requirements, reuse the existing Non-BL UE PDSCH demodulation requirements with TM2 EPA5 2x2, QPSK TBS=4392bits (8.11.1.1.3.2 Test 3 for FDD and 8.11.1.2.3.2 Test 3 for TDD).

Proposal 3: For FDD, reuse the same requirements as eMTC non-BL demodulation requirements.

Proposal 4: For TDD, set UL/DL configuration to 2, and schedule DCI on SF#3 and SF#8 avoid the DL transmission in special subframe. Repetition level is set to 2 and the requirements are reused by adding 3.0dB because the repetition level is changed from 4 to 2.

# 4	References

R4-180692, “Status report of WI Ultra Reliable Low Latency Communication for LTE”, Ericsson.

RP-180693, “Summary of WI Ultra Reliable Low Latency Communication for LTE”, Ericsson.