3GPP TSG-RAN WG4 Meeting #88	R4-1810308

Gothenburg, Sweden, 20 - 24 Aug, 2018

Agenda item:	9.11

Source:	Dish Network

Title:	n65 requirements

Document for:	Approval

# 1	Introduction

This contribution discusses the regional requirements for n65 of the approved n65 WI at the June plenary. [1]

# 2	Discussion

We begin by recapping LTE Band 65 requirements. LTE B65 has requirements for both Region 1 and Region 3.

- Region 1 requirements are straightforward, with exception of a note that mandates B65 UEs to meet also B1 requirements.

- Region 3 requirements are significantly more burdensome due to B34 protection requirements in some countries and hence need two NS values and a lot of simulations in RAN4 to provide B34 protection. We note that the A-MPR and guard bands to protect B34 are in many cases very large, up to 17dB. Also, NS_05 to protect PHS is applicable within B1 frequency range and respective requirements are copied from B1.

For NR, the focus in the WID proposal was Region 1, hence, there are several options to define n65:

Option 1: Define n65 for Region 1 only

Option 2: Define n65 for Region 1, but also include co-existence requirements for all Region 3 bands excluding B34

Option 3: Define n65 for Regions 1 and 3, including Band 34 protection requirements and associated NS values

Option 1 would be straightforward and minimize the work load in RAN4 as n65 is targeted for Region 1 deployment.

Option 2 would still be Region 1 compliant, however it would also address protection for Region 3 countries when B34 is not deployed or used.  This option is also straightforward and minimize the workload in RAN4. We note in some Region 3 countries the B34 spectrum has been re-allocated or is not being used.

Option 3 would offer the exact same regional solution as B65, but would require impractical UL restrictions when operating at upper portion of n65. Simulation assumptions and guard band assumptions for “NS_24/25” tables would need to be agreed in this meeting to allow enough time for extensive simulations – see [2].

We propose defining n65 according to option 1, however option 2 is also acceptable since this extend the flexibility and usage of this band to some countries in Region 3 where B34 is protection is not needed.

Proposal: Define n65 for Region 1 only (option 1)

# 3	Conclusions

In this contribution, options to define n65 were discussed. It is proposed to define n65 as follows.

Proposal: Define n65 for Region 1 only (option 1)

# 4	References

[1] RP-181437 “New WID on Band 65 for New Radio”, Dish Network, HNS, Thales, RAN#80

[2] 3GPP TR36.861