3GPP TSG-RAN WG4 Meeting #88	R4-1810318

Gothenburg, Sweden, 20 - 24 Aug, 2018

Agenda item:	12.1

Source:	Dish Network

Title:	NR – NB-IoT Co-existence

Document for:	Approval

# 1	Introduction

This contribution discusses scenarios to be evaluated in NR - NB-IoT co-existence study according to the WF endorsed in RAN#80 [1].

# 2	Discussion

The RF co-existence for NB-IoT with NR has not been yet been considered. The differences between NR and LTE are different SCS options and different spectrum utilization which varies as a function of SCS and channel bandwidth.

It is essential for the industry to have well-characterised co-existence between NB-IoT and NR. Hence all NB-IoT deployment modes used in LTE should be verified with NR. We discuss each of these modes below and describe scenarios we find important to be analysed in co-existence study.

In-band:

Case with 15kHz SCS in NR is pretty similar to LTE as the LTE SCS is always 15KHz. However, the ½ SC shift need to be included for the LTE re-farming bands to allow orthogonality and to avoid slight overlap between NR and NB-IoT SC’s at BS receiver.

Cases with 30kHz and 60kHz SCS are different to 15kHz because in these cases the BS does not see contiguous transmission bandwidth in case only one NB-IoT RB is placed in-band. NB-IoT RB is 180KHz while one NR RB with 30kHz or 60KHz SCS are 360kHz or 720kHz respectively. The potential impacts of this are likely small yet they should be clarified. The loss of orthogonality may also have an impact to performance that should be studied. Lastly, as NB-IoT and NR SC’s should not overlap for LTE re-farming bands the potential need for the SC shift(s) should be studied.

General for all NR SCS, possibility to maintain the +6dB power boost needs to be verified

Proposal1: 7.5kHz shift for NR with 15kHz SCS for the remaining LTE re-farming bands should be added. The impacts for 30kHz and 60kHz SCS need to be analysed as well as the potentially needed shift(s) to guarantee NR and NB-IoT SC’s don’t overlap in any scenario. Possibility to maintain 6dB power boost needs to be verified.

Guard band:

In LTE, the guard band is fixed 10% of CC BW for the carriers where guard band deployment is allowed. The situation is completely different in NR, where the guard bands are asymmetric and vary actually quite a lot. Higher BW has higher spectrum utilization and hence less guard band while larger SCS leads to lower spectrum utilization for each BW and hence to larger guard band. For which NR CC BW/SCS combinations the guard band deployment is possible needs to be studied. We recognize the spectrum utilization discussion was very intense also from the BS perspective, so we envision guard band deployment might in general be more difficult for NR than it is for LTE. Possibility to maintain the +6dB power boost needs to be verified.

Proposal2: The possibility of guard band deployment needs to be studied for different NR CC BW/SCS combinations. Possibility to maintain the +6dB power boost needs to be verified.

Standalone:

In LTE, the standalone NB-IoT RB can be placed adjacent to LTE CC with a minimum 100kHz offset between the edge of LTE CC and the edge of NB-IoT CC. In NR, due to smaller guard band in NR, the gap between the transmission bandwidths of NR and NB-IoT would be smaller if the same 100kHz offset is applied. It needs to be studied whether the impact of smaller gap between the transmission bandwidths is negligible or whether different requirements accounting varying NR guard band should be defined.

Proposal3: Impact of smaller gap between the transmission bandwidths of NR and NB-IoT in standalone deployments needs to be studied to conclude whether it is negligible or whether different requirements accounting varying NR guard band should be defined.

# 3	Conclusions

In this contribution our views on scenarios to be included in the co-existence study of NR and NB-IoT were provided.

Proposal1: 7.5kHz shift for NR with 15kHz SCS for the remaining LTE re-farming bands should be added. The impacts for 30kHz and 60kHz SCS need to be analysed as well as the potentially needed shift(s) to guarantee NR and NB-IoT SC’s don’t overlap in any scenario. Possibility to maintain 6dB power boost needs to be verified.

Proposal2: The possibility of guard band deployment needs to be studied for different NR CC BW/SCS combinations. Possibility to maintain the +6dB power boost needs to be verified.

Proposal3: Impact of smaller gap between the transmission bandwidths of NR and NB-IoT in standalone deployments needs to be studied to conclude whether it is negligible or whether different requirements accounting varying NR guard band should be defined.

# 4	References

[1] RP-181437 “Way forward on the study of co-existence of NB-IoT with NR”, Huawei, Hisilicon, Dish, T-Mobile, Lenovo, Motorola Mobility, Ericsson, Nokia, RAN#80