3GPP TSG-RAN WG4 Meeting #88	R4-1810362

# Gothenburg, SW, 20th – 24th Aug, 2018

Source:	NTT DOCOMO, INC.

Title:	MU of OTA OBUE, SEM, ACLR absolute requirement

Agenda item:	6.1.3.3.2

Document for:	Approval

- Introduction

This contribution discusses the MU of OTA OBUE, SEM, ACLR absolute requirement.

2. MU assessment of OTA OBUE, SEM, and ACLR absolute requirement for eAAS

In the last RAN4#AH1807 meeting, the measurement uncertainty (MU) assessment of OTA OBUE, SEM, and ACLR absolute requirement for eAAS was approved as shown in Table 1[1].

Table 1. MU assessment for OTA OBUE, SEM, and ACLR absolute requirement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.15 | 0.27 | Gaussian | 1 | 1 | 0.15 | 0.27 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.45 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.88 | 1.06 |

In this table, the value of the test equipment is the value presented in [2]. However, there is a comment from Keysight that the MU value for test equipment shall be new one that will be presented in RAN4 #88.

The new MU is shown in Table 2[3].

Table 2. Test equipment MU in [3]

| Instrument | Use Case | Condition | 30MHz < f ≤ 3GHz | 3GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz | 6GHz < f ≤ 12.75GHz | 12.75GHz < f ≤ 26GHz | Distribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RF power measurement (Signal Spectrum Analyzer) | Absolute power measurement | Total Amplitude accuracy input signal level down to -70dBm | 0.14 | 0.26 | 0.26 | 0.26 | 0.37 | Gaussian |  |
|  |  | input signal level below -70dBm and 20dB above noise floor | 0.15 | 0.27 | 0.27 | 0.27 | 0.38 | Gaussian |  |
|  |  | Input signal level down to S/N=10dB to Noise floor with enough dynamic range (1)(2) | 0.28 | 0.40 | 0.40 | 0.78 | 0.93 | Gaussian |

Here, we re-calculate the MU for OTA OBUE, SEM, and ACLR absolute requirement as shown in Table 3.

Table 3. Modified MU assessment for OTA OBUE, SEM, and ACLR absolute requirement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.28 | 0.40 | Gaussian | 1 | 1 | 0.28 | 0.40 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.51 | 0.62 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.99 | 1.21 |

Proposal1: Use Table 3 for MU per point assessment of OTA OBUE, SEM, and ACLR absolute requirement of eAAS for indoor anechoic chamber.

3. MU assessment of OTA OBUE, SEM, and ACLR absolute requirement for NR FR1

For NR FR1, MU for the frequency range (4.2 GHz <f≦6 GHz) is required. When we see the table 3, frequency dependent parameters are the uncertainty of the RF power measurement equipment (UID 7), impedance mismatch in the receiving chain (UID 8), uncertainty of the network analyser (UID 17), and uncertainty of the absolute gain of the reference antenna (UID 21). The uncertainty of the RF power measurement equipment and uncertainty of the network analyser for the frequency range (4.2 GHz <f≦6 GHz) is provide by the table 2 and [2], and their values are 0.4 and 0.2, respectively. The uncertainty of the absolute gain of the reference antenna becomes slightly small when the frequency is increased. Though the MU of the reference antenna for the frequency range (4.2 GHz <f≦6 GHz) may smaller than that for the frequency below 4.2 GHz, we can reuse the value, 0.43, for the frequency below 4.2 GHz as the worst case.

With regard to the impedance mismatch in the receiving chain, it is required to calculate the value for the frequency range (4.2 GHz <f≦6 GHz) because the value tends to increase depending on the frequency. In [4], we proposed the error due to mismatch using the multi reflection model as shown in Fig. 1.

Fig.1 Calculation model

The mismatch error is derived by the following equation [4].

Mismatch error = 20log(|(1-ΓSGΓadapter ) (1-ΓadapterΓantenna )- ΓSGΓantennaΓadapter^2|)    (1)

The reflection coefficients of the adaptor and antenna were calculated from the VSWR shown in Table 4 [4].

Table 4  VSWR of adaptor and antenna.

|  | Adapter | Antenna |
| --- | --- | --- |
| VSWR | 1.25:1 | 1.5:1 |

It is required to fine a value for test equipment (signal analyzer) for the frequency range below 6 GHz. Table 5 is the VSWRs for the typical signal analyzer for different vendors.

Table 5 VSWR of typical Signal Analyzer

Since the majority value is 1.5, SWR of TE is set to 1.5. By using these parameters, reflection coefficients of TE, adaptor, and antenna are 0.2, 0.1, and 0.2. The MU due to the impedance mismatch in the receiving chain is calculated as 0.39 by the equation (1).

From above considerations, the MU table for the frequency range for NR can be shown in Table 6

Table 6 MU table for NR FR1.

| UID | Uncertainty source |  | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Uncertainty value 4.2 GHz < f ≦ 6 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna |  | 0.03 | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna |  | 0.3 | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 | 0.17 |
| 3 | Quality of quiet zone |  | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna |  | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna |  | 0.00 | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
| 6 | Phase curvature |  | 0.05 | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment |  | 0.28 | 0.40 | 0.40 | Gaussian | 1 | 1 | 0.28 | 0.40 | 0.40 |
| 8 | Impedance mismatch in the receiving chain |  | 0.14 | 0.33 | 0.39 | U-shaped | √2 | 1 | 0.10 | 0.23 | 0.28 |
| 9 | Random uncertainty |  | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
|  |  | Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer |  | 0.05 | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. |  | 0.05 | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 | 0.04 |
| 13 | Quality of quiet zone |  | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna |  | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna |  | 0.00 | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
| 16 | Phase curvature |  | 0.05 | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer |  | 0.13 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 |
| 18 | Influence of the reference antenna feed cable |  | 0.05 | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty |  | 0.06 | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable |  | 0.05 | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna |  | 0.5 | 0.43 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna |  | 0.00 | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 0.51 | 0.62 | 0.64 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 0.99 | 1.21 | 1.25 |

We propose the MU for OBUE, SEM, and ACLR absolute requirement of NR FR1 for indoor anechoic chamber as shown in Table 6.

Proposal2: Use Table 6 for MU per point assessment of OTA OBUE, SEM, and ACLR absolute requirement of NR FR1 for indoor anechoic chamber

4.  TRP MU of OTA OBUE, SEM, and ACLR absolute requirement

Since the OTA OBUE, SEM, and ACLR are TRP requirements, the MU for these requirements are calculated with the TRP systematic error of 0.75 by using following equation:

TRP MU = SQRT((Expanded MU per point)2+(TRP systematic error = 0.75)2)

Table 7 shows combined uncertainty and expand uncertainty per point and TRP MU of OTA OBUE, SEM, and ACLR absolute requirements.

| Frequency range | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz | 4.2 GHz < f ≦ 6 GHz |
| --- | --- | --- | --- |
| Total standard uncertainty (1) | 0.51 | 0.62 | 0.64 |
| Expanded uncertainty (2) | 0.99 | 1.21 | 0.25 |
| TRP MU | 1.24 | 1.43 | 1.46 |

Proposal3: Use Table 7 for MU of OTA OBUE, SEM, and ACLR absolute requirement for indoor anechoic chamber for eAAS and NR FR1.

# 4	Summary

In this document, we have discussed the MU of OTA OBUE, SEM and ACLR absolute requirement. We made the following proposals.

Proposal1: Use Table 3 for MU per point assessment of OTA OBUE, SEM, and ACLR absolute requirement of eAAS for indoor anechoic chamber

Proposal2: Use Table 6 for MU per point assessment of OTA OBUE, SEM, and ACLR absolute requirement of NR FR1 for indoor anechoic chamber

Proposal3: Use Table 7 for MU of OTA OBUE, SEM, and ACLR absolute requirement for indoor anechoic chamber for eAAS and NR FR1.

References

[1] R4-1809164, “MU of OTA OBUE, SEM, ACLR absolute requirement,” NTT DOCOMO, June, 2018

[2] R4-1805879, “Test Equipment uncertainty values for FR1”, Keysight, April, 2018

[3] R4-1809725, “Test Equipment uncertainty values update for FR1/FR2”, Keysight, Aug. 2018

[4] R4-161631, “TP for TR37.842: Adding uncertainty value to the EIRP measurement with Indoor Anechoic Chamber,” SEI, DOCOMO, RAN4 #78bis, April, 2016