# 3GPP TSG-RAN WG4 Meeting #88                                 R4-1810486

# Gothenburg, Sweden, 20th - 24th Aug, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-2: FR2 TDD configuration (Section 6.1.1 and 6.1.2)

Agenda Item:	7.9.2.3

Document for:	Approval

1	Introduction

This contribution provide a TP to TS38.141-2[2] for FR2 TDD configuration and general configurations in subclause 6.1.2 based on [1]. In addition, the FR1 NR test models referred in the TS38.141-1 are also clarified.

2	Reference

[1] R4-1810479, Discussion on TDD configurations for NR test model,ZTE

[2] 38.141-2 v0.4.0

--------------Start of TP  -------------

### 6.1.1	FR1 NR Test Models

For the FR1 NR test models, the set-up of physical channels for transmitter tests is defined in the TS38.141-1 sub-clause 6.1.1.

### 6.1.2	FR2 NR Test Models

For the FR2 NR test models, the set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	The test models are defined for a single antenna port (using p = 0); 1 code word (q = 0), 1 layer, precoding is not used; unless specified otherwise

-	Duration is 10 subframes (10 ms)

-	Normal CP

-	Virtual resource blocks of localized type, no intra-subframe hopping for PDSCH

-	UE-specific reference signals are used

For NR TDD, test models are derived based on the slot configurations defined in TS38.213, i.e. as showing in the table 6.1.2-1. Number of frames for the test models is 2.

Table 6.1.2-1: Configurations of TDD gNB test models

| SCS [KHz] | 60 | 120 |
| --- | --- | --- |
| tdd-UL-DL-Configuration, SIB1 |  |  |
| uref [KHz], referenceSubcarrierSpacing | 60 | 120 |
| P [ms],dl-UL-TransmissionPeriodicity | 5 | 5 |
| dslots , nrofDownlinkSlots | 4 | 8 |
| dsym ,nrofDownlinkSymbols | 44 | 88 |
| Uslots ,nrofUplinkSlots | 12 | 24 |
| usym, nrofUplinkSymbols | 8 | 16 |
| tdd-UL-DL-ConfigurationComm2, SIB1 |  |  |
| uref,2 [KHz], referenceSubcarrierSpacing | 60 | 120 |
| P2 [ms], dl-UL-TransmissionPeriodicity | 5 | 5 |
| dslots,2, nrofDownlinkSlots | 20 | 40 |
| dsym,2 , nrofDownlinkSymbols | 0 | 0 |
| uslots,2, nrofUplinkSlots | 0 | 0 |
| usym,2, nrofUplinkSymbols | 0 | 0 |
| Note: slot configuration are defined in the TS38.213 and TS38.331. |  |  |

--------------End of TP ------------