3GPP TSG-RAN4 Meeting #88	R4- 1810510

Gothenburg, Sweden, 20th – 24th August 2018

Source:	Ericsson

Title:	TP to TR: MU framework and values for OTA Base station output power for anechoic chamber for FR2

Agenda item:	7.9.4.3.3.1

Document for:	Approval

# Introduction

This contribution presents our view on the MU budget for the OTA BS output power for FR2 for the anechoic chamber. We believe that similarly to FR1, the MU budget can be based on the EIRP accuracy budget as described below.

# Discussion

Based on OTA radiated transmit power, the framework on uncertainty budget has been reused.

Some issues remain open for the EIRP accuracy MU budget; one is the instrument uncertainty and the second is the need for an LNA.

We believe that the presented figures represent reasonable MU for instrumentation, but the discussion on the values should be taken separately.

Considering a BS output power TRP of around 30dBm and a pathloss (including gain in the measurement probe and cable losses) of e.g. 80-90dB in the far field, the measured TRP level will be around -50 to -60dBm (in up to 400MHz). This is close to the noise floor for a spectrum analyser. Of course, in regions where the power is concentrated there will be array gain and thus the measured power will be greater, and thus it is not clear that an LNA is likely to be needed. Even if an LNA is needed, careful calibration can remove much of the LNA uncertainty from the MU budget.

Indoor anechoic chamber uncertainty contributions for OTA Base station output power measurement

| UID | Description of uncertainty contribution | Details in annex of [1] |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | B1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. | B1-2 |
| 3 | Quality of quiet zone | B1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | B1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | B1-5 |
| 6 | Phase curvature | B1-6 |
| 7 | Uncertainty of the RF power measurement equipment | E |
| 8 | Impedance mismatch in the receiving chain | B1-8 |
| 9 | Random uncertainty | B1-9 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | B1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | B1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer | B1-12 |
| 13 | Quality of quiet zone | B1-3 |
| 14 | Polarization mismatch for reference antenna | B1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | B1-5 |
| 16 | Phase curvature | B1-6 |
| 17 | Uncertainty of the Network Analyzer | E |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | B1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | B1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | B1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E |
| 22 | Uncertainty of the absolute gain of the receiving antenna | B1-18 |

# Indoor anechoic chamber uncertainty assessment for OTA Base station output power measurement

| UID | Uncertainty source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 24.25<f <29.5GHz | Standard uncertainty ui [dB] 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.018 | 0.018 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.07 | 0.07 | Gaussian | 1 |  | 0.07 | 0.07 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.50 | 0.70 | Gaussian | 1 | 1 | 0.50 | 0.70 |
| 8 | Impedance mismatch in the receiving chain | 0.42 | 0.42 | U-shaped | √2 | 1 | 0.30 | 0.30 |
| 9 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.43 | 0.57 | U-shaped | √2 | 1 | 0.30 | 0.40 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.43 | 0.43 | Rectangular | √3 | 1 | 0.25 | 0.25 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.43 | 0.57 | U-shaped | √2 | 1 | 0.30 | 0.40 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.018 | 0.018 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.07 | 0.07 | Gaussian | 1 | 1 | 0.07 | 0.07 |
| 17 | Uncertainty of the network analyzer | 0.30 | 0.30 | Gaussian | 1 | 1 | 0.30 | 0.30 |
| 18 | Influence of the reference antenna feed cable | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 20 | Influence of the receiving antenna feed cable | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.52 | 0.52 | Rectangular | √3 | 1 | 0.30 | 0.30 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.92 | 1.11 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.81 | 2.17 |

The basic budget considered here may need to be combined with any Systematic Error from the TRP process; the size of such a component is the subject of a separate discussion.

# Conclusion

This contribution presents a proposal of the measurement uncertainty framework and budget for OTA Base station output power measurements.

# References

TS 37.842, Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

Huawei, R4-1808500, “Draft CR for 37.843”, RAN4#87, May 2018