3GPP TSG-RAN WG4 Meeting #88	R4-1810533

Gothenburg, Sweden, 20-24 August, 2018

Agenda item:	5.3.2.2

Source: 	Qualcomm Incorporated

Title: 	RSTD measurement accuracy for colliding NPRS based on new NPRS sequence design

Document for:	Discussion

# Introduction

Earlier RAN4 identified the RSTD measurement performance degradation in NB-IoT UE in case of colliding NPRS and sent an LS to RAN1 to address the issue [1]. In the last meeting, RAN1 has sent an LS [2] back to inform the modification in the NPRS sequence generation to address the RSTD measurement performance degradation in case of colliding NPRS.

In this paper, we present the RSTD measurement accuracy simulation result based on the modified NPRS sequence design.

# Simulation Result

Figure 1 and 2 show the CDF of the RSTD measurement error under legacy NPRS sequence and the new NPRS sequence as agreed in [2], in normal coverage and enhanced coverage, respectively.

It is shown that the RSTD measurement accuracy in colliding NPRS scenario becomes comparable to that of non-colliding NPRS scenario when using the modified NPRS sequence that randomizes the interference from other cells across time while the legacy NPRS sequence shows the large degradation in the measurement accuracy in the colliding NPRS scenario.

- Non-colliding NPRS

(b) Colliding NPRS

Figure 1. CDF of the RSTD measurement accuracy of NB-IoT UE in AWGN in normal coverage

- Non-colliding NPRS

(b) Colliding NPRS

Figure 2. CDF of the RSTD measurement accuracy of NB-IoT UE in AWGN in enhanced coverage

Current RSTD accuracy requirement in TS36.133 is written in a way agnostic to the colliding/non-colliding NPRS scenario but should not be applicable to the UE configured with the legacy NPRS sequence in the colliding NPRS scenario. Therefore, we propose to add the side condition to clarify that the existing RSTD measurement accuracy requirement defined in 9.1.22.10-13 is applicable only in the non-colliding NPRS scenario or when the UE is configured with the new NPRS sequence.

Proposal 1. RSTD measurement accuracy requirement for NB1 defined in the sub-clauses 9.1.22.10, …,  9.1.22.13 in TS 36.133 is only applicable when

- the NB-IoT UE is configured with the new NPRS sequence, or

- NPRS from the different measured cells does not collide with each other in the same RE in any of the NPRS subframe

# Conclusions

In this paper, we presented the RSTD measurement accuracy simulation result based on the new NPRS sequence from the recent RAN1 agreement [2]. It is shown that the modified NPRS sequence can resolve the RSTD measurement accuracy issue of the legacy NPRS sequence observed in the colliding NPRS scenario. Summary of the proposals in this paper is captured as follows:

Proposal 1. RSTD measurement accuracy requirement for NB1 defined in the sub-clauses 9.1.22.10, …,  9.1.22.13 in TS 36.133 is only applicable when

- the NB-IoT UE is configured with the new NPRS sequence, or

- NPRS from the different measured cells does not collide with each other in the same RE in any of the NPRS subframe

A companion paper with the draft CR [3] is also prepared to capture the proposal in this paper.

# References

[1] R4-1805486, “LS on NPRS design enhancement for colliding NPRS”, Qualcomm, TSG-RAN4 #86bis

[2] R1-1807567, “LS on Rel-14 NPRS enhancements”, Qualcomm, TSG-RAN1 #93

[3] R4-1810560, “Correction to RSTD measurement accuracy requirement in NB-IoT R14”, Qualcomm TSG-RAN4 #88