3GPP TSG-RAN WG4 Meeting #88	R4-1810534

Gothenburg, Sweden, 20th – 24th Aug, 2018

Agenda item:	6.12.2

Source: 	Qualcomm Incorporated

Title: 	On applicability rule for 8Rx demodulation requirement

Document for:	Discussion

# Introduction

As a part of 8Rx WI, RAN4 needs to determine the applicability rules for the legacy 2Rx/4Rx test cases as well as the new 8Rx test cases. In this paper, we discuss the applicability rule for the demodulation requirement for 8Rx UE.

# Discussion

## Control channel demodulation

During the 8Rx SI phase, it was observed that 4Rx already provides sufficient PDCCH performance and the additional PDCCH Pm-dsg performance improvement from 8Rx is of little practical importance since it is achieved either at the extremely low SNR where UE may well handover to the other cell, or at the unnecessarily low target Pm-dsg much lower than 0.1% that has little impact to the overall UE throughput [1]. Therefore, our view is that UE should not be mandated to use 8Rx for PDCCH demodulation regardless of the PDSCH scheduling or not. To this end, we propose the following applicability rule for the existing 2Rx and 4Rx PDCCH/PCFICH/PHICH/PBCH/ePDCCH demodulation test.

Proposal 1. 8Rx-capable UEs are not mandated to use 8Rx for the demodulation of the broadcast/control channels including PDCCH/PCFICH/PHICH/PBCH/ePDCCH.

Proposal 2. For 8Rx capable UEs, adopt the following applicability rule for the legacy 2Rx and 4Rx-based PDCCH/PCFICH/PHICH/PBCH/ePDCCH demodulation tests:

- Single carrier

  - Test is conducted in 2Rx (or 4Rx) supported RF band, if there exists any.

  - Otherwise, the test is conducted on the 8Rx supported RF bands

    - Fading channel from each Tx antenna is duplicated by 4 (or 2) times and add independent noise for each Rx antenna.

    - Based on the UE’s declaration and AP configuration, the system-simulator-to-receive-antenna mapping ensures that fading channels seen at the declared set of 2 and 4 receive antennas retains the same diversity gain as the 2Rx (or 4Rx) test case.

    - The SNR requirement should remain the same as 4Rx UE.

- CA

  - Test is conducted in 2Rx (or 4Rx) supported RF band combinations, if there exists any

  - Otherwise, the test is conducted based on the band combination including the 8Rx supported RF band(s).

    - For any PCell and/or SCell and/or PSCell on an 8Rx supported RF band, fading duplication and antenna mapping should follow the same principle as single carrier.

Figure 1 shows the two different fading duplication and antenna mapping that could be considered when testing 2Rx requirement on the 8Rx supported RF band. Consider that UE’s PDCCH demodulation uses only 4Rx, RX0-3, and such information is known based on the UE declaration and AP configuration. For such UE, the mapping in Figure 1 (a) can ensure the UE still sees the expected full rank channel after fading duplication, while the mapping in Figure 1 (b) results in a rank-deficient channel from RX0-3 perspective and hence making the test cases more challenging than the original one. Proposal 2 ensures that 8Rx UE, when tested on the 8Rx supported RF bands, is tested based on the fading duplication and antenna mapping scheme according to Figure 1 (a).

(a)

(b)

Figure 1. Fading duplication and antenna mapping diagram for testing 2Rx requirement on the 8Rx supported RF band: (a) Connection with full rank sub-channel for RX0-3, (b) Connection with rank deficient sub-channel for RX0-3

## Transmission mode

From the capability signaling perspective, there is no signaling to indicate that UE supports a certain number of receive antennas. The number of receive antennas is implied by the maximum number of layers UE supports on a certain band. However, in 8Rx case, 8-layer transmission is strictly tied with the transmission mode 9 and 10. Considering that CRS-based transmission mode can support at most rank-4 transmission, whether UE would use 8Rx-based demodulation for CRS-based transmission modes is neither clear nor mandated. Therefore, for the new or legacy PDSCH demodulation requirement and SDR requirement based on CRS-based transmission, whether to tighten the SNR requirement or not should be based on the manufacturer declaration on the 8Rx support in the CRS-based transmission modes. To this end, we propose the following principles regarding the PDSCH demodulation requirement for CRS-based transmission modes:

Proposal 3. Use the manufacturer declaration to determine whether UE supports 8Rx-based PDSCH demodulation when not configured with TM9 or TM10.

Proposal 4. New 8Rx TM2 and TM3 demodulation requirement is tested only for the UEs declared supporting 8Rx-based PDSCH demodulation even when not configured with TM9 or TM10.

Proposal 5. For 8Rx capable UEs, adopt the applicability rule for the legacy 2Rx and 4Rx-based PDSCH demodulation test and SDR test as follows:

- Single carrier

  - Follow the same applicability rule and the fading duplication/mapping rule as PDCCH

  - When tested on the 8Rx supported RF band, SNR requirement is tightened by [3.0]dB for the 2Rx test cases and [1.5]dB for 4Rx test cases if

    - The test is defined under TM9 or TM10, or

    - The test is defined under the TM other than TM9 and TM10, and the UE is declared to support 8Rx-based PDSCH demodulation when not configured with TM9 or TM10

- CA

  - Follow the same applicability rule and the fading duplication/mapping rule as PDCCH

  - For any serving cell on the 8Rx supported RF band(s), SNR requirement is tightened by [3.0]dB for the 2Rx test cases and [1.5]dB for 4Rx test cases if

    - The test is defined under TM9 or TM10, or

    - The test is defined under the TM other than TM9 and TM10, and the UE is declared to support 8Rx-based PDSCH demodulation when not configured with TM9 or TM10

## Advanced receiver tests

In 4Rx WI, for 4Rx capable UE without any 2Rx supported RF bands, the 2Rx test requirement for advanced receivers captured in Table 8.1.2.6.1-0 in TS36.101 was decided to be not applicable.

Table 8.1.2.6.1-0 Requirement lists for 8Rx capable UEs [TS 36.101]

Requirement lists

Enhanced downlink control channel performance requirements type A

Enhanced downlink control channel performance requirements type B

Enhanced performance requirements type B

Enhanced performance requirements type C

Requirements with demodulation subframe overlaps with aggressor cell ABS

Requirements with demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured

Requirements with CRS assistance information configured

RAN4 can follow the same approach and consider any 2Rx/4Rx requirement for the advanced receiver type A/B/C, ABS, and CRS assistance information would not be tested for 8Rx capable UE without any 2Rx/4Rx supported RF bands.

Proposal 6. For 8Rx capable UE without any 2Rx (or 4Rx) supported RF bands, the test cases regarding 2Rx (or 4Rx) requirement for the advanced receiver type A/B/C, ABS, and CRS assistance information can be skipped.

## 4Rx fallback condition

While 8Rx does provide additional diversity/array/multiplexing gain, enabling eight RF receive chains and running 8Rx demapper also comes with the substantial power/cycle impact in the UE. Therefore, it is of interest to identify potential scenarios where UE should be allowed to fallback to 4Rx (or 2Rx) when the power/performance tradeoff cannot be justified.

First of all, following the assumptions accepted during 4Rx WI, 8Rx UE should be allowed to operate with fewer number of receive antennas, 2Rx or 4Rx, when the UE is consistently not scheduled with any unicast PDSCH data.

Proposal 7. 8Rx capable UE is allowed to fall back to 2Rx when UE is not configured with any unicast PDSCH data.

Fixed lower rank combined with high SNR could be another scenario where 8Rx may not provide any performance benefit. When the UE is configured with TM1, 2, or 6, or other TM’s with some codebook subset restriction to disallow higher rank, using only 4Rx could be often sufficient to achieve the spectral efficiency limit at high SNR, e.g., reporting CQI = 15, or achieving negligible BLER even for the highest MCS not exceeding the code rate limit.

Observation 1. When UE is high-layer configured with a fixed rank, based on the transmission mode or codebook subset restriction, 8Rx may not provide additional performance benefit at a moderate to high SNR where 4Rx based demodulation readily achieves the spectral efficiency limit of the given rank.

For the UE in the scenario according to observation 1, the UE may safely disable 4 out of 8Rx and falls back to 4Rx mode. However, since the demodulation requirements in RAN4 are typically defined based on the SNR at which the 70% of the maximum configured throughput is achieved and the corresponding FRCs are sufficiently below the spectral efficiency limit, UE’s 4Rx fallback behavior based on the observation 1 won’t affect the outcome of the legacy 2Rx/4Rx demodulation requirements when following the applicability rule proposed by Proposal 1-7.

# Conclusions

In this paper, we discussed the open issues in the applicability rule for 8Rx capable UE. Observations and proposals made in this paper is summarized as follows:

Proposal 1. 8Rx-capable UEs are not mandated to use 8Rx for the demodulation of the broadcast/control channels including PDCCH/PCFICH/PHICH/PBCH/ePDCCH.

Proposal 2. For 8Rx capable UEs, adopt the following applicability rule for the legacy 2Rx and 4Rx-based PDCCH/PCFICH/PHICH/PBCH/ePDCCH demodulation tests:

- Single carrier

  - Test is conducted in 2Rx (or 4Rx) supported RF band, if there exists any.

  - Otherwise, the test is conducted on the 8Rx supported RF bands

    - Fading channel from each Tx antenna is duplicated by 4 (or 2) times and add independent noise for each Rx antenna.

    - Based on the UE’s declaration and AP configuration, the system-simulator-to-receive-antenna mapping ensures that fading channels seen at the declared set of 4 receive antennas retains the same diversity gain as the 2Rx (or 4Rx) test case.

    - The SNR requirement should remain the same as 4Rx UE.

- CA

  - Test is conducted in 2Rx (or 4Rx) supported RF band combinations, if there exists any

  - Otherwise, the test is conducted based on the band combination including the 8Rx supported RF band(s).

    - For any PCell and/or SCell and/or PSCell on an 8Rx supported RF band, fading duplication and antenna mapping should follow the same principle as single carrier.

Proposal 3. Use the manufacturer declaration to determine whether UE supports 8Rx-based PDSCH demodulation when not configured with TM9 or TM10.

Proposal 4. New 8Rx TM2 and TM3 demodulation requirement is tested only for the UEs declared supporting 8Rx-based PDSCH demodulation even when not configured with TM9 or TM10.

Proposal 5. For 8Rx capable UEs, adopt the applicability rule for the legacy 2Rx and 4Rx-based PDSCH demodulation test and SDR test as follows:

- Single carrier

  - Follow the same applicability rule and the fading duplication/mapping rule as PDCCH

  - When tested on the 8Rx supported RF band, SNR requirement is tightened by [3.0]dB and [1.5]dB for the 2Rx and 4Rx test cases, respectively, if

    - The test is defined under TM9 or TM10, or

    - The test is defined under the TM other than TM9 and TM10, and the UE is declared to support 8Rx-based PDSCH demodulation when not configured with TM9 or TM10

- CA

  - Follow the same applicability rule and the fading duplication/mapping rule as PDCCH

  - For any serving cell on the 8Rx supported RF band(s), SNR requirement is tightened by [3.0]dB and [1.5]dB for the 2Rx and 4Rx test cases, respectively, if

    - The test is defined under TM9 or TM10, or

    - The test is defined under the TM other than TM9 and TM10, and the UE is declared to support 8Rx-based PDSCH demodulation when not configured with TM9 or TM10

Proposal 6. For 8Rx capable UE without any 2Rx (or 4Rx) supported RF bands, the test cases regarding 2Rx (or 4Rx) requirement for the advanced receiver type A/B/C, ABS, and CRS assistance information can be skipped.

Proposal 7. 8Rx capable UE is allowed to fall back to 2Rx when UE is not configured with any unicast PDSCH data.

Observation 1. When UE is high-layer configured with a fixed rank, based on the transmission mode or codebook subset restriction, 8Rx may not provide additional performance benefit at a moderate to high SNR where 4Rx based demodulation readily achieves the spectral efficiency limit of the given rank.

# References

[1] R4-17010311, “Simulation result for 8Rx-based PDCCH and PDSCH demodulation”, Qualcomm, TSG-RAN4 #84bis