3GPP TSG-RAN WG4 Meeting #88	R4-1810542

Gothenburg, Sweden, 20 - 24 August, 2018

Agenda item:	6.9.1.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on open issues in idle mode Scell candidate measurement

Document for:	Discussion

# Introduction

RAN4 has been discussing the RRM requirement for the idle mode SCell measurement and the way-forward [1] was agreed in the last RAN4 #87 meeting to capture the remaining open issues in the idle mode SCell measurement.

In this paper, we discuss the open issues in the idle mode Scell candidate measurement.

# Discussion

In the last RAN4 meeting, the following way-forward [1] was agreed to capture the open issues in the idle mode Scell candidate measurement.

- Aspects for further discussions: - Minimum number of cells and inter-frequency layers to measure: - UE can be configured with up to 3 carriers. - As agreed in RAN4#86bis, UE measurement capability is not affected by euCA. - Number of cells follows existing idle mode measurement requirements. - Detected cell condition for early measurement reporting - Whether to include or not the presence of the previous valid measurement report during connected mode - Measurement time limitation for SIB5 based configuration: - No limit - RAN4 defined limit (x seconds, T331 or other) - Measurement accuracy: - Will be discussed during the performance part. - Detected cells in connected: - T331 and SIB5

## Minimum number of cells and inter-frequency layers to measure

For the minimum number of frequency layers to monitor, we can take the same approach as considered in defining the measurement period for the idle mode SCell candidate measurement. More specifically, when the serving cell quality does not meet the S criteria and UE is supposed to perform the periodic inter-frequency measurement for the possible inter-frequency cell reselectoin, UE is required to provide the measurement report for up to 3 “overlapping” carriers. Otherwise, UE is required to measure at least one carrier, provided that there exists at least one detected/known cell in that carrier. Note that per recent RAN2 agreement [2], UE is not supposed to report the inter-frequency layer that cannot be supported in combination with the frequency layer UE is currently camping on per its CA band combination capability.

Proposal 1. Introduce the following requirement regarding the minimum number of inter-frequency layers to measure for SCell candidate:

- UE measures up to three overlapping frequency layers when the UE is already performing the inter-frequency measurement for potential cell reselection

- Otherwise, UE measures at least one frequency layer, provided that

  - there exists at least one detected/known cell on the concerned frequency layer, and

  - the concerned frequency layer is among the supported CA band combination with respect to the frequency UE is camping on

Regarding the minimum number of cells to measure per frequency layer, our view is that it does not need to be specified, which implicitly means that UE measures and send the measurement report for at least one cell of the concerned frequency layer. Note that existing requirements for inter-frequency measurement in the idle state do not specify the minimum number of cells that UE should measure either. Given that SCell candidate measurement is not related to the idle mode mobility, it makes even more sense not to mandate more than one cells to measure per frequency.

Proposal 2. Do not specify the minimum number of cells to measure per frequency layer.

## Detected cell condition for early measurement reporting

One remaining open issue regarding the detected cell condition is whether to include or not the presence of the valid measurement report for the concern cell during the connected state. While such condition was included when defining the “known” cell condition in the SCell activation delay requirement, a valid point was made in the last meeting that it is possible that UE might have detected a certain detectable cell yet did not send any measurement report due to none of the reporting conditions is met. Considering that the actual SCell activation delay test for known cell does not configure such measurement reporitng either, we propose not to include the the presence of the previous valid measurement report during the connected mode in the detected cell conditions.

Proposal 3. Do not include the presence of the previous valid measurement report in the detected cell condition.

## Handling for SIB5 based configuration

In the last meeting, another use case was identified where UE is provided with only T331 timer in the RRCConnectionRelease message without any dedicated SCell candidate lists. T331 timer in this case is supposed to be used in combination with the the SCell candidate list indicated by SIB5, provided that the acquired SIB5 remains valid. Therefore, we propose the following modification on the basic assumption for the detected cell:

- Basic assumptions:

  - A detected cell in connected mode prior to connection release is assumed to remain in detected state when UE has entered Idle mode, provided that the following conditions are met:

    - The UE is provided with a list of cells and/or carrier frequencies for early measurement reporting by

      - Dedicated RRC signaling

      - SIB5, provided that the acquired system information remains valid by the time the UE is provided with T331

    - The detected cell is among the list of cells and/or on the carrier frequencies provided for early measurement reporting

    - The UE is provided with T331 by dedicated RRC signaling, or optionally regardless of the presence of T331 up to UE implementation

    - A detected cell remains detectable until UE reconnects to the network and sends the early measurement reporting

  - Conditions for a detected cell applies for serving cells and known cells in connected mode.

  - Idle mode measurement requirements apply for carriers configured for mobility.

Proposal 4. When T331 timer is provided without a dedicated frequency list, the detected cell condition and measurement time limitation applies to the list of cells and/or carrier frequencies indicated by SIB5, provided that the acquired SIB5 remains valid by the time UE is provided with T331.

# Conclusion

In this paper, we discussed the remaining open issues in the idle mode Scell candidate measurement. The proposals based on the discussion are summarized as follows:

Proposal 1. Introduce the following requirement regarding the minimum number of inter-frequency layers to measure for SCell candidate:

- UE measures up to three overlapping frequency layers when the UE is already performing the inter-frequency measurement for potential cell reselection

- Otherwise, UE measures at least one frequency layer, provided that

  - there exists at least one detected/known cell on the concerned frequency layer, and

  - the concerned frequency layer is among the supported CA band combination with respect to the frequency UE is camping on

Proposal 2. Do not specify the minimum number of cells to measure per frequency layer.

Proposal 3. Do not include the presence of the previous valid measurement report in the detected cell condition.

Proposal 4. When T331 timer is provided without a dedicated frequency list, the detected cell condition and measurement time limitation applies to the list of cells and/or carrier frequencies indicated by SIB5, provided that the acquired SIB5 remains valid by the time UE is provided with T331.

# Reference

[1] R4-1807545, “Way forward on euCA RRM requirements”, Nokia, Nokia Shanghai Bell, RAN4 #87, May 2018

[2] R2-1809271, “LS on final RAN2 agreements for euCA”, Nokia, Nokia Shanghai Bell, RAN2 #102, May 2018