3GPP TSG RAN WG4 Meeting #88	R4-1810609

Gothenburg, Sweden, August 20 – 24,  2018

Agenda Item:	7.11.8.3

Source: 	Ericsson

Title:	SCell activation for SCell in FR2

Document for:	Discussion

# Introduction

During RAN4 #AH1807 it was agreed that for SCell activation of SCell in FR2, a differentiation regarding the activation delay requirement was to be made based on whether there already is an activated serving cell in the FR2 band to which the SCell-to-be-activated belongs. The justification is that inter-cell timing requirements are very tight for CA in FR2, and the UE therefor would have precise knowledge of the timing, including where to find SSBs, of the SCell-to-be-activated. Moreover, due to UE complexity-related restrictions, the baseline UE would have to receive the SCell-to-be-added using the same Rx beams as used for receiving the active serving cell(s) in the same band, and therefor Rx beam sweeping would not be needed.

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell in SCG.

The delay within which the UE shall be able to activate the deactivated SCell in SCG depends upon the specified conditions.

Upon receiving SCG SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+ [THARQ + Tactivation_time + TCSI_Reporting ]

Where:

THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

[...]

If the SCell being activated belongs to FR2, and there is at least one active serving cell on that FR2 band, then Tactivation_time is [3ms+ TBD],

If the SCell being activated belongs to FR2, and there is no active serving cell on that FR2 band, then, Tactivation_time is [3ms+ TBD+2ms].

Editor’s note1: N1 is the scaling factor due to Rx beam sweeping, and the value is [TBD].

Editor’s note2: Whether it is necessary to differentiate known/unknown for the case of “SCell being activated belongs to FR2, and there is at least one active serving cell on that FR2 band” is FFS.

TCSI_reporting is the delay uncertainty in acquiring the first available CSI reporting resources as specified in [2].

[...]

The agreement was captured in the draft CR of 38.133 15.2.1 [1] (here high-lighted and with change marks removed):

In this contribution we are further discussing the activation times for the case when SCell being activated belongs to FR2, and there is at least one active serving cell in the FR2 band.

# Discussion

Companies provided different proposals to RAN4#AH1807 on the SCell activation time when there is already an active serving cell in the concerned FR2 frequency band, ranging from 0 to 4 SMTC periods plus a few milliseconds margin.

In our view, if the SCell is known, the UE already has a good starting point for the AGC and the fine tuning of the timing. The UE may still need to fine tune both AGC and cell timing, but can do this simultaneously by conducting power measurements while receiving and further processing the SSB. Any modifications of AGC setting and cell timing can then be done prior to receiving CSI-RS for CQI estimation (staggered approach). Hence, for the case of known SCell with already active serving cell(s) in the same FR2 frequency band, we propose the activation time delay to be up to 1 SMTC period plus some milliseconds overhead.

Proposal 1: For activation of known SCell, with already activated serving cells in the same FR2 band, Tactivation_time is 1 SMTC period + X, where 3ms ≤ X ≤ 5ms.

Further, in our view if the SCell is unknown, it should in principle be possible to meet the same activation time as had the SCell been known, provided that there already is an active serving cell in the same FR2 band. However, since there is more uncertainty involved since there are no previous power measurements or gain states for the subband available, and there is no previous timing information and the UE therefor has to consider a timing difference of up to ±3us relative to the serving cell, we can consider having slightly relaxed requirements compared to the case of known SCell. “Slightly relaxed” since the UE still should be able to use the serving cell gain state as starting point when determining a suitable gain when adding the concerned SCell, and hence would not have to go through all steps associated with a gain search.

Proposal 2: Introduce differentiated requirements based on whether the SCell-to-be-activated is known or unknown to the UE. If the latter, one or few more SMTC periods activation delay can be considered for handling gain and timing uncertainties.

# Summary and Conclusions

In this contribution we have provided our view on activation times for activation of SCell in FR2 when there already is an active serving cell in the same FR2 band. We have the following proposals.

Proposal 1: For activation of known SCell, with already activated serving cells in the same FR2 band, Tactivation_time is 1 SMTC period + X, where 3ms ≤ X ≤ 5ms.

Proposal 2: Introduce differentiated requirements based on whether the SCell-to-be-activated is known or unknown to the UE. If the latter, one or few more SMTC periods activation delay can be considered for handling gain and timing uncertainties.

# References

- Draft CR TS 38.133 v15.2.1 (2018-07) circulated on the RAN4 reflector