TSG-RAN Working Group 4 (Radio) meeting #88	R4-1810618

# Gothenburg, Sweden, 20th – 24th August 2018

Source:	Ericsson

Title:	TP to TS 38.141-02: Section 6.7.2 OTA Occupied Bandwidth

Agenda item:	7.9.4.2.2

Document for:	Approval

# 1.	Introduction

In the last RAN4 meeting test specification for section occupied bandwidth both for radiated and conducted was agreed.  It was noticed that sections in TS 38.141-02 was not aligned to TS 38.141-01 for FR1 tables.  In this contribution, the update to align this table has been made.

Additional FFS in FR2 table was also filled with proposed values.

# 2.	References

- 3GPP TS 38.141-2 v0.3.0

[Start of Text Proposal]

#### 6.7.2.4.2	Procedure

The OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.6.x, Dx.x) of the BS with the test system.

3) Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit signal.

6)	Measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.7.2.4.2-1 and table  6.7.2.4.2-2. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding. There are at least two ways to be power responding. The spectrum analyser can be set to "sample" detection, with its video bandwidth setting at least three times its RBW setting. Or the analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

Table 6.7.2.4.2-1: Span and number of measurement points for OBW measurements for NR FR1

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | > 20 | > 20 |
| Span (MHz) | 10 | 20 | 30 | 40 |  |  |
| Minimum number of measurement points | 400 | 400 | 400 | 400 |  |  |
| NOTE 1: T = 200 kHz, when the BS channel bandwidth of outermost carriers are both larger than 50 MHz; Otherwise, T = 100 kHz. |  |  |  |  |  |  |

Table 6.7.2.4.2-2: Span and number of measurement points for OBW measurements for NR FR2

| Bandwidth |  | BS channel bandwidth BWChannel (MHz) |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 |  | 100 | 200 | 400 |  | > FFS |  |
| Span (MHz) | 100 |  | 200 | 400 | 800 |  |  |  |
| Minimum number of measurement points | 400 |  | 400 | 400 | 400 |  | 2×BWChannelT |  |
| NOTE 2: T = 400 kHz, when the BS channel bandwidth of outermost carriers are both larger than 400 MHz; Otherwise, T = 200 kHz. |  |  |  |  |  |

7)	Compute the total of the EIRP, P0, (in power units, not decibel units) of all the measurement cells in the measurement span. Compute P1, the EIRP outside the occupied bandwidth on each side. P1 is half of the total EIRP outside the bandwidth. P1 is half of (100 % - (occupied percentage)) of P0. For the occupied percentage of 99 %, P1 is 0.005 times P0. The EIRP calculation depends on whether the test facility supports dual polarization:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Measure and sum the EIRP on both polarizations to obtain P0 or P1.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power as the sum over both polarizations to obtain P0 or P1

8)	Determine the lowest frequency, f1, for which the sum of all EIRP in the measurement cells from the beginning of the span to f1 exceeds P1.

9)	Determine the highest frequency, f2, for which the sum of all EIRP in the measurement cells from the end of the span to f2 exceeds P1.

10)	Compute the OTA occupied bandwidth as f2 - f1.

In addition, for multi-band RIB(s), the following steps shall apply:

11) For multi-band RIBs and single band tests, repeat the steps 6) - 10) above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

[End of Text Proposal]