3GPP TSG-RAN4 Meeting 88 	R4-1810910

# Göteborg, Sweden, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 37.843 Upper bound for reference angular steps in sub-clause 10.9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the original contribution R4- 1805047 a maximum of 15 degree sampling steps was used to make sure that appropriate TRP values are achieved also for electrically small EUTs. EUT dimensions of a couple of wave lengths. This upper bound has been dropped. If not re-introduced the current formula will allow for too sparse angular sampling for small antennas or at low frequencies. The 15-degree step is in line with test methods for UEs proposed by CTIA “Test plan for wireless device Over-the-Air performance — method of measurement for radiated RF power and receiver performance,” Certification Program Test Plan, Nov. 2016, version 3.6.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - General editorial corrections Upper bound on reference angular steps |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, TR 37.843, sub-clause 10.9.5 might allow for too large angular steps for electrically small antennas resulting in un-controlled TRP values, |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sub-clause 10.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------ start of changes------------

#### 10.9.2.1	General

With the spherical equal angle grid, the grid spacing is uniform in the θ and ϕ directions. The range of θ angles from 0 to π is divided into N equally spaced subintervals and the range of ϕ angles from 0 to 2π is divided into M equally spaced subintervals. The width of each subinterval in the θ- and ϕ-angle is given as

Δθ= πN  and ∆ϕ= 2πM

The total number of angular sampling points is equal to (N-∓1)×M.

Let n and m be the indices used to denote the nth θ and mth ϕ angles, respectively. In pactice, discrete samples of EIRP are measured at each sample point (θn, ϕm) by measuring its two orthogonally polarized components, EIRPp1(θn,ϕm) and EIRPp2(θn,ϕm). The EIRP sample are then used to approximate the definite integral for TRPReference as

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn.

The above equation can be simplified considering sinθ0 = sinθN = 0. Thus the total number of angular sampling points is equal to (N – 1)M.

Note TRPEstimate = TRPReference as N and M approach ∞.

There is a trade-off between the accuracy of the TRPEstimate and the total number of sampling points. A large number of sampling points leads to long measurement time. Thus it is important to achieve short measurement time and fulfilling the minimum TRP systematic error. Subclauses 10.9.3.1 and 10.9.3.2 outline the criteria for determining the minimum number of sampling points to characterize. Other means for set the number of sampling points are not precluded.

For each  frequency, the reference angular steps   Δϕ and Δθ in radians are calculated as:

Δϕref=min⁡(λDcyl,π12)

Δθref=min⁡(λD,π12)

Δϕref=λ/Dcyl

Δθref=λ/D

, where D and Dcyl are defined in subclause 10.9.2.1. If  constant angular steps are to be used in a frequency band, the highest frequency must be used when calculating the reference angular steps.

According to [x] and [y] using a grid with reference angular step is enough for accurate error-free TRP estimation. The reference angular steps are dependent only on the electrical size of the radiation source (i.e., physical size and the emission frequency) and is not affected by the correlation level. When sampling with the reference angular step, fine details of the radiation pattern are not observed, but the TRP value is not affected.

The equal angle grid points are not uniformly distributed on the sphere but are clustered towards the poles as shown in Figure 10.9.2-1.

--------- end of changes……….

# References

[x] R4- 1805047 “TP to TR 37.843: TRP test method for unwanted emissions for eAAS BS”, Ericsson

[y] CTIA, “Test plan for wireless device Over-the-Air performance — method of measurement for radiated RF power and receiver performance,” Certification Program Test Plan, Nov. 2016, version 3.6.1.