3GPP TSG-RAN4 Meeting 88 	R4-1810937

# Gothenburg, Sweden, 20th – 24th August 2018

Source:	Ericsson

Title:	Definition of TRP related errors

Agenda item:	6.1.3.4.1

Document for:	Discussion

# Introduction

Based on [1] we see a need to agree on the definitions of TRP related errors

# Background

The TRP procedures based on measurements in an ShAC or a CATR is based on measurement of power density or EIRP in a set of directions on a sphere. These results are obtained with a certain MU according to the measurement procedure used. Thereafter, these values are numerically integrated. The error of the numerical integration depends on the grid type and integration method, and the number of grid points used

Number of points used

Grid type and integration method

Power density in one direction

Figure 1 Three parts of the TRP assessment procedure. Each part corresponds to an error of the final TRP value. Note, the choice of grid type and integration method is not completely independent.

The reference (true) TRP can be defined as the limit where an infinite number of points, covering a full sphere, are used. However, for any size of radiating object there is a certain angular resolution (denoted reference angular steps [3]) below which the integration error rapidly decreases to practically zero, see also [2,4-5]. Especially if an FFT-based integration method is used [4-5]. One example of an FFT-based integration method is to use a Spherical Wave Expansion based on FFT and then calculate the TRP value using this expansion [5]. However, the most widespread integration method for TRP is

TRP= m,npDθm,ϕnr2sinθmΔθmΔϕn

This integration method can have an error in the ball park 0.5 to 1 dB at the reference angular step, see [4,6], but in other cases a much lower value can be reached [2].

Note: For EIRP values the relation 14πEIRPθ,ϕ=pD(θ,ϕ)r2 can be used.

We propose to add the integration error at reference steps as an uncorrelated error to the per-point MU.

Only for spurious emissions (excluding harmonic frequencies) where the sources of radiation can be assumed to have a low correlation, the effect of selecting sparser sampling than the reference steps is recommended. For correlated sources with high beam-forming gain the use of sparse grids can come with huge errors. The method to calculate the error due to sparsity of grid points has been discussed but is not yet agreed [7].

In some contributions and discussions, like in the second paragraph of Sec. 10.9.1 of TR 37.843, we have observed that the TRP is compared to the conducted power. The difference between the conducted power and the TRP is not an error, it is the combined mis-match loss at the antenna conductor and the radiation efficiency. This parameter can only be calculated for antennas with an available connector to a matched transmission line. This is not the case for an AAS. Hence the reference TRP should not be the conducted power.

Definition 1: The per-point uncertainty (MU) is the uncertainty given by the measurement procedure for power density (or EIRP).

Definition 2: The TRP integration error (IE) is the error of the numerical integration at reference angular steps, compared to the true TRP.

Definition 3: The TRP systematic correction factor is the added value used to ensure that the TRP value is over-estimated by a certain confidence (if not stated 95% confidence is understood).

We propose to calculate the uncertainty of the TRP estimate (using reference angular steps) using

MU2+IE2

What is called systematic error arises form numerical integration and we prefer a more descriptive name in Definition 2. Moreover, this error can be partly systematic and partly random in nature.

When performing the sums in the numerical integration it is noted that random uncorrelated errors will reduce when evaluating the sums. However, systematic errors in the “per-point” values will add in a systematic way. This error propagation is a delicate matter and we will not dive further into it. Based on these observations, and assuming an IE clearly below the per-point MU, we propose to set the IE to identically zero.

# Conclusions

Conclusion 1: To move forward we need to agree on the definition of the TRP related errors and their relation to the MU of power density or EIRP values.

Proposal 1: The per-point uncertainty (MU) is defined as the uncertainty given by the measurement procedure for power density (or EIRP).

Proposal 2: The TRP integration error (IE) is defined as the error of the numerical integration at reference angular steps, compared to the true TRP.

Proposal 3: The TRP systematic correction factor is defined as the factor that is added to the TRP estimate used to ensure that the TRP value is over-estimated by a certain confidence (if not stated 95% confidence is understood).

Proposal 4: Calculate the uncertainty of the TRP estimate (using reference angular steps) as

MU2+IE2

Proposal 5: Numerical values to use: MU= uncertainty for pointwise measurements including alignment errors. IE = 0.

# References

[1] R4-1809436 Way forward on TRP systematic error, Nokia, Nokia Shanghai Bell, NTT DoCoMo

[2] R4-1809205 Discussion on TRP systematic error and MU, Huawei

[3] R4-1807481 Reference angular steps for TRP measurements, Ericsson

[4] R4-1807482 Grid summation method for TRP, Ericsson

[5] Hald, J.; Hansen, J. E.; Jensen, F. & Holm Larsen, F. Hansen, J. (Ed.) Spherical Near-Field Antenna Measurements, Peter Peregrinus Ltd., 1998, vol 26

[6] R4-1804463 On TRP Measurement Grids for mm-wave, Ericsson

[7] R4-1809093 Sparse grid concept for TRP measurements, Ericsson