3GPP TSG-RAN WG4 #88	R4-1811106

Goteborg, Sweden, August 20th – 24th, 2018

Agenda item:	7.6.17.4

Source: 	Qualcomm Incorporated

Title: 	Why an OOB blocking requirement for FR2 is needed

Document for:	Approval

# Introduction

Although several contributions proposing an OOB blocking (OOBB) requirement were made [1], the current version of TS 38.101-2 does not specify any requirement yet. Since RAN4 #88 is the last meeting available to specify such requirement for Rel15 specifications, in this contribution we try to clarify why it is important to include an OOBB requirement for FR2 specifications.

# Discussion

Discussion about OOBB requirement for FR2 started several meetings ago without the possibility to reach an agreement. In [2], we gave an overview of possible systems which could be blockers for NR FR2 UEs. Given the expected blocker level and the probability to get interfered by Local Multipoint Distribution Service (LMDS) point-to-point fixed systems, RAN4 do not include a requirement for protection of fixed services (FS), while protection against Fixed Satellite Services operating in UL (earth to space) could be achieved through exclusion region.

Based on the above observations, in [3] we proposed to define a different kind of OOBB requirement for FR2. In particular, our preference is to define a requirement which is based on the principle of protection in FR2 band against a blocker operating in another FR2 band available on the same region. The main objections raised to this proposal were the following:

- Testing time

- Why is the requirement needed if we already have IBB requirement which is similar aggressor level?

Regarding the testing time, we already replied to the comments in [5] saying that the number of test points will be down-selected by RAN5 and that, if needed a down selection of the frequency location of blocker/wanted signals can be implemented in RAN4 specification.

Observation 1: as for other RF requirements, RAN5 can down-select the number of test points for the OOBB test. If needed, RAN4 can also provide a down-selection of the frequency locations for blocker/wanted signals.

Regarding the need to define a requirement which has same level of IBB, we already commented online why this is needed, however we will provide a further example below. One of the issue is that since IF and RF LO location are not specified a blocker operating in an FR2 band X could become an image for a receiver operating in FR2 Y. This will happen anytime FR2 blocker in band X is placed at RF LO – IF. A pictorial illustration of this issue is shown in Figure 1.

Figure 1. Pictorial illustration of blocking impact from an FR2 blocker located at RF LO – IF.

Observation 2: depending on IF and RF LO location, an OOBB operating on one FR2 band could become image for a receiver operating on another FR2 band in the same region.

Based on the above observation we propose again to specify an OOBB requirement for FR2 specifications based on the criteria of protecting one FR2 band against another FR2 band operating in the same region. The proposal is implemented in the draft CR to TS 38.101-2 available in [5].

Proposal: to define an OOBB requirement based on the principle to protect one FR2 band against another FR2 band operating in the same geographical region.

# Conclusions

In this contribution we addressed some concerns raised about the introduction of an OOBB requirement for FR2 specifications by observing the following:

Observation 1: as for other RF requirements, RAN5 can down-select the number of test points for the OOBB test. If needed, RAN4 can also provide a down-selection of the frequency locations for blocker/wanted signals.

Observation 2: depending on IF and RF LO location, an OOBB operating on one FR2 band could become image for a receiver operating on another FR2 band in the same region.

Accordingly, we made the following proposal:

Proposal: to define an OOBB requirement based on the principle to protect one FR2 band against another FR2 band operating in the same geographical region.

The proposal is implemented in the draft CR to TS 38.101-2 available in [5].

# Reference

- R4-1807654, “Update of OOBB requirement for FR2”, Qualcomm Incorporated.

- R4-1713639, “On UE OOB blocking requirement for mmW”, Qualcomm Incorporated.

- R4-1802829, “Definition of UE OOB blocking requirement for FR2”, Qualcomm Incorporated.

- R4-1804139, “Out-of-Band-Blocking requirement for FR2”, Intel Corporation.

- R4-1811105, “Update of OOBB requirement for FR2”, Qualcomm Incorporated.