3GPP TSG-RAN WG4 Meeting #88                                                                  R4-1811150

Gothenburg, SE, 20 - 24 Aug 2018

Source: 	Huawei, HiSilicon, Etisalat

Title: 	TP: Add parameters band n50 in TS 38.141

Agenda Item:	7.9.4.3.4.2

Document for:	Approval

# Background

After RAN4-AH-1807 meeting in Montreal, the draft TS 38.141-2 was revised by Email. The text proposed below is to update this draft TS 38.141 in correct and add several parameters about the band n50.

# Text Proposal

<Start of the test proposal>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ,REFSENS 	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees

BWChannel	BS channel bandwidth

BWChannel_CA 	Aggregated BS Channel Bandwidth, expressed in MHz. BWChannel_CA = Fedge_high- Fedge_low

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12

f 	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax 	f_offsetmax minus half of the bandwidth of the measuring filter

ΔFGlobal 	Global frequency raster granularity

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔminSENS 	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS 	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster 	Channel raster granularity

Δsample               	The difference between the nominal and extreme power measurements during extreme EIRP testing, Pmax,sample,nom  - Pmax,sample,ex

ΔSUL	Channel raster offset for SUL

EISminsens		The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FC	RF reference frequency on the channel raster

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC_low 	The Fc of the lowest carrier, expressed in MHz

FC_high 	The Fc of the highest carrier, expressed in MHz

Fedge_low 	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset_low

Fedge_high 	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset_high

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset_low

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset_high

Foffset_high 	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge, or from F C,block, high to the upper sub-block edge

Foffset_low 	Frequency offset from FC_low to the lower Base Station RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF,SUL		RF reference frequency for Supplementary Uplink (SUL) bands

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

NRB	Transmission bandwidth configuration, expressed in resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

PEM,n50,ind	Declared emission level for Band n50 in the band 1518-1559 MHz; ind = a, b

Pmax,c,EIRP 	The maximum carrier EIRP when the NR BS is configured at the maximum rated carrier output TRP (PRated,c,TRP)

Pmax,c,EIRP, extreme 	The maximum carrier EIRP when the NR BS is configured at the maximum rated carrier output TRP (PRated,c,TRP) under extreme conditions, either measured directly or calculated

Pmax,c,TRP	Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

Pmax,sample,nom             The measured sample power in extreme conditionals chamber when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP), under nominal conditions

Pmax,sample,ext      	The measured sample power in extreme conditionals chamber when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP), under extreme conditions

PRated,c,EIRP           The rated carrier output EIRP when the NR BS is configured at the rated carrier output TRP (PRated,c,TRP)

Prated,c,TRP	Rated carrier TRP output power declared per RIB

Prated,t,TRP	Rated total TRP output power declared per RIB

PREFSENS	Conducted reference Sensitivity power levelSSREF	SS block reference frequency position

TTOTA	Test tolerance for OTA requirements

Wgap	Sub-block gap or Inter RF Bandwidth gap size

## 3.3	Abbreviations

<Text avoided>

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Subclause | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Some NR operating bands may be applied regionally. |
| 6.7.5.4 | Additional spurious emissions requirements | These requirements may apply in certain regions as additional Operating band spurious emission limits. |
| 6.6.2, 9.7.2 | Occupied bandwidth, OTA occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in present specification. |
| 6.6.3.4, 9.7.3.2 | Absolute ACLR, OTA absolute ACLR, | The emission limits specified for OTA absolute ACLR as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |
| 6.6.4.2.5.1 | Limits in FCC Title 47 | The BS may have to comply with the additional requirements, when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.4.4, 9.7.4.2 | Operating band unwanted emissions, OTA out-of-band emissions | The emission limits specified for OTA out-of-band emissions as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |
| 6.6.5.2.1, 9.7.5.2 | Tx spurious emissions, OTA Tx spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [2], may apply regionally. The emission limits specified as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |
| 6.6.5.2.3, 9.7.5.3.3 | Tx spurious emissions: additional requirements, OTA Tx spurious emissions: additional requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the BS operating band. |
| 7.6.4, 10.7.2 | Rx spurious emissions, OTA Rx spurious emissions | The emission limits specified as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |

## 4.5	BS configurations

<Text avoided>

##### 6.7.5.4.5	Test Requirement

###### 6.7.5.4.5.1		Test requirement for BS type 1-O

The power of any spurious emission shall not exceed the test limits in table 6.7.5.4.5 -1 for a BS where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the Note column of table 6.7.5.4.5 -1 apply for each supported operating band.

Table 6.7.5.4.5-1: BS spurious emissions test limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Test limit | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| GSM900 | 921 – 960 MHz | -48 dBm + FFS | 100 kHz | This requirement does not apply to BS operating in band n8 |
|  | 876 – 915 MHz | -52 dBm + FFS | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in subclause 6.7.5.4.3 |
| DCS1800 | 1805 – 1880 MHz | -38 dBm + FFS | 100 kHz | This requirement does not apply to BS operating in band n3. |
|  | 1710 – 1785 MHz | -52 dBm + FFS | 100 kHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| PCS1900 | 1930 1990 MHz | -38 dBm + FFS | 100 kHz | This requirement does not apply to BS operating in band n2, n25 or band n70. |
|  | 1850 – 1910 MHz | -52 dBm + FFS | 100 kHz | This requirement does not apply to BS operating in band n2 or n25 since it is already covered by the requirement in subclause 6.7.5.4.3. |
| GSM850 or CDMA850 | 869 – 894 MHz | -48 dBm + FFS | 100 kHz | This requirement does not apply to BS operating in band n5. |
|  | 824 – 849 MHz | -52 dBm + FFS | 100 kHz | This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 – 2170 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n1 |
|  | 1920 – 1980 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n1, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1930 – 1990 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n2 or n70. |
|  | 1850 – 1910 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1805 – 1880 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n3. |
|  | 1710 – 1785 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n66 |
|  | 1710 – 1755 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 869 – 894 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n5. |
|  | 824 – 849 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 | 860 – 890 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 815 – 830 MHz | -40 dBm + FFS | 1 MHz |  |
|  | 830 – 845 MHz | -40 dBm + FFS | 1 MHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2620 – 2690 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n7. |
|  | 2500 – 2570 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n7, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 925 – 960 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n8. |
|  | 880 – 915 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n3. |
|  | 1749.9 – 1784.9 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n66 |
|  | 1710 – 1770 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 – 1510.9 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n50 or n75. |
|  | 1427.9 – 1447.9 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n50, n51, n75 or n76. |
|  | 1447.9 – 1462.9 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n50 or n75. |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 729 – 746 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n12. |
|  | 699 – 716 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n12, since it is already covered by the requirement in sub-clause 6.7.5.4.3. |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 – 756 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 777 – 787 MHz | -40 dBm + FFS | 1 MHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 – 768 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 788 – 798 MHz | -40 dBm + FFS | 1 MHz |  |
| E-UTRA Band 17 | 734 – 746 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 704 – 716 MHz | -40 dBm + FFS | 1 MHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 791 – 821 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n20 or n28. |
|  | 832 – 862 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 3410 – 3490 MHz | -40 dBm + FFS | 1 MHz |  |
| E-UTRA Band 24 | 1525 – 1559 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 1626.5 – 1660.5 MHz | -40 dBm + FFS | 1 MHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70. |
|  | 1850 – 1915 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n25 since it is already covered by the requirement in subclause 6.7.5.4.3. For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in subclause 6.7.5.4.3. |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 – 894 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n5. |
|  | 814 – 849 MHz | -40 dBm + FFS | 1 MHz | For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in subclause 6.7.5.4.3. |
| E-UTRA Band 27 | 852 – 869 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n5. |
|  | 807 – 824 MHz | -40 dBm + FFS | 1 MHz | This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 5). |
| E-UTRA Band 28 or NR Band n28 | 758 – 803 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n20 or n28. |
|  | 703 – 748 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| E-UTRA Band 29 | 717 – 728 MHz | -43 dBm + FFS | 1 MHz |  |
| E-UTRA Band 30 | 2350 – 2360 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 2305 – 2315 MHz | -40 dBm + FFS | 1 MHz |  |
| E-UTRA Band 31 | 462.5 -467.5 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 452.5 -457.5 MHz | -40 dBm + FFS | 1 MHz |  |
| UTRA FDD band XXXII or E-UTRA band 32 | 1452 – 1496 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n50 or n75. |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 – 1920 MHz | -43 dBm + FFS | 1 MHz |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 – 2025 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n34. |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -43 dBm + FFS | 1 MHz |  |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n2 or n25. |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -43 dBm + FFS | 1 MHz |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n39. |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n40. |
| E-UTRA Band 41 or NR Band n41 | 2496 – 2690 MHz | -43 dBm + FFS | 1 MHz | This is not applicable to BS operating in Band n41. |
| E-UTRA Band 42 | 3400 – 3600 MHz | -43 dBm + FFS | 1 MHz |  |
| E-UTRA Band 43 | 3600 – 3800 MHz | -43 dBm + FFS | 1 MHz |  |
| E-UTRA Band 44 | 703 – 803 MHz | -43 dBm + FFS | 1 MHz | This is not applicable to BS operating in Band n28. |
| E-UTRA Band 45 | 1447 – 1467 MHz | -43 dBm + FFS | 1 MHz |  |
| E-UTRA Band 46 | 5150 – 5925 MHz | -43 dBm + FFS | 1 MHz |  |
| E-UTRA Band 47 | 5855 – 5925 MHz | -43 dBm + FFS | 1 MHz |  |
| E-UTRA Band 48 | 3550 – 3700 MHz | -43 dBm + FFS | 1 MHz |  |
| E-UTRA Band 50 or NR Band n50 | 1432 – 1517 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75 or n76. |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75 or n76. |
| E-UTRA Band 65 | 2110 – 2200 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n1, |
|  | 1920 – 2010 MHz | -40 dBm + FFS | 1 MHz | For BS operating in Band n1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in subclause 6.7.5.4.3. |
| E-UTRA Band 66 or NR Band n66 | 2110 – 2200 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n66. |
|  | 1710 – 1780 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| E-UTRA Band 67 | 738 – 758 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n28. |
| E-UTRA Band 68 | 753 -783 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n28. |
|  | 698-728 MHz | -40 dBm + FFS | 1 MHz | For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in subclause 6.7.5.4.3. |
| E-UTRA Band 69 | 2570 – 2620 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n38. |
| E-UTRA Band 70 or NR Band n70 | 1995 – 2020 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70 |
|  | 1695 – 1710 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n70, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| E-UTRA Band 71 or NR Band n71 | 617 – 652 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n71 |
|  | 663 – 698 MHz | -40 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n71, since it is already covered by the requirement in subclause 6.7.5.4.3. |
| E-UTRA Band 72 | 461 – 466 MHz | -43 dBm + FFS | 1 MHz |  |
|  | 451 – 456 MHz | -40 dBm + FFS | 1 MHz |  |
| E-UTRA Band 74 | 1475 – 1518 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in band n50 or n75. |
|  | 1427 – 1470 MHz | -40 dBm + FFS | 1MHz | This requirement does not apply to BS operating in band n50, n51, n75 or n76. |
| E-UTRA Band 75 or NR Band n75 | 1432 – 1517 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75 or n76. |
| E-UTRA Band 76 or NR Band n76 | 1427 – 1432 MHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75 or n76. |
| NR Band n77 | 3.3 – 4.2 GHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n77 and n 78 |
| NR Band n78 | 3.3 – 3.8 GHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n77 and n78 |
| NR Band n79 | 4.4 – 5.0 GHz | -43 dBm + FFS | 1 MHz | This requirement does not apply to BS operating in Band n79 |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in 6.7.5.4.5 -1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.7.5.4.5 -1 assumes that two operating bands, where the frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE:	For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between 10 MHz below the lowest BS transmitter frequency of the downlink operating band and 10 MHz above the highest BS transmitter frequency of the downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.7.5.4.5-2: BS spurious emissions test limits for BS for co-existence with PHS

| Frequency range | Test limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 1884.5 – 1915.7 MHz | -32 + FFS dBm | 300 kHz | Applicable when co-existence with PHS system operating in 1884.5 - 1915.7MHz |

The following requirement may apply to E-UTRA BS operating in Band n41 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.7.5.4.5-3: Additional BS spurious emissions test limits for Band n41

| Frequency range | Test limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 2200 – 2345 MHz | -36 dBm + FFS | 1 MHz |  |
| 2362.5 – 2365 MHz | -16 dBm + FFS | 1 MHz |  |
| 2365 – 2367.5 MHz | -31 dBm + FFS | 1 MHz |  |
| 2367.5 – 2370 MHz | -33 dBm + FFS | 1 MHz |  |
| 2370 – 2395 MHz | -36 dBm + FFS | 1 MHz |  |

In certain regions, the following requirement may apply to NR BS operating in Band n50 within 1432-1452 MHz or n51. Emissions shall not exceed the maximum levels specified in Table 6.7.5.4.5-4.

Table 6.7.5.4.5-4: Additional operating band unwanted emission test limits for NR BS operating in Band n50 within 1432-1452 MHz or n51

| Filter centre frequency, Filter | Maximum Level [dBm] | Measurement Bandwidth |
| --- | --- | --- |
| Ffilter = 1413.5 MHz | -42 + FFS | 27 MHz |

NOTE:	The regional requirement, included in [15], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in 3GPP TS 38.104 [2] Annex E.

In certain regions, the following requirement may apply to BS operating in NR Band n50 within 1492-1517 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to Table 6.6.5.2.3-5, shall neither exceed the maximum emission level PEM,n50,a nor PEM,B50,b declared by the manufacturer.

Table 6.6.5.2.3-5: Operating band n50 within 1492-1517 MHz declared emission in the band 1518-1559 MHz

| Filter centre frequency, Ffilter | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 1518.5 MHz ≤ Ffilter ≤ 1519.5 MHz | PEM, n50,a | 1 MHz |
| 1520.5 MHz ≤ Ffilter ≤ 1558.5 MHz | PEM,n50,b | 1 MHz |

NOTE:	The regional requirement, included in [15], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in 3GPP   TS 38.104 [2] Annex E.

###### 6.7.5.4.5.1		Test requirement for BS type 2-O

Editor’s note: Additional spurious emissions requirement for protecting specific services are ffs.

## 6.8	OTA transmitter intermodulation

<End of the text proposal>