3GPP TSG-RAN WG4 Meeting #88                	R4-1811297

Gothenburg, Sweden, August 20th – 24th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Finalization of euCA RRM requirements

Agenda item:	6.9.1

Document for:	Discussion

Introduction

During the Busan meeting the discussions related to the euCA WI continued with very good progress. A WF was agreed in [7] listing the agreements as well as including some open aspects for further discussion. In this paper, we continue the discussion related to the open RRM aspects related to euCA and early measurement reporting, direct SCell activation and dormant SCell UE requirements.

Discussion

In the following sections we discuss the topics which we see still needs some further clarification and agreements related to RRM requirements for a UE supporting euCA. We look at Idle mode measurements for fast CA setup, Direct SCell activation and Dormant SCell requirements.

2.1.1	Idle mode measurements for fast CA setup

The basic frame work for idle mode measurements for fast CA setup were agreed in Busan meeting and captured in Stage-2, MAC, UE capability and RRC CRs [2, 3, 4, 5] and in draft baseline CRs [6], leaving the following open issues to be further discussed – which we give our preference in line:

- Minimum number of cells and inter-frequency layers to measure:

  - UE can be configured with up to 3 carriers.

    - Nokia: 3 carriers was decided in RAN2 to align with existing inter-frequency measurement requirements, and is in the baseline CR [6]

  - As agreed in RAN4#86bis, UE measurement capability is not affected by euCA.

    - Nokia: This was already agreed and captured in the baseline CR [6]

  - Number of cells follows existing idle mode measurement requirements.

    - Nokia: We do not see a need for increasing the existing requirements. Existing requirement is used in the baseline CR [6]

- Detected cell condition for early measurement reporting

  - Whether to include or not the presence of the previous valid measurement report during connected mode

    - Nokia: As expressed in the earlier discussions, we think having as accurate measurements reported will be beneficial from system point of view. I.e. both for the UE, the network and the user. More precise measurements help network in the decision making configuring the UE with the most optimal CA configuration in an early phase of the connection – which has been shown to give significant user experience improvements. This should be reflected in the final accuracy requirements – how the UE fulfil such accuracy of course need to account UE complexity and power savings while still accounting also the network needs.

Include the the previous valid measurement report during connected mode.

- Measurement time limitation for SIB5 based configuration:

  - No limit or RAN4 defined limit (x seconds, T331 or other)

    - Nokia: From network point it would often be beneficial to receive early measurement results from the UE once it enters Connected mode. If obtaining such measurements on UE are not causing noticeable power penalty we do not see any immediate reason why to introduce any time limitations. If seen necessary to introduce a time limitation due to UE power consumption concerns, such time limit could be defined e.g. as a RAN4 defined timer.

Do not introduce any time limitations for SIB5 limitation.

- Detected cells in connected:

  - T331 and SIB5

    - Nokia: this relates for how long time the connected mode detected cell is assumed being detectable. As such we do not see why there would need to be any explicit time limit on for how long time a cell is considered detected. E.g. if the UE is not moving there would be no immediate reason why a cell detected in connected mode wouldn’t continue being detectable in connected mode and also in Idle mode as long as the cell fulfils the requirements for being detectable.

Do not introduce explicit time limit on for how long time a cell is considered detected.

2.1.2	Direct Scell activation

As discussed in Busan meeting, some UE concerns were raised concerning the direct SCell complexity. Specifically, it was mentioned that it is more challenging for a UE to meet a given SCell activation delay timeline when directly activating more than one SCell. However, from network point view there are some use cases where it can be beneficial to configure more than 1 SCell directly. E.g. for intra-band CA case it is useful to configure more than SCell directly. Configuring more than 1 intra-band SCell directly should as such not have big impact on the UE complexity (e.g. it would not include starting an additional RF chain).

There are use cases where simultaneous direct configuration of more than 1 SCell is beneficial.

Additionally, it was also mentioned that not all UEs will be able to directly activate the same number of SCells within a given SCell activation delay timeline. Having different UE capability for this might not be very beneficial and would only cause additional feature complexity. On the other hand, if the UE would be configured with more than SCell directly within the same SCell activation delay timeline, the UE would be allowed some additional relaxation in a similar manner as is known from existing SCell activation requirements.

If more SCells are directly configured simultaneously, the UE requirements would be relaxed accordingly.

We propose that RAN4 decided a specific number of SCells that can be expected that all UE shall be able to simultaneously directly activate. We would prefer that at least for intra-band case the UE shall support simultaneous direct configuration of more than 1 SCell.

A UE shall be able to support direct activation of at least two intra-band SCells simultaneously.

For inter-band case it would be acceptable with a lower number.

A UE shall be able to support direct activation of at least one inter-band SCell.

RAN4 would of course need to agree on such number, but in principle the most important is that the UE support direct activation of at least one SCell.

In the last meeting the actual activation delay was left undecided with the following two options on the table [9]:

- Proposal 1:

  - Tdirect_activation = TRRC_PROCESS + TUL_GRANT + TRRC_PHY + TPHICH_PROC + 20ms

  - where

    - TRRC_PROCESS is 20ms which is the maximum RRC processing delay defined in TS36.331, and

    - TUL_GRANT is the amount of the time until UE receives the UL grant for RRCConnectionReconfigurationComplete, and

    - TRRC_PHY is the amount of the time from the reception of the UL grant for RRCConnectionReconfigurationComplete until the reception of the ACK for RRCConnectionReconfigurationComplete, and

    - TPHICH_PROC is the processing delay for PHICH, which is [2] ms.

- Proposal 2:

  - Direct Activated SCell activation delay is defined as n+ Tconfig_ Direct_SCell where:

    - Tconfig_Direct_SCell = 20ms + Tactivation_time

    - Tactivation_time is the SCell activation delay. If the SCell is known, then Tactivation_time is 20ms.

As discussed in last meeting we see large synergies between the direct SCell activation and the procedure for PSCell addition. Procedure wise they are both configurations received in an RRC message and applied immediately by the UE. Once the network sends the Direct SCell activation command to the UE, the UE has up to 20ms to process and apply the RRC configuration. The UE shall be ready at n+20ms if the RRC message is received in time slot n. From n+20 the network takes the new PUCCH configuration into use and UE can start transmitting CSI for the directly activated SCell from n+20ms.

For the existing MAC based SCell activation the UE is allowed a certain delay. Different conditions and different UE implementations leads to different SCell activation delays. Similar situation is present when it comes to Direct SCell activation with the exception that there is no need for any MAC command delay. I.e. for direct SCell activation the UE SCell activation delay should be the same as for normal MAC based SCell activation excluding MAC command.

As the UE shall have processed and applied the RRC command (including the Direct SCell activation) no later than at n+20 and the SCell activation delay, Tactivation_time, should be the same as for MAC based activation command, we see that proposal 2 of the two options is the correct Direct SCell activation delay.

Direct Activated SCell activation delay is defined as n+ Tconfig_ Direct_SCell where:

Tconfig_Direct_SCell = 20ms + Tactivation_time

- Tactivation_time is the SCell activation delay. If the SCell is known, then Tactivation_time is 20ms.

RAN4 also need to define requirements for the unknown SCell. Proposal 5 has been used in the baseline CR [8] for introduction of direct SCell activation – accounting also the unknown SCell.

The aspect of interrupts requirements at direct SCell Activation was also left open in last meeting. We propose to apply the same interrupt requirement as is defined for PSCell addition combined with the SCell addition. I.e. the interrupt due to Direct SCell activation shall happen during the RRC processing delay (like when configuring a PSCell) while the interrupt duration is as defined for SCell addition in section 7.8.

Interrupt due to direct SCell activation shall happen during the RRC processing delay.

Interrupt duration from a direct SCell activation shall be as defined for SCell addition in section 7.8.

2.1.3	Dormant SCell

In the last meeting RAN4 agreed that the UE shall report CQI according to the SCell CQI reporting requirements and that the CQI reporting in dormant SCell state following the PCell DRX cycle, shall follow the existing rules as defined for CQI reporting in DRX.

Our understanding is that this means that Dormant SCell CQI reporting follows the PCell DRX. I.e. UE reports Dormant SCell CQI according to the MAC setting (periodic CQI reporting is limited to On-durations or not).

If the CQI reporting for the Dormant SCell is often this could have significant negative impact on the scheduling in case UE causes an interrupt for each CQI report. In order to enable UE power saving we suggest allowing the UE some interrupts aligned with the existing interrupt requirements for measurements on a deactivated SCell.

This would mean that PCell interruptions due to CQI measurements on dormant SCell are allowed with up to 0.5% probability of missed ACK/NACK.

PCell interruptions due to CQI measurements on dormant SCell are allowed with up to 0.5% probability of missed ACK/NACK.

Conclusion

In this paper, we continue the detailed discussion related to the open RRM aspects related to euCA and early measurement reporting, direct SCell activation and dormant SCell UE requirements. Based on the discussion we propose:

- Include the the previous valid measurement report during connected mode.

Do not introduce any time limitations for SIB5 limitation.

Do not introduce explicit time limit on for how long time a cell is considered detected.

If more SCells are directly configured simultaneously, the UE requirements would be relaxed accordingly.

A UE shall be able to support direct activation of at least two intra-band SCells simultaneously.

A UE shall be able to support direct activation of at least one inter-band SCell.

Direct Activated SCell activation delay is defined as n+ Tconfig_ Direct_SCell where:

Tconfig_Direct_SCell = 20ms + Tactivation_time

- Tactivation_time is the SCell activation delay. If the SCell is known, then Tactivation_time is 20ms.

Interrupt due to direct SCell activation shall happen during the RRC processing delay.

Interrupt duration from a direct SCell activation shall be as defined for SCell addition in section 7.8.

PCell interruptions due to CQI measurements on dormant SCell are allowed with up to 0.5% probability of missed ACK/NACK.

References

[1]	R4-1805498, Way forward on euCA RRM requirements, Nokia, Nokia Shanghai Bell

[2]	R2-1809245, Stage-2 description of euCA, Nokia, Nokia Shanghai Bell

[3]	R2-1809246, UE capabilities for euCA, Nokia, Nokia Shanghai Bell

[4]	R2-1809269, MAC functionality for euCA, Nokia, Nokia Shanghai Bell

[5]	R2-1809270, Signalling for euCA, Nokia, Nokia Shanghai Bell

[6]	R4-18xxxx, CR introducing enhanced utilization of CA and idle mode measurements for early reporting, Nokia, Nokia Shanghai Bell

[7]	R4-1807545, WF for RRM requirements for euCA, Nokia, Nokia Shanghai Bell

[8]	R4-18xxxx, baseline CR for euCA, Nokia, Nokia Shanghai Bell (cr for direct)

[9]	R4-1807936, Summary of open issues and proposals for euCA, Nokia, Nokia Shanghai Bell