3GPP TSG-RAN WG4 Meeting #88	R4-1811412

Goteborg, Sweden, 20 – 24 August 2018

Agenda Item:	7.13.1.6

Source:	Ericsson

Title:	Simplified propagation channel models based on TDL-D/E

Document for:	Discussion

# 1	Introduction

RAN4#AH1807 discussed the simplification of TDL channel models for NR UE demodulation and CSI reporting requirements and agreed with the way forward [1].

- Simplification methods - Option 1: Choose strongest paths that contribute to [X%] of total power - Option 1: X=90% for TDL-A; X=87% for TDL-C - Option 2: X = 90% or 95% for TDL-A and TDL-C - Option 2: Choose 9 paths for TDL-A and TDL-C using the frequency correlation method - TDL-A: Tap#1-6, 9, 11, 15 defined in Table 7.7.2-1 of TR 38.901 - TDL-C: Tap#1-2, 4-8, 13-14 defined in Table 7.7.2-3 of TR 38.901 - Number of path: less than or equal to 12 is acceptable from test equipment point of view

RAN4 is discussing the used channel modes for FR2 are Option1: TDL channel modeling or Option 2: CDL channel modeling, but RAN4 AH1807 agreed with the working assumption option 1 is used for FR2.

Working assumption: Channel Model Option 1 will be used for FR2. - Companies will provide the simulation results based on working assumption - Companies are requested to provide more input to confirm the working assumption in August meeting. And final comfirmation of working assumption will be based on majority companies’ view.

Since there is no difference with regard to delay position and relative powers between TDL and CDL in TR38.901 [2], we discuss the simplification of delay position and relative powers for these models for FR2 also.

# 2	Simplified TDL propagation channel models

## 2.1	Simplification procedure

As RAN4#87 discussed, the resolution of delay path grids for CDL/TDL channel models in TR38.901 is 1 pico-second (1ps). To avoid the complexity of the test setting, RAN4 discussed to assume the simplification of the channel model.

The simplification method in this paper is same as what we have done for FR1 [3].

Step 1: Remove the path if the path power is less than threshold. Option 1: Choose strongest paths that contribute to [X=90,95]% of total power. Option 2: Choose [Y=9] strongest paths that contribute for NLOS PDPs. Step 2: Apply the desired delay spread after removing the weak paths. Option 1: Adjust the delay position to keep the delay spread to the desired delay spread (e.g., 30ns). Option 2: Keep the original delay position. This results the effective delay spread may be smaller than the desired delay spread. Step 3: Decide the path grid based on resolution, and round the path delay position close to the grid. Set the grid step size to [5] ns. Step 4: If two or more paths are rounded to the same grid τ, the path power at τ is set as a sum of all the path power values (in linear) rounded to τ. Step 5: Normalize the power so that the relative power of the strongest path becomes 0dB.

## 2.2	Simplified	 TDL-D and TDL-E

The difference between TDL-A/B/C and TDL-D/E is the LOS path is included or not. Considering the radio propagation characteristics, RAN4 should also consider the channel models including LOS path for FR2.

### 2.2.1	TDL-D

We studied the simplified TDL-D path delay profile as shown in Figure 1 and Figure 2. In Figure 1, we did not trim any paths. But some delay paths are close each other, after rounding to the closest path grid. Therefore, some paths are combined and therefore the total number of paths are reduced from the original one.

Figure 2 is the results of removing the paths whose relative power is less than -25dB. Left figure (case 1) is the case the path position is adjusted to keep the desired delay spread to 30ns; right figure (case 2) is the case the original path position is kept. As the results of that, the effective DS for case 1 is 30.2ns, but the DS for case 2 is 24.1ns. The difference of both cases is observed if we compare the frequency correlation characteristics.

Table 1 shows the path positions and their relative powers for simplified TDL-D channel models. Because of the overlap after applying the desired delay spread factor, the number of non-LOS paths is reduced from 13 to 10. If it is preferable to reduce some weakest paths further, we can remove the paths whose relative power is less than -25dB, but it keeps 99.3% of original path power.

Figure 1	Example of simplified TDL-D (Desired DS=30ns); No path trimming.

Figure 2	Example of simplified TDL-D (Desired DS=30ns) with removing the path whose power < -25dB.  Adjust the delay position to keep DS to 30ns (left) and keep the original delay position (right).

Table 1	Tap delay and relative power for the simplified TDL-D with Desired DS=30ns

|  | No trimming |  | Delay position adjusted |  | Keep the original delay position |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | X = 100% |  | X = 99.3% |  | X = 99.3% |  |
|  | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Tap delay [ns] | Tap delay [ns] | Tap delay [ns] |
|  | 0 | -0.2 (LOS) | 0 | -0.2 (LOS) | 0 | -0.2 (LOS) |
| 1 | 0 | -13.5 | 0 | -13.5 | 0 | -13.5 |
| 2 | 20 | -21.1 | 25 | -21.1 | 20 | -21.1 |
| 3 | 40 | -16.7 | 50 | -22.9 | 40 | -16.7 |
| 4 | 55 | -18.3 | 55 | -18 | 55 | -18.3 |
| 5 | 80 | -22 | 65 | -23 | 80 | -22 |
| 6 | 120 | -27.9 | 70 | -20.2 | 240 | -23.7 |
| 7 | 240 | -23.7 | 95 | -22 | 285 | -24.9 |
| 8 | 285 | -24.9 | 300 | -23.7 |  |  |
| 9 | 290 | -30.1 | 355 | -24.9 |  |  |
| 10 | 375 | -27.8 |  |  |  |  |
|  |  | DS=29.2ns |  | DS=30.2ns |  | DS=24.1ns |

### 2.2.1	TDL-E

We have done the same exercise for TDL-E as shown in Figure 3 and Figure 4. As same as TDL-D, we set the desired delay spread to 30ns considering the FR2. It is observed that the the relative powers of non-LOS paths for TDL-E are also very weak.

Table 2 shows the path positions and their relative powers for simplified TDL-E channel models. Because of the overlap after applying the desired delay spread factor, the number of non-LOS paths is reduced from 14 to 10. If it is preferable to reduce some weakest paths further, we can remove the paths whose relative power is less than -25dB, but it keeps 99.5% of total power.

Figure 3	Example of simplified TDL-E (Desired DS=30ns); No path trimming.

Figure 4	Example of simplified TDL-E (Desired DS=30ns) with removing the path whose power < -25dB.  Adjust the delay position to keep DS to 30ns (left) and Keep the original delay position (right).

Table 2	Tap delay and relative power for the simplified TDL-E with Desired DS=30ns

|  | No trimming |  | Delay position adjusted |  | Keep the original delay position |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | X = 100% |  | X = 99.5% |  | X=99.5% |  |
|  | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
|  | 0 | -0.03 (LOS) | 0 | -0.03 (LOS) | 0 | -0.03 (LOS) |
| 1 | 0 | -22.03 | 0 | -22.03 | 0 | -22.03 |
| 2 | 15 | -12.4 | 25 | -12.4 | 15 | -12.4 |
| 3 | 20 | -22.4 | 35 | -22.4 | 20 | -22.4 |
| 4 | 55 | -18.6 | 90 | -18.6 | 55 | -18.6 |
| 5 | 60 | -18.6 | 95 | -18.6 | 60 | -18.6 |
| 6 | 80 | -22.3 | 130 | -22.3 | 80 | -22.3 |
| 7 | 110 | -25.6 | 265 | -20.2 | 165 | -20.2 |
| 8 | 165 | -20.2 |  |  |  |  |
| 9 | 360 | -29.8 |  |  |  |  |
| 10 | 620 | -29.2 |  |  |  |  |
|  |  | DS=30.0ns |  | DS=30.2ns |  | DS=19.2ns |

# 3	Simulation results

We investigated the PDSCH performance impact due to the simplification. Table 3 is the simulation parameters.

Table 3	Simulation parameters for PDSCH

|  | Case 1 | Case 2 |
| --- | --- | --- |
| Propagation channel | Model 1: TDL-D DS=30ns, Doppler=80Hz (~3km/h@30GHz) Model 2: TDL-E DS=30ns, Doppler=300Hz (~10km/h@30GHz) |  |
| Frequency range | FR2 | FR2 |
| UE system bandwidth | 100MHz | 100MHz |
| SCS | 120kHz | 120kHz |
| Antenna configuration and MIMO layers | 2x2 Low, 2 layers | 2x2 Low, 2 layers |
| MCS | MCS4 (QPSK 0.33) | 64QAM MCS 24 (64QAM ???) |
| Max HARQ transmission | 4 | 4 |

Figure 5	PDSCH Simulation results for TDL-D with MCS4.

Figure 6	PDSCH Simulation results for TDL-D with MCS24.

Figure 7	PDSCH Simulation results for TDL-E with MCS4.

Figure 8	PDSCH Simulation results for TDL-E with MCS24.

# 4	Conclusion

# 5	References

R4-1809398, “Way Forward on Channel Model for NR UE Demodulation and CSI Requirements”, Huawei.

3GPP TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

R4-1810159, “Demodulation performance impacts due to simplified propagation channel models”, Ericsson.

# Appendix

Table 7.7.2-4. TDL-D.

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -13.5 | Rayleigh |
| 2 | 0.035 | -18.8 | Rayleigh |
| 3 | 0.612 | -21 | Rayleigh |
| 4 | 1.363 | -22.8 | Rayleigh |
| 5 | 1.405 | -17.9 | Rayleigh |
| 6 | 1.804 | -20.1 | Rayleigh |
| 7 | 2.596 | -21.9 | Rayleigh |
| 8 | 1.775 | -22.9 | Rayleigh |
| 9 | 4.042 | -27.8 | Rayleigh |
| 10 | 7.937 | -23.6 | Rayleigh |
| 11 | 9.424 | -24.8 | Rayleigh |
| 12 | 9.708 | -30.0 | Rayleigh |
| 13 | 12.525 | -27.7 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 13.3 dB and a mean power of 0dB. |  |  |  |

Table 7.7.2-5. TDL-E.

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.03 | LOS path |
|  | 0 | -22.03 | Rayleigh |
| 2 | 0.5133 | -15.8 | Rayleigh |
| 3 | 0.5440 | -18.1 | Rayleigh |
| 4 | 0.5630 | -19.8 | Rayleigh |
| 5 | 0.5440 | -22.9 | Rayleigh |
| 6 | 0.7112 | -22.4 | Rayleigh |
| 7 | 1.9092 | -18.6 | Rayleigh |
| 8 | 1.9293 | -20.8 | Rayleigh |
| 9 | 1.9589 | -22.6 | Rayleigh |
| 10 | 2.6426 | -22.3 | Rayleigh |
| 11 | 3.7136 | -25.6 | Rayleigh |
| 12 | 5.4524 | -20.2 | Rayleigh |
| 13 | 12.0034 | -29.8 | Rayleigh |
| 14 | 20.6519 | -29.2 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 22 dB and a mean power of 0dB. |  |  |  |