3GPP RAN WG4 Meeting #88		R4-1809788
Gothenburg, Sweden, August 20 – 24, 2018

Agenda item:	7.6.2.1

Source:	Apple Inc.

Title:	Applicability of FR2 UL RMCs to maximum output power requirements

Document for:	Approval

# Introduction

One of the remaning issues of the NR Work Item [1] is the definition of the RMCs for the UE RF requirements, as had been discussed during the 3GPP RAN #80 meeting [2].  It was the decision of TSG RAN to extend the core part of the WI, which was documented with an approved exception sheet [3].

This contribution provides a proposal for the applicability of UL RMCs to FR2 maximum output power requirements based on prior agreements.

# Discussion

During RAN4 #85 the reference waveform for the maximum output power requirement was agreed [7]:

- The waveform defined by BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK, 128RB0 is the reference waveform with 0dB MPR and is used for the power class definition - MPR requirements for waveforms that operate at higher output power than the reference waveform can be defined with a negative value - The Pcmax requirement includes an additional term to account for negative MPR in both Pcmax_L and Pcmax_H terms

This agreement can be captured in the applicability of the defined RMCs to the maximum output power requirement.

Proposal 1: Based on the agreements in [7], Table 1 summarizes those RMCs which are applicable to the maximum output power requirements for FR2 UEs.

Table 1: Applicability of UL RMCs to the maximum output power requirements

| Waveform | Modulation | SCS (kHz) | CBW (MHz) | Allocated RBs | Applicable to |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | QPSK | 60 | 50 | 64 | 6.2.1, 6.2A.1, 6.2D.1 |
| DFT-s-OFDM | QPSK | 60 | 100 | 128 |  |
| DFT-s-OFDM | QPSK | 60 | 200 | 256 |  |
| DFT-s-OFDM | QPSK | 120 | 50 | 32 |  |
| DFT-s-OFDM | QPSK | 120 | 100 | 64 |  |
| DFT-s-OFDM | QPSK | 120 | 200 | 128 |  |
| DFT-s-OFDM | QPSK | 120 | 400 | 256 |  |

# Conclusion

This contribution has provided the following proposal for the applicability of UL RMCs to FR2 maximum output power requirements:

Proposal 1: Based on the agreements in [7], Table 1 summarizes those RMCs which are applicable to the maximum output power requirements for FR2 UEs.

# References

- RP-181474, “New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #80, June 2018

- RP-181278, “New Radio (NR) Access Technology Status Report to TSG,” 3GPP RAN #80, June 2018

- RP-181473, “Rel-15 Work Item Exception for New Radio Access Technology – Core,” NTT DOCOMO, 3GPP RAN #80, June 2018

- TS38.101-2 v15.2.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, June 2018

- R4-1809345, “LS reply on RAN4-RAN5 5G-NR RF pending issues: RMCs, OCNG patterns and FR2 RSRP accuracy,” 3GPP RAN4 AH #1807, July 2018

- R4-1809573, “Draft CR on UL RMC for FR2 RF tests,” Qualcomm, 3GPP RAN4 AH #1807, July 2018

- R4-1714445, “WF on pi/2 BPSK spectrum shaping and power class for FR2,” Intel Corporation, Apple, IITH, Qualcomm, LGE, 3GPP RAN4 #85, November 2017