3GPP TSG-RAN WG4 88	R4-1809814

Gothengurg, Sweden 20st – 24th August, 2018

# Source:	Intel Corporation

# Title:	Correction on single UL capability for DC_B2_n66

# Agenda Item:	7.6.3

# Document for:	Approval

# 1	Introduction

In RAN4#84, the general principals on single UL transmission capability were agreed [1]. Some of key agreements are copied below:

- Inter-band combinations that have IM2 are categorized difficult and inter-band combinations that have IM3 for L-L and H-H combinations are categorized difficult. - Only IM3 with L-L (below 1GHz) or H-H (1.695-2.695GHz) combinations is difficult

In this paper, we analyze the single UL capability for DC_B2_n66 based on the principals.

# 2	Discussion

Table 5.2B.4.1-1 in 38.101-3 [2] shows single UL capability for all intra-band cases with two bands. From the table, DC_B2_n66 is not allowed for the capability. Table 1 below is excerpted from the table 5.2B.4.1-1.

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n28 | 1 | n28 | No |
| DC_1_n40 | 1 | n40 | No |
| DC_1_n51 | 1 | n51 | No |
| DC_1_n773 | 1 | n77 | DC_1_n77 |
| DC_1_n783 | 1 | n78 | No |
| DC_1_n793 | 1 | n79 | No |
| DC_2_n5 | 2 | n5 | No |
| DC_2_n66 | 2 | n66 | No |

Table 1. Band combination for EN-DC (two bands; excerpted from Table 5.2B.4.1-1 [2])

However, both band 2 (DL: 1930 – 1990 MHz; UL: 1850 – 1910 MHz) and n66 (DL: 2110 – 2200 MHz; UL: 1710 – 1780 MHz) can be categorized as high band (1.695 – 2.695 GHz) from the band group definition in the agreement.

Based on an IMD calculation, the band combination, DC_B2_n66, generates IM3 which falls into the band 2 DL range. Table 2 shows the IMD analysis.

In 36.101, the same band combination for CA (CA_2A-66A) allows 20 dB MSD for an IMD3 into DL B2 and 4 dB MSD for an IMD5 into DL B66. Table 3 shows the MSD table excerpted from 36.101.

From the agreement and the IMD analysis, a single UL transmission is supposed to be allowed for DC_B2_n66.

Proposal: A single UL transmission is being supported for DC_B2_n66.

Table 2. Intermodulation distortion analysis for DC_B2_n66

Table 3. MSD for LTE CA_2A-66A (excerpted from Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations in 36.101)

# 3	Conclusions

In this paper, IMD analysis was done for DC_B2_n66 and the following proposal was made:

Proposal: A single UL transmission is being supported for DC_B2_n66

# 4	Reference

[1] R4-1711618, “WF on single Tx switched UL”, Apple, RAN4#84Bis, October 2017

[2] TS 38.101-3 V15.2.0 (2018-06)