3GPP TSG-RAN4 Meeting #88 	R4-1809866

# Gothenburg, Sweden, 20 – 24 Aug, 2018

Title:	Draft LS for UE capability of measurement gap for inter-RAT NR measurement not yet configured with EN-DC

Response to:	R2-1809203

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

CC:

Contact Person:

#### Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N/A

1. Overall Description:

RAN4 would like to thank RAN2 for the LS on UE capability of measurement gap for inter-RAT NR measurement not yet configured with EN-DC in which RAN2 would like to ask RAN4 whether the MR-DC band combinations reported in UE capability can allow the network to identify the cases where the LTE UE not yet configured with EN-DC can perform gapless measurements.

RAN4 would like to inform RAN2 the following conclusion in RAN4.

In short, MR-DC band combinations reported in UE capability cannot allow the network to identify the cases where the LTE UE not yet configured with EN-DC can perform gapless measurements. Furthermore, followings are understandings from RAN4,

- for UEs who support per-UE gap, if RAN2 won’t have NeedForGap indication in Rel-15, it shall be assumed that the inter-RAT NR measurement always needs MG

- for UEs who support per-FR gap, if RAN2 won’t have NeedForGap indication in Rel-15 and configured NR MOs are associated with FR1 or FR1+FR2, it shall be assumed that the inter-RAT NR measurement always needs MG.

- for UEs who support per-FR gap, if RAN2 won’t have NeedForGap indication in Rel-15 and configured NR MOs are only associated with FR2, inter-RAT NR measurement for FR2 can be conducted without MG.

2. Actions:

To RAN WG2:

RAN4 respectfully asks RAN2 to take the above information into account in their future work.

3. Date of Next RAN4 Meetings:

3GPP RAN4 #88bis 		8 - 12 Oct 2018, 		Chengdu, China

3GPP RAN4 #89 		12 - 16 Nov 2018, 		Spokane, US