3GPP TSG-RAN WG4 Meeting #88 	R4-1809874

Gothenburg, Sweden, August 20 – 24, 2018

Agenda item:	7.11.4.3

Source:	Intel Corporation

Title:	On SFTD measurement for the new scenarios

Document for:	Discussion

# Introduction

At the RAN4#85 meeting, RAN2 sent the LS regarding SFTD measurement [1]. RAN2 asks following questions:

In addition, RAN2 would like to understand the feasibility in the following cases - In case an NR PSCell is configured, determining SFTD for NR neighbour cells at a monitored carrier frequency (with an NR PSCell, or an NR SCG serving cell) - In case an NR PSCell is configured, determining SFTD for NR neighbour cells at a non-monitored carrier frequency - SFTD measurements for NE-DC and NR-NR DC

In RAN4#87 an LS [2] was sent from RAN4 to RAN2 regarding clarifications and justifications concerning the new SFTD scenarios, and up to now RAN4 is still waiting for the reply. In this contribution we are providing a brief analysis of these cases and come up with proposals on SFTD measurement for new scenarios.

# Discussion

We further specify the scenarios for SFTD measurement as the following,

- Case 1: SFTD measurements for neighbour NR cell(s) on monitored carrier with NR PSCell or SCG serving cell

- Case 2: SFTD measurements for neighbour NR cell(s) on non-monitored carrier with NR PSCell configured

  - Intra-band case where LTE PCell,  NR PSCell and target cell are intra-band

  - Inter-band case where both LTE PCell and NR PSCell are inter-band to target cell

- Case 3: SFTD measurements for NE-DC

In the following sections, we provide our view on the above four cases one by one. Since RAN4 is waiting for RAN2 reply on clarification for NE-DC and NR-NR DC scenarios, this contribution focuses on the case where PSCell is configured in Case 3.

## SFTD measurements for neighbour NR cell(s) on monitored carrier with NR PSCell or SCG serving cell

For Case 1, where the NR PSCell is configured, SFTD measurement on intra-frequency NR neighbour cell might be needed if there is timing drifting between the NR PSCell and neighbour NR. The key question is that if the NR neighbour cell SSB can fall within the SMTC window configured for the NR SCG serving cell.  If the SMTC configured for neighbour cell can fully overlap with one SMTC window configured for NR SCell, UE can use the SMTC window for the measurement object for NR SCell to perform the SFTD measurement; otherwise, UE would not be able to do SFTD measurement.

Proposal 1: The SFTD measurement for NR neighbor cells at a monitored carrier frequency with NR PSCell or SCG serving cell in EN-DC is feasible

- if the SMTC window configured on this monitored frequency can cover the SSB of the NR neighbor cell.

In this case, UE can perform SFTD measurement based on SMTC window configured for the frequency carrier.

## SFTD measurements for neighbour NR cell(s) on non-monitored carrier with NR PSCell configured

For Case 2, the SFTD measurement is performed for the neighbour NR cell which is in inter-frequency to the configured NR PSCell. If there is timing drifting between serving carrier and the non-monitored carrier, network would not know the appropriate MG/SMTC timing for inter-frequency NR neighbour cell. Therefore, interruption –based SFTD measurement is needed. During the SFTD measurement, UE first obtain the timing and MIB information for the NR PSCell and then tune its RF to the non-monitored carrier frequency to obtain the timing and MIB information for the NR neighbour cell.

Case 2 can be further divided into the following two scenarios:

- Intra-band case where LTE PCell,  NR PSCell and target NR cell are intra-band

- Inter-band case where both LTE PCell and NR PSCell are inter-band to the target NR cell

For the intra-band case, the requirements for intra-band SFTD measurement on the non-configured NR cells without measurement gaps in EN-DC case, which is specified in TS36.133 section 7.35.2, can be reused. For the inter-band case, the requirements specified in TS 36.133 section 8.1.2.4.25 for inter-RAT SFTD measurement in EN-DC can be reused. Thus, we propose:

Proposal 2: For the case of SFTD measurement for neighbor cell on non-monitored carrier, the requirements for inter-RAT SFTD measurement with interruption specified in TS 36.133 can be reused. Specifically,

- The UE is allowed an interruption of up to [10] subframes on LTE PCell and activated NR SCells if configured during Tmeasure_SFTD1 as specified in TS36.133 section 8.1.2.4.25 if the LTE PCell,  NR PSCell and target NE neighbor cell are intra-band.

- The UE is allowed an interruption on PCell and activated SCells if configured during Tmeasure_SFTD1 as specified in TS36.133 section 8.1.2.4.25 if LTE PCell or NR PSCell is inter-band to the target NE neighbor cell.

## SFTD measurements for NE-DC

For Case 3, the SFTD measurement is performed in NE-DC. If SFTD measurement is performed before LTE PSCell is configured UE can use LTE measurement gap to complete the SFTD measurement since LTE SS is sent every 5ms. If SFTD measurement is performed after LTE PSCell is configured, then the requirements for SFTD measurement of serving cells in NE-DC can be reused.

Proposal 3: For SFTD measurement when LTE PSCell is configured in NE-DC, the SFTD measurement requirements for the serving cells in EN-DC in TS 36.133 section 8.17.2.2 can be reused.

# Summary and Conclusions

This contribution provides the analysis on the SFTD measurement for the new scenarios. The conclusions are drawn as follows:

Proposal 1: The SFTD measurement for NR neighbor cells at a monitored carrier frequency with NR PSCell or SCG serving cell in EN-DC is feasible

- if the SMTC window configured on this monitored frequency can cover the SSB of the NR neighbor cell.

In this case, UE can perform SFTD measurement based on SMTC window configured for the frequency carrier.

Proposal 2: For the case of SFTD measurement for neighbor cell on non-monitored carrier, the requirements for inter-RAT SFTD measurement with interruption specified in TS 36.133 can be reused. Specifically,

- The UE is allowed an interruption of up to [10] subframes on LTE PCell and activated NR SCells if configured during Tmeasure_SFTD1 as specified in TS36.133 section 8.1.2.4.25 if the LTE PCell,  NR PSCell and target NE neighbor cell are intra-band.

- The UE is allowed an interruption on PCell and activated SCells if configured during Tmeasure_SFTD1 as specified in TS36.133 section 8.1.2.4.25 if LTE PCell or NR PSCell is inter-band to the target NE neighbor cell.

Proposal 3: For SFTD measurement when LTE PSCell is configured in NE-DC, the SFTD measurement requirements for the serving cells in EN-DC in TS 36.133 section 8.17.2.2 can be reused.

# References

- R2-1804119, “LS On SFTD measurements”, Ericsson, RAN2 #101 meeting

- R4-1808442, “Reply LS on SFTD measurements”, ZTE, RAN4 #87 meeting