3GPP TSG-RAN WG4 Meeting #88               	R4-1809953

Gothenburg, Sweden, 20 Aug – 24 Aug 2018

Agenda item:	7.13.2.2

Source:	Samsung

Title:	Initial simulation results for NR PUSCH

Document for:	Discussion

# 1	Introduction

During RAN4 AH07 meeting, the NR BS demodulation performance requirements are further discussed on the general issue and performance requirements test. Some general issue about NR PUCCH performance requirements are captured and agreed in the WF [1].In order to facilitate simulation alignment of NR BS PUSCH performance between each company, some test cases are introduced. In this contribution, the initial simulation results of NR PUSCH are provided.

# 2	Simulation Results

In this section, based on our preferred, the initial simulation results of PUSCH are provided as table 5. As agreed in WF [2] last meeting, some tests are introduced to alignment purpose. Here, we just list the main simulation assumption as follows table 1.

Table 1: Simulation assumption for NR PUSCH

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Transmission scheme | n.a. | n.a. |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1 |
| symbols length | 14 | X |
| Time domain resource allocation type | type A | type B |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | TDL-A 30ns 10Hz | AWGN |
| SCS and BW | 15kHz: 10 30kHz: 40 | 60kHz: 100 120kHz: 100 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |
| Total cases | 12 | 6 |

As for the fading channel, current agreement is that TDL channel will be applied for FR1. In FR1, two kinds of simplified TDL channel are proposed, one is based on the strongest paths that contribution to [X%] of total power, another is to select the fixed number of paths. As for FR2, whether TDL or CDL should be used is still under discussion. In this contribution, we apply the TDA-C without reduction.

For FR2, since currently there is no conclusion for the channel model and Phase noise model, in this meeting, we just only provide the simulation results for FR1.

Based on the simulation assumption, the initial simulation can be indicated as the following table.

Table2: The target SNR performance for NR PUSCH for FR1

| Case | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) | Target SNR for MCS16 (dB) | Target SNR for MCS20 (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1 | 10 | -3.866 | 8.042 | 11.173 |
| 2 | 15 | 1+1 | 10 | -4.463 | 7.943 | 11.181 |
| 3 | 30 | 1 | 40 | -4.124 | 7.771 | 11.907 |
| 4 | 30 | 1+1 | 40 | -4.687 | 7.858 | 11.046 |

Based on the results, we can observe that with additional DMRS symbol, channel estimation accuracy can be improved.

# 3	Conclusion

In this contrition, based on the agreement of WF on the NR BS PUSCH demodulation, the initial simulation result of NR PUSCH demodulation requirement was provided.

# References

R4-1809366, WF on NR PUSCH demodulation requirements , Nokia

# Appendix

Table3 FRC table for FR1 with 1 DMRS

| CBW(MHz) | 10MHz | 10MHz | 10MHz | 40MHz | 40MHz | 40MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 30 | 30 | 30 |
| RB | 52 | 52 | 52 | 106 | 106 | 106 |
| Modulation order | 2 | 4 | 6 | 2 | 4 | 6 |
| MCS index | 2 | 16 | 20 | 2 | 16 | 20 |
| Code Rate | 1/5 | 2/3 | 1/5 | 1/5 | 2/3 | 1/5 |
| Num of DMRS | 1 | 1 | 1 | 1 | 1 | 1 |
| Channel bits | 16224 | 32448 | 48672 | 33072 | 66144 | 99216 |
| Final TBS (A) | 3104 | 21000 | 27144 | 6280 | 42016 | 55304 |
| CRC for TB (A>3824, 24; otherwise 16) | 16 | 24 | 24 | 24 | 24 | 24 |
| C (Num of CB) | 1 | 3 | 4 | 1 | 5 | 7 |
| Base Graph Type | BG2 | BG1 | BG1 | BG1 | BG1 | BG1 |

Table 4, FRC table for FR1 with 1 front-loaded DMRS and 1 additional DMRS

| CBW(MHz) | 10MHz | 10MHz | 10MHz | 40MHz | 40MHz | 40MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 30 | 30 | 30 |
| RB | 52 | 52 | 52 | 106 | 106 | 106 |
| Modulation order | 2 | 4 | 6 | 2 | 4 | 6 |
| MCS index | 2 | 16 | 20 | 2 | 16 | 20 |
| Code Rate | 1/5 | 2/3 | 1/5 | 1/5 | 2/3 | 1/5 |
| Num of DMRS | 1+1 | 1+1 | 1+1 | 1+1 | 1+1 | 1+1 |
| Channel bits | 14976 | 19952 | 44928 | 30528 | 61056 | 91584 |
| Final TBS (A) | 2856 | 19464 | 25104 | 5768 | 38936 | 50184 |
| CRC for TB (A>3824, 24; otherwise 16) | 16 | 24 | 24 | 24 | 24 | 24 |
| C (Num of CB) | 1 | 3 | 3 | 1 | 5 | 6 |
| Base Graph Type | BG2 | BG1 | BG1 | BG1 | BG1 | BG1 |

Table 5, FRC table for FR2 with 1 front-loaded DMRS

| CBW(MHz) | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 60 | 60 | 60 | 120 | 120 | 120 |
| RB | 132 | 132 | 132 | 66 | 66 | 66 |
| Modulation order | 2 | 4 | 6 | 2 | 4 | 6 |
| MCS index | 2 | 16 | 20 | 2 | 16 | 20 |
| Code Rate | 1/5 | 2/3 | 1/5 | 1/5 | 2/3 | 1/5 |
| Num of DMRS | 1 | 1 | 1 | 1 | 1 | 1 |
| Channel bits | 41184 | 82368 | 123552 | 20592 | 41184 | 61776 |
| Final TBS (A) | 7680 | 53288 | 67584 | 3848 | 26632 | 33816 |
| CRC for TB (A>3824, 24; otherwise 16) | 24 | 24 | 24 | 16 | 24 | 24 |
| C (Num of CB) | 1 | 7 | 9 | 1 | 4 | 5 |
| Base Graph Type | BG1 | BG1 | BG1 | BG1 | BG1 | BG1 |

Figure1   Throughput performance of MCS 2 with 1 front and 1additional DMRS in SCS=15KHz, BW=10MHz

Figure2   Throughput performance of MCS 16 with 1 front and 1additional DMRS SCS=15KHz, BW=10MHz

Figure3 Throughput performance of MCS 20 with 1 front and 1additional DMRS, SCS=15KHz,BW=10MHz

Figure4   Throughput performance of MCS 2 with 1 front and 1additional DMRS, SCS=30KHz, BW=40MHz

Figure5   Throughput performance of MCS 16 with 1 front and 1additional DMRS, SCS=30KHz,BW=40MHz

Figure6   Throughput performance of MCS 20 with 1 front and 1additional DMRS, SCS=30KHz,BW=40MHz