3GPP TSG-RAN WG4 Meeting RAN4#88                                               R4-180xxxxx

Gotherburg ,Sweden, 20 -24 Aug, 2018

Agenda Item:   7.13.1.5

Source:        Samsung

Title:          Test case design for static CQI test cases

Document for:  Discussion

Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (FFS for wideband or sub-band) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - Sub-band CQI test cases (2nd priority) - Test 1: FR1 FDD 10MHz&15kHz, 2*2,2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2,2*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, initial simulation results for static CQI test cases provided.

Discussion

Simulation assumption

| Parameter | Unit | Static CQI test (FR1 FDD) | Static CQI test (FR1 TDD) | Static CQI test (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | FFS | FFS |
| Propagation channel |  | AWGN | AWGN | AWGN |
| Correlation and antenna configuration |  | 2Rx 2*2 with 4Rx: 2*4 with | 2Rx 2*2 with 4Rx: 2*4 with | 2Rx 2*2 with FFS |
| CSI-RS resource setting (periodic) |  |  |  |  |
| ResourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | [5/0] | [5/0] | [5/0] |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (periodic) |  |  |  |  |
| reportConfigType |  | periodic | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| Codebook Restriction |  | 010000 (fixed rank2) | 010000 (fixed rank2) | 010000 (fixed rank2) |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Simulation results

To be updated

Conclusion

In this contribution, we provided initial simulation results for static CQI test with below observations:

To be updated

Reference

[1] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung