3GPP TSG-RAN4 Meeting #88 	R4-1810014

# Gothenburg, Sweden, Aug. 20th – 24th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SCell activation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-8-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The specific requirements for NR SCell activation delay is not align with the requirement for SCell activation interruption for intra-band case- The cell specific reference signals from the PSCell, activated SCell and the SCells being added or released are available in the same slot - The SCell activation AGC retuning SMTC periodicity should differ intra-band and inter-band - The requirement for allowed PSCell interruption occasion should follow the interruption time in the requirement of SCell activation interruption. It should differ the requirement with intra-band and inter-band. - The condition for unknown cell search in FR2 is missing. - Delay requirement in FR2 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SMTC periodicity definition in the equation; - Add the condition for intra-band SCell activation requirement; - Differ PSCell interruption occasion based on intra-band and inter-band; - Add the condition for unknown cell search in FR2; - Update delay requirement in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The SCell activation requirement is misalignment with the SCell activation Interruption. - The requirement of allowed time location for PSCell interruption will be wrong. - Delay requirement in FR2 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

8.3	SCell Activation and Deactivation Delay

8.3.1	Introduction

This section defines requirements for the delay within which the UE shall be able to activate a deactivated SCell in SCG and deactivate an activated SCell in SCG in E-UTRA-NR DC.

The requirements shall apply for E-UTRA-NR DC where the E-UTRA is the master (Option 3/3a/3x in TR 38.801[1] section 10.1.2)

8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell in SCG.

The delay within which the UE shall be able to activate the deactivated SCell in SCG depends upon the specified conditions.

Upon receiving SCG SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+ [THARQ + Tactivation_time + TCSI_Reporting ] provided the SCell can be successfully detected on the first attempt.

Where:

THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

Tactivation_time is the SCell activation delay. If the SCell is known and belongs to FR1, Tactivation_time is:

-	[3ms+  TSSB +2ms], if the SCell measurement cycle is equal to or smaller than [160ms].

-	[3ms+ TSSB,max + TSSB +2ms], if the SCell measurement cycle is larger than [160ms].

If the SCell is unknown and belongs to FR1, Tactivation_time is:

-	[3ms+ 2*TSSB,max + 2*TSSB +2ms].

If the SCell being activated belongs to FR2, and there is at least one active serving cell on that FR2 band, then Tactivation_time is:

-	[3ms+ 2*TSSB,max + 2*TSSB +2ms].

If the SCell being activated belongs to FR2, and there is no active serving cell on that FR2 band, then, Tactivation_time is:

-	[3ms+ N1* 2* TSSB,max + (N1+1)* TSSB + 2ms].

Where,

TSSB,max equals TSSB if the SCell being activated is not in the same band with any active serving cell, or it equals the max among TSSB and SMTC periodicities of all active serving cell in the same band with the SCell being activated, provided the cell specific reference signals from any active serving cell and from the SCell being activated are available in the same slot;

TSSB is the SMTC periodicity of SCell being activated.

Editor’s note1: N1 is the scaling factor due to Rx beam sweeping, and the value is [TBD].

Editor’s note2: Whether it is necessary to differentiate known/unknown for the case of “SCell being activated belongs to FR2, and there is at least one active serving cell on that FR2 band” is FFS.

TCSI_reporting is the delay uncertainty in acquiring the first available CSI reporting resources as specified in [2].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max([5] measCycleSCell,  [5] DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in section 9.2 and 9.3.

-	the SSB measured during the period equal to max([5] measCycleSCell, [5] DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in section 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [2] for an SCG SCell at the first opportunities for the corresponding actions once the SCell is activated.

The interruption on PSCell or any activated SCell in SCG specified in section 8.2 shall not occur before slot n+[THARQ] and not occur after slot n+[THARQ + X2 slot] when PSCell and any activated SCell in SCG is not in the same band as the SCell being activated, where X2 is specified in Table 8.2.1.2.4-1.

The interruption on PSCell or any activated SCell in SCG specified in section 8.2 shall not occur before slot n+[THARQ] and not occur after slot n+[THARQ + TSSB,max + Y2 slot + TSMTC_duration] when PSCell or any activated SCell is in the same band as the SCell being activated, where Y2 is specified in Table 8.2.1.2.4-1, and TSMTC duration is the duration of the SMTC of the SCell being activated.

Starting from the slot specified in section 4.3 of [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

8.3.3	SCell Deactivation Delay Requirement for Activated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell in SCG

Upon receiving SCG SCell deactivation command or upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n+[ THARQ +3ms ].

The interruption on PSCell or any activated SCell in SCG specified in section 8.2 shall not occur before slot n+[THARQ] and not occur after slot n+[THARQ +X2 slot], where X2 is specified in Table 8.2.1.2.4-1.

<< End of Change >>