3GPP TSG-RAN WG4 Meeting #88	R4-1810141

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	6.6.4.4

Source:	Ericsson

Title:	Simulation results of CQI reporting test for 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#87 agreed with the evaluation parameters for PDSCH demodulation with 1024QAM as follow [1].

- Definition test:

  - Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols): CRS based TM single codeword with TM1 under static channel in Annex B.1

    - Bandwidth:

      - FDD: 10MHz

      - TDD: 20MHz

In this contribution we provide our initial simulation results.

# 2	Discussion

## 2.1	CQI to MCS mapping

We first discuss the CFI configuration for CQI reporting test. Although PDSCH demodulation requirements assume CFI=1 to avoid higher SNR, CQI definition test should use CFI=3 because TS36.214 2.1.3 specifies:

In the CQI reference resource, the UE shall assume the following for the purpose of deriving the CQI index: - The first 3 OFDM symbols are occupied by control signaling

Figure 1 illustrates the CQI to MCS index mapping with CFI=1, 2, and 3, based on TM1 (1 CRS port). It is observed from this figure that the different CQI index is map to different MCS index for CFI=3, except for CQI index 12 and 13. With CFI=1, on the other hand, MCS index is not well distributed especially in higher CQI indexes. For example, CQI indexes 11,12, and 13 are all mapped to MCS index 33 and CQI indexes 14 and 15 are mapped to MCS index 37. Since we are discussing the CQI definition test for 1024QAM supported CQI table, RAN4 should set CFI=3 the CQI definition test.

Proposal 1: Set CFI=3 for CQI definition test for 1024QAM.

Figure 1	CQI to MCS mapping with different CFI configuration.

## 2.2	Simulation results

Figure 2 shows the median of reported CQI with the CQI table supporting 1024QAM (See Table 3 in Appendix). If we set the test points corresponding to CQI indexes supporting 1024QAM (CQI indexes = 14 and 15), the test point should be set to 27 or lager. Considering the SNR test point we are discussing the PDSCH demodulation requirements, we propose to set SNR to 28 and 29dB. For reference, we should our simulation results if CQI definition test with CQI table supporting 1024QAM in Table 1.

Observation: Considering the CQI index supporting 1024QAM, the test point is 27dB or higher.

Figure 2	Median of reported CQI.

Table 1	Simulation results of CQI definition test with CQI table supporting 1024QAM (CFI=3).

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI -1 | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| 25 | 13 | 100 | 0.02 | 0 | 1 |
| 26 | 13 | 100 | 0.01 | 0 | 1 |
| 27 | 14 | 100 | 0.41 | 0 | 1 |
| 28 | 14 | 100 | 0.12 | 0 | 1 |
| 29 | 14 | 100 | 0.04 | 0 | 1 |
| 30 | 14 | 100 | 0.02 | 0 | 1 |
| 31 | 14 | 100 | 0.01 | 0 | 1 |
| 32 | 15 | 100 | 0.86 | 0.01 | N/A |

# 3	Conclusion

Proposal 1: Set CFI=3 for CQI definition test for 1024QAM.

Observation: Considering the CQI index supporting 1024QAM, the test point is 27dB or higher.

# References

- R4-1807966, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon.

# Appendix

Table 2	CQI to MCS table.

|  |  | CFI | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 | 50 | 50 |
|  |  | Available REs | 7500 | 6900 | 6300 |
| CQI index | Code rate | Modulation |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 |
| 2 | 0.19 | 2 | 2 | 2 | 2 |
| 3 | 0.44 | 2 | 8 | 6 | 6 |
| 4 | 0.37 | 4 | 11 | 11 | 11 |
| 5 | 0.6 | 4 | 15 | 15 | 15 |
| 6 | 0.55 | 6 | 21 | 20 | 18 |
| 7 | 0.65 | 6 | 23 | 22 | 21 |
| 8 | 0.75 | 6 | 24 | 24 | 23 |
| 9 | 0.85 | 6 | 24 | 24 | 24 |
| 10 | 0.69 | 8 | 31 | 30 | 28 |
| 11 | 0.78 | 8 | 33 | 32 | 30 |
| 12 | 0.86 | 8 | 33 | 33 | 32 |
| 13 | 0.93 | 8 | 33 | 33 | 32 (*) |
| 14 | 0.83 | 10 | 37 | 36 | 34 |
| 15 | 0.93 | 10 | 37 | 37 | 36 |

(*) TBS=33 is the closest to the target code rate but it exceeds 0.925.

Table 3	CQI table supporting 1024QAM.

Table 7.2.3-4: 4-bit CQI Table 4

| CQI index | modulation | code rate x 1024 x | Efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |