3GPP TSG RAN WG4 Meeting #88	R4-1810161

Gothenburg, Sweden, August 20 – 24, 2018

Agenda Item:	7.9.2.3

Source:	Huawei, HiSilicon

Title:	Test model designs for NR

Document for:	Approval

# Introduction

In RAN#AH-1807, WF [2] listed several areas of study.

- Agreement: Consider PDCCH, PDSCH only as channels for TM design.  PBCH (the impact/need for SS for EVM requirement is to be analyzed)

- PDCCH and PDSCH lay out specifics should be considered (RSs to be considered and their densities), with the objective of the impact to the overall waveform on RF requirements

- Companies are encouraged to provide parameterized designs

This document examines test model designs for NR and provides proposals for the models.

# Discussion

One aspect of the test model design is to introduce parameters to simplify specification and improve maintainability. To illustrate the scope of the design model, it was noted that the description must span 33 bandwidth / numerology combinations (for FR1), approximately 8 test models, and FDD/TDD combinations. Furthermore, the number of slots / subframe also varies with the numerology.

A summary of test models and measured signal features is presented in Table 1.

Table 1. LTE Tested signal features

|  | E-TM1.1 | E-TM1.2 | E-TM2 | E-TM3.1 | E-TM3.2 | E-TM3.3 |
| --- | --- | --- | --- | --- | --- | --- |
| BS output power |  |  |  |  |  |  |
| Occupied bandwidth |  |  |  |  |  |  |
| ACLR |  |  |  |  |  |  |
| Operating band unwanted emissions |  |  |  |  |  |  |
| Transmitter spurious emissions |  |  |  |  |  |  |
| Transmitter intermodulation |  |  |  |  |  |  |
| RS absolute accuracy |  |  |  |  |  |  |
| Total power dynamic range |  |  |  |  |  |  |
| EVM |  |  |  |  |  |  |
| Frequency error |  |  |  |  |  |  |

## Bandwidths

For FR1, the set of allowable bandwidths in MHz can be expressed as

- BW=5nnstart≤n≤510(n-3)6≤n≤nstop

- where nstart = 1 for µ=0 and µ=1; nstart = 2 for µ=2

- where nstop = 8 for µ=0; nstop= 13 for µ=1 and µ=2

A similar formula can be used for FR2

- BW=50n1≤n≤2200(n-2)3≤n≤nstop

- where nstop = 3 for µ=2; nstop= 4 for µ=3

Note it may be possible to obtain an empirical formula relating the number of resource blocks as a function of bandwidth. For several numerologies, a curve-fit expression was obtained (e.g. NRB=round(5.4508671BW-2.7398844) for 15 kHz SCS). However, there were some differences between the computed and listed values. In general, a closed form expression is difficult to obtain. A table, such as described in Appendix C, should be used.

## PDCCH

The PDCCH can occupy multiples of 6 RB blocks in frequency. Based on discussions, there were two possible approaches for the PDCCH:

- PDCCH occupies 1 (2) first symbols of a slot, spanning the bandwidth.

- PDCCH occupies 1 (2) blocks in frequency for 1 (2) symbols. The remainder of 2 symbols is filled with PDSCH.

While each approach has its own benefits, the occupying the span of the bandwidth is preferable considering the ensemble of tests, as Table 2

Table 2. Comparison of PDCCH

|  | Advantages | Disadvantage |
| --- | --- | --- |
| Span bandwidth | - DMRS present across (most) bandwidth - Can be used for power reference - Evaluations are typically symbol based | - Larger percentage of overhead (PDCCH) - May need power boosting when there are unused RBs (similar to LTE) |
| Occupy 1 (2) blocks | - Can minimize overhead due to PDCCH - More samples for PDSCH testing for certain models. | - For TDD tests, some “special” slots are 2 symbols wide. - For 1 PRB test (E-TM2), what should be done with PDSCH in 2 first symbols? There will be different power levels depending on whether the locations of PDCCH and PDSCH overlap |

When consider the ensemble of tests (especially TM2), it is preferred to decouple the PDCCH and PDSCH.

Proposal 1: PDCCH should occupy the entire first symbol(s) of a slot

The formula for the dimension and location of the PDCCH RBs for a bandwidth NRB is:

Pi=i;i=0,⋯, 6NRB6-1

Since there can be unoccupied RBs, the unused power should be redistributed over the used RBs. Specifically, if the total power is PT (linear), then the power per RB is PT/NRB. With the number of unused RBs given by NRB- 6NRB6, the power per RB is thus

PRB=PTNRB1+NRB- 6NRB66NRB6=PT6NRB6

To simplify instantiation of the PDCCH, the ControlResourceSet information element [6] should be set so that:

- cce-REG-MappingType is nonInterleaved

- precoderGranularity is allContiguousRBs

- pdcch-DMRS-ScramblingID is µ+1 where µ is the numerology

## SS block

One FFS item was the need for the SS block. Examining Annex F.3.2 of 36.141 [3], there is a statement regarding EVM

The BS shall transmit a signal according to the Test models, intended for EVM. The primary synchronisation signal and the reference signal shall be used to find the centre of the FFT window.

Test models TM2, TM3.1, TM3.2, and TM3.3 are used for EVM testing. The following considerations for the SS block are listed below.

### Time location

38.213 [5] lists which symbols can bear the SS block in a half-frame. For 15 kHz SCS: symbols 2 and 8 for slots 0 and 1 can carry the SS block. The same numbered symbols for slots 2 and 3 carry the SS block for f>3 GHz. For 30 kHz SCS Case B, symbols 4 and 8 of slot 0, 2 and 6 of slot 1 can carry the SS block. The same numbered symbols of slots 2 and 3 can carry SS block for f>3 GHz. For 30 kHz SCS Case C, symbols 2 and 8 of slots 0 and 1 can carry the SS block. The same numbered symbols of slots 2 and 3 can carry SS block for f>3 GHz. For FR2 120 kHz SCS, the first symbols have indices {4, 8, 16, 20} + 28*n. For FR2 240 kHz SCS, the first symbols have indices {8, 12, 16, 20, 32, 36, 40, 44} + 56*n.

Assuming the TDD pattern in [8], Figure 1 and Figure 2 show which downlink and special slots, respectively, can carry the SS block for FR1. Note for frequencies above 3 GHz, slots 2 and 3 are UL for 15 kHz SCS. Table 3 indicates the maximum possible number of SS blocks for FR1.

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

Slot 0

Slot 0

Slot 1

Case B

Case C

Figure 1. First slot(s) with SS block, 15 and 30 kHz SCS (cases B and C). Each color indicates possible SS block locations.

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

0

1

6

7

12

13

2

3

4

5

8

9

10

11

Slot 1

Slot 2

Slot 3

Case B

Figure 2. Special subframe (slot). Gray shading denotes non-DL symbols while other colors indicate SS blocks

Table 3. Maximum number of SS blocks for first 5 ms FDD and TDD.

| SCS | ≤ 3 GHz, TDD | > 3 GHz, TDD | ≤ 3 GHz, FDD | > 3 GHz, FDD |
| --- | --- | --- | --- | --- |
| 15 kHz | 3 | 3 | 4 | 8 |
| 30 kHz, Case B | 4 | 7 | 8 | 8 |
| 30 kHz, Case C | 4 | 7 | 8 | 8 |

In LTE, the PSS is present every 5 ms.

If the SS block were used, it is possible to place an SS block with periodicity 5 ms and at the same symbol location independent of numerology (symbol 8 of slot 0). There are several reasons for this:

- Slot 0 is always downlink. Symbol 8 of a subframe is always a candidate location for SS block. This will simplify test description.

- By avoiding symbol 2, DMRS for the PDSCH is always present (not overwritten by SS block).

- It is possible to make the DMRS of PDSCH QCL-ed to the PSS

Proposal 2: If SS block were used, the periodicity is 5 ms and located on symbol 8 of slot 0.

### Frequency location

Unlike the SS block for LTE which is located in the center, the SS block for NR is floating with the location is indicated by the GSCN. Note that in some bands, it is not possible to place the SS block at the center of the band. For example a 5 MHz carrier in band n1 located in [2110-2115], the GSCN is [5279 … 5419]. The frequencies for the first four GSCN are 2112.05, 2112.15, 2112.25, and 2113.25 MHz. However, the center 2112.5 MHz is not referenced by the GSCN. The 5279 GSCN value is 450 kHz from the center but is an exact number of resource elements (30 15 kHz SCS REs from the carrier).

Observation 1: The location of the SS block within a carrier is defined by the GSCN and may not be at the carrier frequency.

### Numerology considerations

Because the SS block occupies 20 RBs, for certain numerology / bandwidth combinations, there is an insufficient number of RBs of that numerology to support the SS block. Those are: (5 MHz, µ=1); (10 MHz, µ=2); and (15 MHz, µ=2). In addition, for FR1 µ=2, a 60 kHz SCS SS block is not defined. Likewise for FR2 µ=2, a 60 kHz SCS SS block is not defined.

If an SS block were used, then for the (5 MHz, µ=1) case, a 15 kHz SCS is needed for the SS block. Likewise, for any FR1 µ=2, a 30 kHz SCS may be needed for the SS block. In this case, 20 RBs  12 REs/RB  30 kHz SCS/RE = 7.2 MHz would be used for 30 kHz SS block while the remaining bandwidth can use 60 kHz SCS.

Observation 2: Mixed numerology may be needed if an SS block were used for any 60 kHz SCS numerology.

Introducing the SS block has several design implications, such as

- Density of SS block and TDD/FDD

- Location of SS block

- Numerology

## Reference symbols

As stated in [7], the overhead for DMRS should be minimized.

For PDSCH, NR has the flexibility to adjust the density of DMRS ranging from minimal (one symbol has DMRS) to a higher frequency of DMRS-bearing symbols. There are two types (type A and type B) [4]. By assuming the first (two) symbol(s) of the slot is PDCCH, the remaining 13 (12) symbols are then PDSCH. Among the Type A specifications for 14 symbols, DL-DMRS-add-pos = 0 has DMRS located on symbol l0. Also single-symbol DM-RS is used if the higher-layer parameter DL-DMRS-max-len is equal to 1. Also assume configuration type 2 is signaled (to lower the number of reference symbols). Note that with TDD, the density aspect may need to be adjusted for the special DL slots. It may be necessary to increase the DMRS for EVM tests. However, the positions of the DMRS-bearing symbols should consider TDD special slots. Those slots have 11, 8, 2, 4, 8 symbols for µ=0…4.

# Summary of tables

The tables in the appendices capture the test models. The tables are structured so that formulas and parameters are used. There may be specific parameters for a given numerology. As example:

High level

- Bandwidth: FR1:  5nnstart≤n≤510(n-3)6≤n≤nstop where nstart and nstop are defined per numerology. Note an analogous formula to determine n from the bandwidth is n=bw/5bw≤25bw10+330≤bw≤100

- Number of resource blocks: This is a lookup table for each numerology and indexed by n above.

- Number of slots for measurements: 10×2μFDD20×2μTDD, either 10 ms for FDD or 20 ms for TDD

- For the first 10 ms, TDD downlink slots are numbered [8]: 0,⋯,25×2μNsymslot-1, 5×2μ,⋯,10×2μ-1, with Nsymslot=14. The pattern repeats for the next 10 ms

- TDD “special” slot is located at slot 25×2μNsymslot and the number of downlink symbols is 25×2μmod Nsymslot in that slot.

PDCCH

- Number of RBs for PDCCH: 6NRB6  and RB locations 0,…,6NRB6-1

- Power level per PDCCH RB: P/6NRB6 where P is the transmit power level.

- PDCCH configuration (cce-REG-MappingType is nonInterleaved, precoderGranularity is allContiguousRBs, pdcch-DMRS-ScramblingID is µ+1 where µ is the numerology)

PDSCH

- For boosting (TM1.2, TM3.2, TM3.3), there are two parameters: Fraction of total RBs, f, and boost level per RB in dB, b. The number of boosted RBs is fNRB. The number of deboosted RBs is NRB-fNRB and the deboost level per RB is 10log10NRB-fNRB×10b/10NRB-fNRB.

- The locations of the boosted RBs are based on a function (parameters are NRB, f1NRB , f2NRB, starting location for a slot g(s)). There will be a table for the parameters with entries given below

| NRB | NRB | f1NRB | f2NRB | g(s) |
| --- | --- | --- | --- | --- |

- For TM2, the location of the single RB can also be based on the same formula.

Proposal 3: The formulas should be used to parameterized the test model design

# Conclusion

This contribution examines some testing parameters

Proposal 1: PDCCH should occupy the entire first symbol(s) of a slot

Proposal 2: If SS block were used, the periodicity is 5 ms and located on symbol 8 of slot 0.

Observation 1: The location of the SS block within a carrier is defined by the GSCN and may not be at the carrier frequency.

Observation 2: Mixed numerology may be needed if an SS block were used for any 60 kHz SCS numerology.

Proposal 3: The formulas should be used to parameterized the test model design

# References

R4-1809115, “On synchronized LTE-NR general Test model parameters for TDD UL/DL configuration,” Ericsson, RAN4#AH-1807, July 2-6, 2018.

R4-1809467, “WF on the test model,” Huawei, RAN4AH-1807, July 2-6, 2018.

36.141

38.211

38.213

38.331

R4-1809240, “Channel parameters for TM design”, Huawei, RAN4#AH-1807, July 2-6, 2018.

R4-1810162, “TDD configurations for MSR considerations”, Huawei, RAN4#88, Aug. 20-24, 2018.

# Appendix

## Appendix A. TDD downlink slots

Reference [8] details the TDD frame structure. Table 4 lists the downlink and uplink slots for the TDD configuration.

Table 4 Nsymslot=14, with 102µ slots each 10 ms

| Slot Type | Time period | slots |
| --- | --- | --- |
| Downlink | 0 – 5 ms | 0,⋯,25×2μNsymslot-1, |
| Special slot with downlink symbols | 0 – 5 ms | Slot 25×2μNsymslot with first 25×2μmod Nsymslot downlink symbols |
| Special slot with uplink symbols | 0 – 5 ms | Slot 5×2μ-44×2μNsymslot-1 with last 44×2μmod Nsymslot symbols reserved for uplink |
| Uplink | 0 – 5 ms | 5×2μ-44×2μNsymslot,⋯, 5×2μ-1 |
| Downlink | 5 - 10 | 5×2μ,⋯,10×2μ-1 |

## Appendix B. RB selector

Table 5 specifies the parameters for the function indexed by the number of resource blocks. This function is used for turbo interleaving in LTE. The parameters can be filled out if this approach is agreed.

The parameters are used in the formula

pk=gs+f1k+f2k2mod NRB'

with

gs=smodNsubframeframe×N/Nsubframeframe

and s is the slot number in a subframe and Nsubframeframe=10. If the value p(k) = NRB, skip that value by shifting the sequence: use p(k) = p(k+1).

Table 5. Function parameters

| NRB | NRB | f1NRB | f2NRB |
| --- | --- | --- | --- |
| 11 | 12 | 1 | 6 |
| 18 | 18 | 11 | 6 |
| 24 | 24 | 13 | 18 |
| 25 | 25 | 13 | 20 |
| 31 | 32 | 29 | 20 |
| 38 | 38 | 10 | 19 |
| 51 | 52 | 7 | 26 |
| 52 | 52 | 7 | 26 |
| 65 | 66 | 46 | 33 |
| 78 | 78 | 32 | 39 |
| 79 | 80 | 71 | 40 |
| 93 | 94 | 40 | 47 |
| 106 | 106 | 2 | 53 |
| 107 | 108 | 85 | 48 |
| 121 | 121 | 117 | 77 |
| 133 | 134 | 98 | 67 |
| 135 | 135 | 31 | 90 |
| 160 | 160 | 157 | 90 |
| 162 | 162 | 160 | 159 |
| 189 | 189 | 160 | 105 |
| 216 | 216 | 215 | 66 |
| 217 | 218 | 166 | 109 |
| 245 | 245 | 161 | 66 |
| 270 | 270 | 4 | 75 |
| 273 | 274 | 4 | 137 |

## Appendix C. Resource blocks

Table 6 parameterizes the bandwidth as a function of numerology and bandwidth.

Table 6. Number of RBs function

|  | 15 kHz SCS µ=0 | 30 kHz SCS µ=1 | 60 kHz SCS µ=2 |
| --- | --- | --- | --- |
| F(n) | [25 52 79 106 133 160 216 270] | [11 24 38 51 65 78 106 133 162 189 217 245 273] | [ n.a., 11 18 24 31 38 51 65 79 93 107 121 135] |

## Appendix D. TM1.1 FR1

Parameterized values for TM1.1 listed in Table 7

Table 7. TM1.1

| Parameter | Formula |
| --- | --- |
| System parameter |  |
| Bandwidth index n | bw/5bw≤25bw10+330≤bw≤100 |
| Number of RBs, NRB | F(n), see Table 6 for the mapping |
| Number of slots | 102µ for FDD, 202µ for TDD, starting from s=0 |
| PDCCH |  |
| Number of symbols | 1 |
| Number of occupied RBs | 6NRB6 |
| Location of RBs | 0,…,6NRB6-1 |
| # of CCEs per PDCCH | 6NRB6 |
| # of REGs per CCE | 4 |
| # of REGs allocated to PDCCH | 4 |
| PRB PA | P/6NRB6 |
| PDSCH |  |
| # of QPSK PDSCH PRBs which are boosted | NRB |
| RB PA = EA/EDMRS PDSCH [dB] | 0 |
| # of QPSK PDSCH PRBs which are de-boosted | 0 |
| PRB PA = EA/ERS [dB] | n/a |

## Appendix E. TM1.2 FR1

Parameterized values for TM1.2 listed in Table 8

Table 8. TM1.2, FR1

| Parameter | Formula |
| --- | --- |
| System parameter |  |
| Bandwidth index n | bw/5bw≤25bw10+330≤bw≤100 |
| Number of RBs, NRB | F(n), see Table 6 for the mapping |
| Number of slots | 102µ for FDD, 202µ for TDD, starting from s=0 |
| PDCCH |  |
| Number of symbols | 1 |
| Number of occupied RBs | 6NRB6 |
| Location of RBs | 0,…,6NRB6-1 |
| # of CCEs per PDCCH | 6NRB6 |
| # of REGs per CCE | 4 |
| # of REGs allocated to PDCCH | 4 |
| PRB PA | P/6NRB6 |
| PDSCH |  |
| Fraction f of boosted RB | 0.4 |
| # of QPSK PDSCH PRBs which are boosted | fNRB |
| RB PA = EA/EDMRS PDSCH [dB] b | 3 dB |
| # of QPSK PDSCH PRBs which are de-boosted | NRB-fNRB |
| RB PA = EA/ERS [dB] | 10log10NRB-fNRB×10b/10NRB-fNRB |
| Position of boosted RBs for downlink slot s | pi=sortgs+f1i+f2i2modNRB i=0, ⋯, f×NRB-1 in Table 5 |

## Appendix F. TM2 FR1

Parameterized values for TM2(a) listed in Table 9 for 64QAM and 256QAM.

Table 9. TM2, FR1

| Parameter | Formula |
| --- | --- |
| System parameter |  |
| Bandwidth index n | bw/5bw≤25bw10+330≤bw≤100 |
| Number of RBs, NRB | F(n) , see Table 6 for the mapping |
| Number of slots | 102µ for FDD, 202µ for TDD, starting from s=0 |
| PDCCH |  |
| Number of symbols | 1 |
| Number of occupied RBs | 6NRB6 |
| Location of RBs | 0,…,6NRB6-1 |
| # of CCEs per PDCCH | 6NRB6 |
| # of REGs per CCE | 4 |
| # of REGs allocated to PDCCH | 4 |
| PRB PA | P/6NRB6 |
| PDSCH |  |
| Fraction f of boosted RB | n.a. |
| # of 64QAM PDSCH PRBs which are boosted | 1 |
| RB PA = EA/EDMRS PDSCH [dB] b | 0 dB |
| # of 64QAM PDSCH PRBs which are de-boosted | 0 |
| RB PA = EA/ EDMRS [dB] | Inf. |
| Position of boosted RBs for downlink slot s | ps=f1s+f2s2modNRB in Table 5 |

## Appendix G. TM3.1 FR1

Parameterized values for TM3.1(a) listed in Table 10

Table 10. TM3.1, FR1

| Parameter | Formula |
| --- | --- |
| System parameter |  |
| Bandwidth index n | bw/5bw≤25bw10+330≤bw≤100 |
| Number of RBs, NRB | F(n), see Table 6 for the mapping |
| Number of slots | 102µ for FDD, 202µ for TDD, starting from s=0 |
| PDCCH |  |
| Number of symbols | 1 |
| Number of occupied RBs | 6NRB6 |
| Location of RBs | 0,…,6NRB6-1 |
| # of CCEs per PDCCH | 6NRB6 |
| # of REGs per CCE | 4 |
| # of REGs allocated to PDCCH | 4 |
| PRB PA | P/6NRB6 |
| PDSCH |  |
| Fraction f of boosted RB | n.a. |
| # of 64QAM PDSCH PRBs which are boosted | NRB |
| RB PA = EA/EDMRS PDSCH [dB] b | 0 dB |
| # of 64QAM PDSCH PRBs which are de-boosted | 0 |
| RB PA = EA/ EDMRS [dB] | n.a |

## Appendix H. TM3.2 FR1

Parameterized values for TM3.2 listed in Table 11

Table 11. TM3.2, FR1

| Parameter | Formula |
| --- | --- |
| System parameter |  |
| Bandwidth index n | bw/5bw≤25bw10+330≤bw≤100 |
| Number of RBs, NRB | F(n), see Table 6 for the mapping |
| Number of slots | 102µ for FDD, 202µ for TDD, starting from s=0 |
| PDCCH |  |
| Number of symbols | 1 |
| Number of occupied RBs | 6NRB6 |
| Location of RBs | 0,…,6NRB6-1 |
| # of CCEs per PDCCH | 6NRB6 |
| # of REGs per CCE | 4 |
| # of REGs allocated to PDCCH | 4 |
| PRB PA | P/6NRB6 |
| PDSCH |  |
| Fraction f of boosted RB | 0.6 |
| # of 16QAM PDSCH PRBs for which EVM is measured | fNRB |
| RB PA = EA/EDMRS PDSCH [dB] b | -3 dB |
| # of QPSK PDSCH PRBs for which EVM is not measured | NRB-fNRB |
| PRB PA = EA/ERS [dB] | 10log10NRB-fNRB×10b/10NRB-fNRB |
| Position of boosted RBs for downlink slot s | pi=sortgs+f1i+f2i2modNRB i=0, ⋯, f×NRB-1 in Table 5 |

## Appendix I. TM3.3 FR1

Parameterized values for TM3.3 listed in Table 12

Table 12. TM3.3, FR1

| Parameter | Formula |
| --- | --- |
| System parameter |  |
| Bandwidth index n | bw/5bw≤25bw10+330≤bw≤100 |
| Number of RBs, NRB | F(n) , see Table 6 for the mapping |
| Number of slots | 102µ for FDD, 202µ for TDD, starting from s=0 |
| PDCCH |  |
| Number of symbols | 1 |
| Number of occupied RBs | 6NRB6 |
| Location of RBs | 0,…,6NRB6-1 |
| # of CCEs per PDCCH | 6NRB6 |
| # of REGs per CCE | 4 |
| # of REGs allocated to PDCCH | 4 |
| PRB PA | P/6NRB6 |
| PDSCH |  |
| Fraction f of boosted RB | 0.5 |
| # of QPSK PDSCH PRBs for which EVM is measured | fNRB |
| RB PA = EA/EDMRS PDSCH [dB] b | -6 dB |
| # of 16QAM PDSCH PRBs for which EVM is not measured | NRB-fNRB |
| RB PA = EA/ EDMRS [dB] | 10log10NRB-fNRB×10b/10NRB-fNRB |
| Position of boosted RBs for downlink slot s | pi=sortgs+f1i+f2i2modNRB i=0, ⋯, f×NRB-1 in Table 5 |

## Appendix J. PDCCH formula

For completeness, this appendix illustrates how a PDCCH can be placed in the center blocks in the first two symbols of a DL slot. For a bandwidth NRB at RBs the function P(i) indicates the RB locations where

Pi=6Nblock-12+i, i=0, …,min6Nblock,12-1

where

Nblock=NRB6

To illustrate the mapping rule, a bandwidth range of 40 to 60 RBs is examined below

Table 13. 2 block PRB example

| NRB | Nblock | P(i) | Center + (-6, 0, 5) |
| --- | --- | --- | --- |
| 40 | 6 | 12 … 23 | 14, 20, 25 |
| 44 | 7 | 18 … 29 | 16, 22, 27 |
| 48 | 8 | 18 … 29 | 18, 24, 29 |
| 52 | 8 | 18 … 29 | 20, 26, 31 |
| 56 | 9 | 24 … 35 | 22, 28, 33 |
| 60 | 10 | 24 … 35 | 24, 30, 35 |