3GPP TSG-RAN WG4 Meeting #88	R4-1810254

# Gothenburg, SE, 20th – 24th Aug, 2018

Agenda item:			7.12.1

Source:	MediaTek Inc.

Title:	Discussion on RRM measurement accuracy

Document for:		Discussion

1	Introduction

In [1], the link-level simulation assumptions for SSB-based measurement accuracy was agreed, and the conditions for RRC_CONNECTED state and RRC_IDLE state are given in [3] and [4], respectively.

In this contribution, the simulation results of RSRP and SINR accuracy for both FR1 and FR2 are provided. The following accuracy requirement for FR1 are proposed:

- SS RSRP inter frequency absolute accuracy in FR1

- SS SINR intra frequency absolute accuracy in FR1

- SS SINR inter frequency absolute accuracy in FR1

- SS SINR inter frequency relative accuracy in FR1

Besides, accuracy requirement for FR2 is discussed.

2	SS-RSRP Inter frequency absolute accuracy in FR1

In the current 38.133 [2], Table 10.1.4.1.1-1 is missing for SS-RSRP inter frequency absolute accuracy in FR1. Since SNR side condition of -4 dB was assumed in [1][3] for inter-frequency measurement, the table is proposed to be changed with the updated Ês/Iot.

Proposal 1: For SS-RSRP inter frequency absolute accuracy in FR1, Ês/Iot is -4dB. The corresponding changes on the table is given:

Table 1: SS RSRP Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [4.5] | [9] | [-4] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -70 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -70 |
| [8] | [11] | [-4] dB | All | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. |  |  |  |  |  |  |  |

3	SS-SINR measurement accuracy for FR1

The SS-SINR simulation results are given in Appendix. Since SNR side condition of -4 dB was assumed in [1][3] for inter-frequency measurement, it is proposed to revise SNR side condition of -6dB to be -4dB for inter-frequency measurement. Since less available REs for estimating the interference part of SINR, comparing to the RSSI part of SS-RSRQ, some margin should be considered. Therefore, the same accuracy requirements as LTE are proposed.

Proposal 2: For SS-SINR measurement accuracy in FR1, the intra frequency absolute accuracy, inter frequency absolute accuracy, and inter frequency relative accuracy are given:

Table 2: SS SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | [-3] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -50 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -50 |
| [3.5] | [4] | [-6] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Table 3: SS SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | [-3] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -50 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -50 |
| [3.5] | [4] | [-4] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Table 4: SS SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | [-3] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -50 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -50 |
| [4] | [4] | [-4] dB | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

4	Discussion on RRM measurement accuracy for FR2

In FR2, calibration errors are higher than in FR1, e.g. more paths of antenna elements and more stages of concatenated RF component in FR2 would introduce more calibration complexity and variation, so a higher margin should be applied.

Proposal 3: For FR2, a higher margin for RRM measurement accuracy should be applied.

However, the FR2 test methodology for RRM has not yet been finalized. The testability of corresponding requirements are still open. We would suggest to keep the previous agreement for now. Once the test methodology is clarified, RAN4 can revisit the accuracy requirement.

5	Summary

In this contribution, the simulation results for SS-RSRP and SS-SINR measurement accuracy are presented.

And we propose:

Proposal 1: For SS-RSRP inter frequency absolute accuracy in FR1, Ês/Iot is -4dB. The corresponding changes on the table is given:

Table 1: SS RSRP Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [4.5] | [9] | [-4] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -70 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -70 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -70 |
| [8] | [11] | [-4] dB | All | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. |  |  |  |  |  |  |  |

Proposal 2: For SS-SINR measurement accuracy in FR1, the intra frequency absolute accuracy, inter frequency absolute accuracy, and inter frequency relative accuracy are given:

Table 2: SS SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | [-3] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -50 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -50 |
| [3.5] | [4] | [-6] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Table 3: SS SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | [-3] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -50 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -50 |
| [3.5] | [4] | [-4] dB | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Table 4: SS SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | [-3] dB | NRFDD_FR1_A, NRTDD_FR1_A | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_B | TBD | TBD | N/A | -50 |
|  |  |  | NRTDD_FR1_C | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_G | TBD | TBD | N/A | -50 |
|  |  |  | NRFDD_FR1_H | TBD | TBD | N/A | -50 |
| [4] | [4] | [-4] dB | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Proposal 3: For FR2, a higher margin for RRM measurement accuracy should be applied.

6	Reference

[1] R4-1806551, Link-level simulation assumptions for SSB-based measurement accuracy, Ericsson, May 2018.

[2] 3GPP TS 38.133: "NR; Requirements for support of radio resource management (V15.2.1).

[3] R4-1809540, CR, “Conditions for measurements and accuracy requirements for RRC_CONNECTED in 38.133”, Ericsson.

[4] R4-1809539, CR, “Conditions for measurements and accuracy requirements for RRC_IDLE in 38.133”, Ericsson.

Appendix:	RSRP simulation results

In this section, we compare the measurement accuracy of NR-SSS based RSRP on different sub-carrier spacing (SCS), sample number (N) and channel type, with detail setting parameters shown in [1]. Here, Table 1 and Table 3 summarizes the measurement accuracies and values of 5%, 50%,95% points on the CDF curves of Delta RSRP in FR1 and FR2, respectively. Table 2 and Table 4 summarizes the measurement accuracy and values of 5%, 50%,95% points on the CDF curves of Delta RSRP in FR1 and FR2, respectively.

Table 1: NR-SSS based RSRP measurement accuracy in FR1, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  |  | SCS = 30 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Accuracy | 5% | 50% | 95% | Accuracy |
| AWGN | 1 | -0.99 | 0.0 | 0.95 | 0.96 | -1.06 | -0.07 | 0.91 | 0.99 |
|  | 3 | -0.8 | -0.12 | 0.57 | 0.73 | -0.87 | -0.16 | 0.53 | 0.76 |
|  | 5 | -0.74 | -0.15 | 0.48 | 0.68 | -0.81 | -0.18 | 0.17 | 0.74 |
| EPA5 | 1 | -1.42 | -0.26 | 0.98 | 1.39 | -1.74 | -0.51 | 0.77 | 1.52 |
|  | 3 | -1.16 | -0.26 | 0.59 | 1.01 | -1.41 | -0.51 | 0.43 | 1.27 |
|  | 5 | -1.02 | -0.26 | 0.45 | 0.89 | -1.34 | -0.49 | 0.33 | 1.22 |
| ETU30 | 1 | -1.53 | -0.27 | 1.11 | 1.36 | N/A | N/A | N/A | N/A |
|  | 3 | -1.15 | -0.29 | 0.84 | 1.08 | N/A | N/A | N/A | N/A |
|  | 5 | -1.09 | -0.29 | 0.77 | 1 | N/A | N/A | N/A | N/A |
| ETU70 | 1 | -1.5 | -0.19 | 1.06 | 1.34 | N/A | N/A | N/A | N/A |
|  | 3 | -1.22 | -0.23 | 0.77 | 1.04 | N/A | N/A | N/A | N/A |
|  | 5 | -1.09 | -0.24 | 0.68 | 0.97 | N/A | N/A | N/A | N/A |

Table 2: NR-SSS based RSRP measurement accuracy in FR1, side condition Es/IoT = -3 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  |  | SCS = 30 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Accuracy | 5% | 50% | 95% | Accuracy |
| AWGN | 1 | -1.08 | 0.14 | 1.41 | 1.3 | -1.01 | 0.26 | 1.46 | 1.32 |
|  | 3 | -0.71 | 0.13 | 1.02 | 0.92 | -0.6 | 0.23 | 1.08 | 0.98 |
|  | 5 | -0.58 | 0.14 | 0.93 | 0.82 | -0.49 | 0.22 | 0.94 | 0.86 |
| EPA5 | 1 | -1.59 | 0.10 | 1.95 | 1.81 | -1.33 | 0.34 | 1.91 | 1.7 |
|  | 3 | -1.04 | 0.10 | 1.31 | 1.18 | -0.7 | 0.34 | 1.37 | 1.2 |
|  | 5 | -0.89 | 0.12 | 1.12 | 1.03 | -0.56 | 0.36 | 1.17 | 1.06 |
| ETU30 | 1 | -1.53 | 0.23 | 2.05 | 1.88 | N/A | N/A | N/A | N/A |
|  | 3 | -1.16 | 0.26 | 1.78 | 1.52 | N/A | N/A | N/A | N/A |
|  | 5 | -1.01 | 0.26 | 1.62 | 1.42 | N/A | N/A | N/A | N/A |
| ETU70 | 1 | -1.66 | 0.21 | 2.03 | 1.89 | N/A | N/A | N/A | N/A |
|  | 3 | -1.25 | 0.23 | 1.63 | 1.49 | N/A | N/A | N/A | N/A |
|  | 5 | -1.03 | 0.275 | 1.46 | 1.34 | N/A | N/A | N/A | N/A |

Table 3: NR-SSS based RSRP measurement accuracy in FR2, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Accuracy |
| TDL-A delay spread: 30 ns UE speed: 3 km/h | 1 | -1.52 | -0.28 | 0.78 | 1.04 |
|  | 3 | -1.14 | -0.29 | 0.48 | 0.83 |
|  | 5 | -1.02 | -0.28 | 0.36 | 0.79 |
| TDL-B delay spread: 30 ns UE speed: 3 km/h | 1 | -1.58 | -0.3 | 1.06 | 1.1 |
|  | 3 | -1.19 | -0.28 | 0.63 | 0.84 |
|  | 5 | -1.06 | -0.30 | 0.45 | 0.78 |
| TDL-C delay spread: 30 ns UE speed: 3 km/h | 1 | -1.42 | -0.28 | 0.88 | 1.02 |
|  | 3 | -1.12 | -0.29 | 0.5 | 0.81 |
|  | 5 | -1.04 | -0.28 | 0.4 | 0.74 |
| TDL-A delay spread: 300 ns UE speed: 30 km/h | 1 | -2.46 | -0.67 | 0.96 | 1.57 |
|  | 3 | -2.04 | -0.37 | 0.5 | 1.41 |
|  | 5 | -1.8 | -0.68 | 0.365 | 1.32 |
| TDL-B delay spread: 300 ns UE speed: 30 km/h | 1 | -2.67 | -0.80 | 0.73 | 1.77 |
|  | 3 | -2.25 | -0.84 | 0.26 | 1.62 |
|  | 5 | -2.02 | -0.88 | 0.11 | 1.56 |
| TDL-C delay spread: 300 ns UE speed: 30 km/h | 1 | -2.05 | -0.56 | 0.65 | 1.35 |
|  | 3 | -1.52 | -0.55 | 0.42 | 1.15 |
|  | 5 | -1.4 | -0.53 | 0.28 | 1.06 |

Table 4: NR-SSS based RSRP measurement accuracy in FR2, side condition Es/IoT = -3 dB

| Channel | N | Distribution of delta RSRP [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Accuracy |
| TDL-A delay spread: 30 ns UE speed: 3 km/h | 1 | -1.4 | 0.23 | 1.98 | 1.77 |
|  | 3 | -1.07 | 0.19 | 1.62 | 1.36 |
|  | 5 | -0.92 | 0.22 | 1.38 | 1.18 |
| TDL-B delay spread: 30 ns UE speed: 3 km/h | 1 | -1.6 | 0.22 | 2.05 | 1.81 |
|  | 3 | -1 | 0.17 | 1.61 | 1.37 |
|  | 5 | -0.88 | 0.17 | 1.38 | 1.12 |
| TDL-C delay spread: 30 ns UE speed: 3 km/h | 1 | -1.6 | 0.21 | 1.91 | 1.75 |
|  | 3 | -0.91 | 0.19 | 1.34 | 1.2 |
|  | 5 | -0.77 | 0.21 | 1.25 | 1.05 |
| TDL-A delay spread: 300 ns UE speed: 30 km/h | 1 | -1.51 | 0.20 | 1.95 | 1.72 |
|  | 3 | -1.04 | 0.19 | 1.38 | 1.29 |
|  | 5 | -0.93 | 0.17 | 1.23 | 1.13 |
| TDL-B delay spread: 300 ns UE speed: 30 km/h | 1 | -1.38 | 0.29 | 2.24 | 1.9 |
|  | 3 | -0.85 | 0.32 | 1.65 | 1.43 |
|  | 5 | -1.68 | 0.35 | 1.44 | 1.23 |
| TDL-C delay spread: 300 ns UE speed: 30 km/h | 1 | -1.61 | 0.14 | 2.01 | 1.86 |
|  | 3 | -1.15 | 0.16 | 1.49 | 1.34 |
|  | 5 | -0.93 | 0.18 | 1.26 | 1.14 |

Appendix:	SINR simulation results

In this section, we compare the measurement accuracy of NR-SSS based SINR on different sub-carrier spacing (SCS), sample number (N) and channel type, with detail setting parameters shown in Section 7. Here, Table 5 and Table 6 summarizes the absolute accuracies and values of 5%, 50%,95% points on the CDF curves of Delta SINR in FR1 and FR2, respectively.

Table 5: NR-SSS based SINR measurement accuracy in FR1, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta SINR [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS = 15 kHz |  |  |  | SCS = 30 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Accuracy | 5% | 50% | 95% | Accuracy |
| AWGN | 1 | -1.12 | -0.08 | 1 | 1.06 | -1.14 | -0.12 | 0.96 | 1.06 |
|  | 3 | -0.94 | -0.20 | 0.55 | 0.85 | -0.95 | -0.23 | 0.51 | 0.86 |
|  | 5 | -0.88 | -0.23 | 0.45 | 0.8 | -0.89 | -0.36 | 0.36 | 0.81 |
| EPA5 | 1 | -1.72 | -0.25 | 1.12 | 1.44 | -1.65 | -0.35 | 0.87 | 1.45 |
|  | 3 | -1.28 | -0.29 | 0.59 | 1.1 | -1.33 | -0.43 | 0.44 | 1.18 |
|  | 5 | -1.13 | -0.33 | 0.48 | 1 | -1.25 | -0.45 | 0.29 | 1.09 |
| ETU30 | 1 | -1.62 | -0.20 | 1.17 | 1.42 | N/A | N/A | N/A | N/A |
|  | 3 | -1.33 | -0.29 | 0.76 | 1.18 | N/A | N/A | N/A | N/A |
|  | 5 | -1.25 | -0.30 | 0.67 | 1.12 | N/A | N/A | N/A | N/A |
| ETU70 | 1 | -1.59 | -0.20 | 1.11 | 1.4 | N/A | N/A | N/A | N/A |
|  | 3 | -1.38 | -0.26 | 0.66 | 1.16 | N/A | N/A | N/A | N/A |
|  | 5 | -1.25 | -0.28 | 0.55 | 1.08 | N/A | N/A | N/A | N/A |

Table 6: NR-SSS based SINR measurement accuracy in FR2, side condition Es/IoT = -6 dB

| Channel | N | Distribution of delta SINR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS = 120 kHz |  |  |  |
|  |  | 5% | 50% | 95% | Accuracy |
| TDL-A delay spread: 30 ns UE speed: 3 km/h | 1 | -1.53 | -0.27 | 0.94 | 1.34 |
|  | 3 | -1.3 | -0.34 | 0.55 | 1.11 |
|  | 5 | -1.17 | -0.34 | 0.44 | 1.02 |
| TDL-B delay spread: 30 ns UE speed: 3 km/h | 1 | -1.6 | -0.28 | 1.08 | 1.4 |
|  | 3 | -1.24 | -0.32 | 0.62 | 1.11 |
|  | 5 | -1.13 | -0.34 | 0.49 | 1.02 |
| TDL-C delay spread: 30 ns UE speed: 3 km/h | 1 | -1.54 | -0.30 | 0.93 | 1.37 |
|  | 3 | -1.19 | -0.33 | 0.48 | 1.06 |
|  | 5 | -1.1 | -0.35 | 0.37 | 0.96 |
| TDL-A delay spread: 300 ns UE speed: 30 km/h | 1 | -2.35 | -0.72 | 0.82 | 2.02 |
|  | 3 | -1.97 | -0.77 | 0.38 | 1.69 |
|  | 5 | -1.73 | -0.80 | 0.2 | 1.59 |
| TDL-B delay spread: 300 ns UE speed: 30 km/h | 1 | -2.52 | -0.80 | 0.72 | 2.2 |
|  | 3 | -2.21 | -0.89 | 0.15 | 1.94 |
|  | 5 | -1.99 | -0.94 | -0.04 | 1.84 |
| TDL-C delay spread: 300 ns UE speed: 30 km/h | 1 | -2.05 | -0.55 | 0.72 | 1.7 |
|  | 3 | -1.67 | -1.62 | 0.46 | 1.48 |
|  | 5 | -1.48 | -0.61 | 0.31 | 1.38 |