3GPP TSG-RAN WG4 Meeting #88                	R4-1810877

Gothenburg, SE, 20th – 24th Aug, 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining general issues for NR BS demodulation

Agenda item:	7.13.2.1

Document for:	Discussion

Introduction

At RAN4#AH-1807 the scope of the work in Rel-15 and the simulation assumptions for alignment have been agreed, and some open issues on general demodulation are listed for further discussion.

The general open issues in [1] are copied below.

- TDD UL-DL configuration - FR1: FFS which UL/DL configuration is used for the TDD tests - FR2: - Option 1: {DDDSU}, S = {D10, G2, U2} - Option 2: {DDSU}, S={D11, G3} - Option 3: {DDSU}, S={D8, G3, U3} - Option 4: {DDDSDDDSUU}, S={D10,G2,U2} - Other options not precluded - FFS if requirement should be defined for one or multiple UL/DL configuration - PT-RS - FFS whether PN is modelled in the test cases. - Antenna configuration - FFS solutions to conduct conformance testing for BS with >8 antenna connectors

In this paper, we will provide our views on the general open issues for NR BS demodulation work.

Discussion

TDD UL/DL configuration

The UL/DL configurations for TDD has been discussed for several meetings in RAN4 for UE REFSENS. As NR supports very flexible UL/DL configuration, it is quite difficult to select a subset of the UL/DL configurations for requirements definition. But when we consider some NR bands are overlapping with LTE bands, TDD configuration of NR shall be aligned with LTE to avoid the interference issue. Therefore, LTE TDD UL/DL configuration #2, that be widely deployed in the LTE network, can be a baseline for BS demodulation. In addition, it is agreed to use this configuration for UE REFSENS.

For BS demodulation, the UL/DL configuration is not really impacting the performance assuming transmission is only scheduled in UL slots. Therefore, there is no need to define BS demodulation requirements with multiple UL/DL configurations, but RAN4 can choose one typical configuration from what is agreed for UE REFSENS tests. The following agreed configuration for UE REFSENS can be used for test setting.

Consider TDD UL/DL configurations as below for NR BS performance requirement.

- For FR1 and FR2:  Slot pattern={DDDSU}, S = {D10, G2, U2}

PTRS

In NR, PTRS was introduced to mitigate phase noise for FR2. It is agreed that PTRS shall be modeled in FR2 tests, but FFS whether PN is modelled in the test cases. In our view, the phase noise should not be modeled in the simulation as it will be very difficult to find an agreeable model based on RAN1 experience. On the other hand, it is nearly impossible to really reflect the phase noise in the performance test.

Phase noise is not explicitly modeled in the performance requirements.

Antenna configuration

As agreed in [1] the Rx antenna setup in LTE, i.e. 2Rx, 4Rx and 8Rx, shall be re-used in NR for conducted requirements and the performance requirement shall be defined for Maximum 8Rx. For BS with more than 8RX, we think the requirements of 2Rx, 4Rx and 8Rx would apply and which one apply to be based on BS declaration. The issue is that how to conduct the test for BS with more than 8Rx. Our proposal is that BS implements a test mode with special Rx beamforming such that the input signals from test equipment can be directly transferred to the baseband.

Conclusion

In this paper, we provided our views on general issues for NR BS demodulation work.

- Proposal 1:Consider TDD UL/DL configurations as below for NR BS performance requirement

- For FR1 and FR2:  Slot pattern={DDDSU}, S = {D10, G2, U2}

- Proposal 2:Phase noise is not explicitly modeled in the FR2 performance requirements.

References

- R4-1809370,	Way forward on General part of NR BS demodulation performance, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, ZTE, CATT