3GPP TSG-RAN WG4 #88	R4-1811231

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	7.13.2.2

Source:	Ericsson

Title:	Discussions for PUSCH demodulation requirements

Document for:	Discussion

# Introduction

In [1], Test setups for NR BS PUSCH were discussed. But there are still quite a lot of open issues left for NR PUSCH. In this paper, we share our view on these open issues.

# Discussion

## Time domain resource allocation issue

For time domain resource allocation, we have the following agreements:

- FR1

  - Slot-based transmission with resource mapping type A

  - FFS:

    - resource mapping type B

    - non-slot-based transmission

- FR2

  - Non-slot-based transmission with resource mapping type B and X UL symbols

    - X is FFS.

  - FFS:

    - slot-based transmission

    - Resource mapping type A

Currently, we already have slot-based transmission with resource mapping type A for FR1 and non-slot-based transmission with resource type B for FR2. If we add resource type B + non-slot-based transmission for FR1 and slot-based + resource mapping type A for FR2, we will double the PUSCH test cases. To reduce the test cases, we prefer to just focus on the slot-based transmission with resource type A for FR1 and non-slot-based transmission with resource mapping type B for FR2.

Number of PUSCH symbols for non-slot-based transmission for FR2

For downlink heavy TDD patterns for FR2, it is necessary to consider scenarios where both PDCCH and PUCCH transmissions can occur in the same slot as PUSCH transmission. For a good compromise between UL/DL latency and UL data throughput, it is suggested to consider PUSCH mapping type B with 8 PUSCH symbols.

## DMRS configuration issue

According to the last meeting agreements, the group will define performance requirements for DMRS type 1, but FFS if requirements are defined for DMRS type 2. According to the RAN1 discussion, DMRS type 2 can get more beneficial for MU-MIMO. In RAN4, in the first release, we define performance requirements only for single UE. Thus, we can down-prioritize the requirements for DMRS-type 2 in Rel-15.

## Limited buffer rate matching issue

For BS, the benefit to use limited buffer rate matching and make sacrifice for the performance, it is not so attractive. Thus, we propose not to consider the limited buffer rate matching for the time being.

## PT-RS pattern issue when transform precoding is enabled

In TS38.214, it is specified that the PTRS pattern is tied with the following association table.

Table 6.2.3-3: PT-RS pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

Defaults thresholds or sample density for PT-RS patterns are not defined in 3GPP 38.214 when transform precoding is enabled. In [2], default thresholds NRB0=0, NRB1=8, NRB2=NRB3=32, and NRB4=108 were proposed and showed good performance. It is thus proposed to consider NRB0=0, NRB1=8, NRB2=NRB3=32, and NRB4=108 for configuring PT-RS when transform precoding is enabled.

# Conclusion

In this paper, we share our view on the open issues for NR PUSCH demodulation. We hope the group can consider these views in the final agreement for NR PUSCH demodulation.

# References

R4-1809336, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, July, 2018

R1-1800713, “Further evaluations on PTRS”, Ericsson