3GPP TSG-RAN WG4 Meeting #88 	R4-1811473

# Gothenburg, Sweden, 20-24 August, 2018 									Revision of R4-1811262

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.101-3 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR TS 38.101-3: Corrections for B41/n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Errors found in the spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleted DC_41-41-n41 as it was not in the WID, Added section header 6.2B.3.10, 6.2B.3.1.2.0, 6.2B.3.2.0 to avoid hanging paragraphs, Added Table 6.2B.3.2-2, corrected various errors, changed Ffilter,high from 2750 MHz to 2745MHz, changed LTE to E-UTRA Changes in R4-18xxxxx: Corrected section numbers for Case A and Case B in 6.2B.3.1.2.0, divided the equations for A by 1,000,000 as A is in MHz, deleted redundant text from section 6.5B.3.1, Added DC_(n)41AA requirements to Table 6.5B.3.1-1, added sections 6.5B.3.2.1 and 6.5B.3.2.2 for intra-band non-contiguous EN-DC, added DC_41A_n41A requirements to Table 6.5B.3.2-1, deleted requirements for DC_(n)41AA and DC_41A_n41A from Table 6.5B.3.3.1-1, Simplified 5.2B.2 so to remove “two bands” since intra-band is only one band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors continue to exist in the spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2B.3.2, 6.2B.3.1.0 (new), 6.2B.3.1.2.0 (new), 6.2B.3.1.2.1, 6.2B.3.1.2.2, 6.2B.3.2.0 (new), 6.2B.3.2.1, 6.5B.3.1, 6.5B.3.1.2, 6.5B.3.2.1 (new), 6.5B.3.2.2 (new), 6.5B.3.3.1, 5.2B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

### 5.2B.2	Intra-band contiguous EN-DC

<Editor’s note: conducted requirements>

Table 5.2B.2-1: Band combinations for intra-band contiguous EN-DC

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_(n)71 | 71 | n71 | No3 |
| DC_(n)41 | 41 | n41 | Yes1 |
| NOTE 1: Single UL allowed due to potential emission issues, not self-interference. NOTE 2: The minimum requirements apply for 15 kHz subcarrier spacing on the SCG. NOTE 3: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. |  |  |  |

### 5.2B.3	Intra-band non-contiguous EN-DC

<Editor’s note: conducted requirements>

Table 5.2B.3-1: Band combinations intra-band non-contiguous EN-DC

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3_n3 | 3 | n3 | Yes1 |
| DC_41_n41 | 41 | n41 | Yes |
| NOTE 1: Only single switched UL is supported in Rel.15 |  |  |  |

<Unchanged sections omitted>

### 6.2B.3	UE additional maximum output power reduction for EN-DC

#### 6.2B.3.1	Intra-band contiguous EN-DC

##### 6.2B.3.1.0	General

For EN-DC band combinations with additional requirements the A-MPR allowed are specified in Table 6.2B.3.1-1 for combinations of network signalling values indicated in E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR allowed below is in addition to the MPR requirements specified in sub-clause 6.2B.2.

Table 6.2B.3.1-1: Allowed power reduction for EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (subclause) |
| --- | --- | --- | --- | --- |
| DC_(n)71B | 6.5B.2.1.2.1 | NS_35 | NS_35 | 6.2B.3.1.1 |
| DC_(n)41AA1 | 6.5B.2.1.2.2 | NS_04 | NS_04 | 6.2B.3.1.2 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The network signalling value for NR is mapped to configured FBI and AdditionalSpecrumEmission values as specified in [4]. |  |  |  |  |

##### 6.2B.3.1.1	A-MPR for DC_(n)71B

For DC_(n)71B with configured with network signaling values as per Table 6.2B.3.1.1-1 the allowed A-MPR is defined by

-	for UE indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE

AMPRDC=CEILMA,DC(A), 0.5

where A-MPRDC is the total power reduction allowed (dB),

-	for OFDM:

MA,DC =	10.00 - 11.67*A;		0.00 < A ≤ 0.30

7.10 -  2.00*A;					0.30 < A ≤ 0.80

5.50;								0.80 < A ≤ 1.00

-	for DFT-S-OFDM:

MA,DC =	10.00 - 13.33*A;  	0.00 < A ≤ 0.30

7.00 - 3.33*A;   					0.30 < A ≤ 0.60

5.00;								0.60 < A ≤ 1.00

where

A=LCRB,LTE+LCRB,NRNRB,LTE+NRB,NR

with LCRB and NRB the number of allocated PRB and transmission bandwidth for the respective CG,

-	for UE not indicating support of dynamicPowerSharing

AMPRLTE=CEIL{MA,LTE, 0.5}

AMPRNR=CEIL{MA,NR, 0.5}

where A-MPR is the total power reduction allowed per CG with

MA,LTE=MA,DC(ALTE,wc)-LTE

MA,NR=MA,DC(ANR,wc)-NR

ALTE,wc=LCRB,LTE+1NRB,LTE+NRB,NR

ANR,wc=1+LCRB,NRNRB,LTE+NRB,NR

∆LTE=10 log10LCRB,LTELCRB,LTE+NRB,NR

∆NR=10 log10LCRB,NRNRB,LTE+LCRB,NR

where ÑRB,NR is the transmission bandwidth configuration of the SCG channel for SCS = 15 kHz.

##### 6.2B.3.1.2	A-MPR for NS_04

6.2B.3.1.2.0	General

When the UE is configured for B41/n41 intra-band contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this subclause.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If FIM3,low_block,low < 2490.5 MHz

Channel Configuration Case B. A-MPRIM3 defined in Subclause 6.2B.3.1.2.2

Else

Channel Configuration Case A. A-MPRIM3 defined in Subclause 6.2B.3.1.2.1

where

-	FIM3,low_block,low = (2 * Flow_channel,low_edge) – Fhigh_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently:

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA, A-MPRIM3 )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3 )

For UEs supporting dynamic power sharing , with backoff applied equally to E-UTRA and NR:

A-MPR = MAX(A-MPRsingle,E-UTRA, A-MPRsingle,NR, A-MPRIM3 )

where

-	A-MPRsingle,E-UTRA is the A-MPR defined for the E-UTRA transmission in [4].

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in [2].

6.2B.3.1.2.1	A-MPRIM3 for NS_04 to meet -13 dBm / 1MHz for 26 dBm UE power

A-MPR in this sub-clause is relative to 26 dBm. For the UE is configured with channel configurations Case A or Case C (defined in Subclause 6.2B.3.2.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 	14	;	 0 ≤ A < 0.5

9	;	 0.5 ≤ A < 1.0

7	; 	1.0 ≤ A < 2.0

5	; 	2.0 < A

Where:

For UEs supporting dynamic power sharing,

A = (NRB_alloc,E-UTRA * 12* SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

A = (NRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

A = (12* SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR)/1,000,000

6.2B.3.1.2.2	A-MPRIM3 for NS_04 to meet -25 dBm / 1MHz for 26 dBm UE power

A-MPR in this sub-clause is relative to 26 dBm. For the UE is configured with channel configurations Case B or Case D (defined in Subclause 6.2B.3.2.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 	14	; 	0 ≤ A < 1.0

13	; 	1.0 ≤ A < 2.0

12	; 	2.0 ≤ A < 5.0

11	; 	5.0 < A

Where:

For UEs supporting dynamic power sharing,

A = (NRB_alloc,E-UTRA * 12* SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

A = (NRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

A = (12 * SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR)/1,000,000

#### 6.2B.3.2	Intra-band non-contiguous EN-DC

##### 6.2B.3.2.0	General

For intra-band EN-DC band combinations with additional requirements the A-MPR allowed are specified in Table 6.2B.3.1-1 for combinations of network signalling values indicated in E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR allowed below is in addition to the MPR requirements specified in sub-clause 6.2B.2.

Table 6.2B.3.2-1: Allowed power reduction for EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (subclause) |
| --- | --- | --- | --- | --- |
| DC_41A_n411 | 6.6.3.3.19 and 6.6.2.2.2 of [4] and 6.5.2.3.2 and 6.5.3.3.1 of [2] | NS_04 | NS_04 | 6.2B.3.2.2 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. |  |  |  |  |

##### 6.2B.3.2.1	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this subclause.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If AND( FIM3,low_block,high < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

Channel Configuration Case C. A-MPRIM3 defined in Subclause 6.2B.3.1.2.1

Else

Channel Configuration Case D. A-MPRIM3 defined in Subclause 6.2B.3.1.2.2

where

-	FIM3,low_block,high = (2 * Flow_channel,high_edge ) – Fhigh_channel,low_edge

-	FIM3,high_block,low = (2 * Fhigh_channel,low_edge) – Flow_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Flow_channel,high_edge is the uppermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,low_edge is the lowermost frequency of upper transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Subclause 6.5B.2.1.2.2.

The UE determines the value of A-MPRACLRoverlap as specified in Table 6.2.B.3.2-2.

E-UTRA

Table 6.2B.3.2-2: A-MPRACLRoverlap

| Wgap | A-MPRACLRoverlap |
| --- | --- |
| < BWchannel,E-UTRA + BWchannel,NR | 4 dB |
| ≥ BWchannel,E-UTRA + BWchannel,NR | 0 dB |
| NOTE 1: Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge |  |

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA, A-MPRIM3, A-MPRACLRoverlap )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap )

For UEs supporting dynamic power sharing , with backoff applied equally to E-UTRA and NR

A-MPR = MAX(A-MPRsingle,E-UTRA, A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap )

where

-	A-MPRsingle,E-UTRA is the A-MPR defined for the E-UTRA transmission in [4].

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in [2].

<Unchanged sections omitted>

### 6.5B.3	Spurious emissions for EN-DC

#### 6.5B.3.1	Intra-band contiguous EN-DC

##### 6.5B.3.1.1	General spurious emissions

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4] and sub-clause 6.5.3.1 of [2] apply beyond any frequencies for which the out-of-band emissions requirements in sub-clause 6.5B.2.1apply.

##### 6.5B.3.1.2	Spurious emission band UE co-existence

The requirements in Table 6.5B.3.1.2-1 apply on each component carrier with all component carriers are active.

Table 6.5B.3.1-1: Requirements for intraband contiguous carrier aggregation

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| DC_(n)71B | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_lowFDL_low | -- | FDL_highFDL_high | -38 | 1 | 3 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| DC_(n)41AA | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |
| … |  |  |  |  |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 [4] from the edge of the channel bandwidth. NOTE 4: This requirement applies when the E-UTRA and NR carriers are confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz |  |  |  |  |  |  |  |

#### 6.5B.3.2	Intra-band non-contiguous EN-DC

##### 6.5B.3.2.1	General spurious emissions

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4] and sub-clause 6.5.3.1 of [2] apply beyond any frequencies for which the out-of-band emissions requirements in sub-clause 6.5B.2.1apply.

##### 6.5B.3.2.2	Spurious emission band UE co-existence

The requirements in Table 6.5B.3.2.2-1 apply on each component carrier with all component carriers are active.

Table 6.5B.3.2-1: Requirements for intraband non-contiguous carrier aggregation

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A_n41A | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 1, 2 |
| … |  |  |  |  |  |  |  |
| NOTE 1: This requirement applies when the E-UTRA and NR carriers are confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 2: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz |  |  |  |  |  |  |  |

#### 6.5B.3.3	Inter-band EN-DC within FR1

< conducted requirements >

< Editor’s note: Chapter numbers to be updated >

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4], sub-clause 6.5.3.1 of [2] and [3] apply for each component carrier.

##### 6.5B.3.3.1	Spurious emission band UE co-existence

< Editor’s note: Chapter numbers to be updated >

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.1-1 apply on each component carrier with all component carriers are active.

Table 6.5B.3.3.1-1: Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A_n28A | E-UTRA Band 18, 19, 27, 31, 32, 72 NR band n5, n7, n8, n20, n26, n38, n40, n41, n50, n51, n74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band42, 43 NR band n78, n75, n76 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR band n3, n34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | E-UTRA Band 65 NR band n1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 18 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,17 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 17 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 10, 16 |
| DC_1A_n40A | Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 17 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 17 |
| DC_1A_n51A | E-UTRA Band 7, 12, 13, 17, 20, 22, 27, 28, 29, 31, 38, 44, 48, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5, 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 17 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 17 |
|  | E-UTRA Band 5, 6, 8, 26, 30, 40, 41, 42, 43, 46 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1A_n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A DC_1A_n84A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n79A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_2A_n5A | Bands 4, 5, 10, 12, 13, 14, 17, 24, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, n71, 74, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 2, 25, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_2A_n66A | Bands 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, n71, 74, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Bands 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2A_n71A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2A_n78A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band 78 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n7A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 NR Band n1, n5, n7, n8, n20, n28, n50, n51, n74, n75, n76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3A_n28A | E-UTRA Band 42, 43, 65 NR band n1, n50, n51, n74, n75, n76, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR band n1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | NR band n3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 27, 31, 72 NR band n5, n7, n8, n20, n26, n34, n38, n40, n41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 14 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 10 |
| DC_3A_n40A | Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3A_n51A | E-UTRA Band 7, 8, 12, 13, 17, 20, 27, 28, 31, 33, 38, 48, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 1, 5, 6, 22, 26, 30, 34, 36, 40, 41, 42, 43, 44, 46, 65, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3A_n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A, DC_3A_n80A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n79A DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n82A | E-UTRA Band 1, 3 7, 8, 20，22, 31, 32, 33, 34, 38, 40, 43, 50, 51, 65, 67, 68, 69, 72,74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5A_n40A | Band 1, 3, 5, 7, 8, 28, 31, 34, 38, 42, 43, 45, 65, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Band 41, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5A_n66A | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 5, 7, 8 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 5, 8 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 7 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| DC_7A_n28A | E-UTRA Band 27, 31, 72 NR band n2, n3, n5, n7, n8, n20, n26, n34, n40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 42, 43, 65 NR band n1, n50, n51, n66, n74, n75, n76, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR band n1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7A_n51A | E-UTRA Band 2, 3, 5, 8, 26, 30, 31, 32, 33, 34, 40, 48, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 17 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 21 |
|  | E-UTRA Band 1, 4, 10, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 5, 7, 8 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 5, 8 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
| DC_8A_n40A | Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 7, 22, 41, 42, 43, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | ３, 13 |
| DC_8A_n77A | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 13 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A | E-UTRA Band 1,8, 20, 28, 34, 39, 40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 13 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | NR Band n258 | 24250 | - | 27500 | -5 | 100 |  |
| DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A | E-UTRA Band 1,8,28,34,39,40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3,41,42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | NR Band n258 | 24250 | - | 27500 | -5 | 100 |  |
| DC_11A_n77A | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_11A_n78A | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_11A_n79A | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_12A_n5A | Bands 2, 5, 12, 13, 14, 17, 24, 25, 30, 42, 43 50, 51, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 4, 10, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_12A_n66A DC_12A_n5A | Bands 2, 4, 5, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 70, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 4, 10, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Bands 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Bands 2, 5, 12, 13, 14, 17, 24, 25, 30, 42, 43 50, 51, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_18A_n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_18A_n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_18A_n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_19A_n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_19A_n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_19A_n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_20A_n8A | E-UTRA Band 1, 3, 7, 22, 28, 31, 32, 34, 38, 42, 43, 65, 75, 76, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20A_n28A DC_20A_n83A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20A_n51A | E-UTRA Band 1, 3, 4, 8, 17, 22, 28, 29, 31, 40, 43, 48, 65, 66, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 32, 33, 34, 35, 36, 37, 38, 39, 41, 42, 46, 69, 70 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20A_n77A | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  |  |  |  |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20A_n78A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  |  |  |  |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_21A_n77A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_21A_n78A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_21A_n79A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_25A_n41A | NR band n5, n28, n66, n71 E-UTRA/NR Band 4, 10, 12, 13 , 14, 17, 24, 26, 27, 29, 30, 42, 45, 48, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n2 E-UTRA/NR Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | EUTRA/NR Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_26A_n41A | E-UTRA/NR Band 1, 2, 3, 4, 5, 10, 12, 13 , 14, 17, 24, 25, 26, 28, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 20 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 20 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26A_n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_26A_n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_26A_n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_28A_n51A | E-UTRA Band 2, 3, 5, 7, 8, 25, 26, 31, 34, 38, 40, 41, 66, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 20, 22, 24, 32, 42, 43, 45, 46, 65, 66, 71, 73 NR band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 10, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 18 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| DC_28A_n77A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_28A_n79A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_30A_n5A | Bands 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_30A_n66A | Bands 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_38A_n78A | N/A |  |  |  |  |  |  |
| DC_39A_n78A | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 19 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 19 |
|  | NR Band n258 | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_39A_n79A | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 19 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 19 |
|  | NR Band n258 | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_40A_n77A | N/A |  |  |  |  |  |  |
| DC_41A_n77A | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 20 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 20 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_41A_n78A | E-UTRA Band 1, 3, 8, 34, 39, 40, 44, 45 or NR Band n1, n8, n34, n40, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_41A_n79A | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 or NR Band n1, n3, n8, n28, n34, n40, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257, n258 | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_42A_n51A | E-UTRA Band 3, 8, 20, 25, 30, 31, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 2, 4, 5, 6, 7, 10, 12, 13, 14, 17, 23, 24, 26, 27, 28, 29, 32, 38, 40, 44, 46, 65, 66, 67, 68, 70, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_42A_n77A | N/A |  |  |  |  |  |  |
| DC_42A_n78A | N/A |  |  |  |  |  |  |
| DC_42A_n79A | N/A |  |  |  |  |  |  |
| DC_66A_n5A | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_66A_n71A | E-UTRA Band 4, 5, 7,10, 13, 14, 17, 22, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range TBD – 3792.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range TBD - 3790 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to TBD RB. NOTE 9: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 10: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE14: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 15: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 16: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 17: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 18: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 19: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 20: This requirement applies when the E-UTRA and NR carriers are confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

<Unchanged sections omitted>