3GPP TSG-RAN Working Group 4 Meeting #88	R4-1811570

Gothenburg, Sweden, 20th August – 24th August 2018	(revision of R4-1811134)

Source:	Nokia and Globalstar

Title:	TP to TR 36.791 (E-UTRA 2.4 GHz TDD Band for US): Special Co-Existence Considerations for Unlicensed Operations in the 2.4 GHz ISM Band

Document for:	Approval

Agenda Item:	          6.2.1

## 1	Introduction

The FCC has recently adopted rules for terrestrial use of the 2.4 GHz MSS band.  This band is authorized for TDD

operation and spans 11.5 MHz from 2483.5 MHz to 2495 MHz.  A new work item was created in RAN#79, to create

specifications for a corresponding new E-UTRA operating band [1].

The WI has a study phase, with the following core objectives:

- Define impact on 2.4 GHz Bluetooth operations

- Define impact on 2.4 GHz 802.11 operations

- Address potential Bluetooth and 802.11 coexistence issues

The present contribution provides a TP to TR 36.791 (E-UTRA 2.4 GHz TDD Band for US) on special co-existence considerations for unlicensed operation in the 2.4 GHz ISM Band.  This information is provided in support of the study phase of the approved work item (WI) for a new US 2.4 GHz TDD Band for LTE.

## 2	References

[1]	  RP-180583, New WI Proposal for E-UTRA 2.4 GHz TDD Band, Globalstar, RAN#79

[2]	  3GPP TR 21.905 “Vocabulary for 3GPP Specifications.”

[3]	Terrestrial Use of the 2473[2483.5]-2495 MHz Band for Low-Power Mobile Broadband Networks; Amendments to Rules for the Ancillary Terrestrial Component of Mobile Satellite Service Systems, IB Docket No. 13-213, Report and Order, FCC 16-181

[4]	47 CFR Part 15.5(b)-(c).  “Operation of an intentional, unintentional, or incidental radiator is subject to the conditions that no harmful interference is caused, and that interference must be accepted”

[5]	Terrestrial Use of the 2473-2495 MHz Band for Low-Power Mobile Broadband Networks; Amendments to Rules for the Ancillary Terrestrial Component of Mobile Satellite Service Systems, IB Docket No. 13-213, Notice of Proposed Rulemaking, FCC 13-147

[6]	47 CFR Part 15.247.  “Operation within the bands 902-928 MHz, 2400-2483.5 MHz, and 5725-5850 MHz.”

[7]	   FCC OET Report, FCC, IB Docket No. 12-213, Filed March 27, 2015.

## 3 	Definitions, Symbols and Abbreviations

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [2].

ATC	Ancillary Terrestrial Component

BAS	Broadcast Auxiliary Service

BRS	Broadband Radio Service

BS	Base Station

DL	Downlink

CFR		Code of Federal Regulation

EBS	Educational Broadband Service

EIRP	Effective Isotropic Radiated Power

E-UTRA	Evolved UTRA

FCC	Federal Communications Commission

FDD	Frequency Division Duplex

ISM	Industrial Scientific Medical

LTE	Long Term Evolution

MSS	Mobile Satellite Service

NOS	Network Operating System

NPRM	Notice of Proposed Rule Making

RF	Radio Frequency

RX	Receiver

TDD	Time Division Duplex

TLPS	Terrestrial Low Power Service

TX	Transmitter

UE	User Equipment

US	United States

## 4	Background

A recent FCC Report and Order has created a band that is authorized for TDD operation and spans 11.5 MHz from 2483.5 MHz to 2495 MHz [3].  This band is the result of a multi-year regulatory effort, which focused principally upon co-existence between Part 25 licensed operations above 2483.5 MHz and Part 15 unlicensed operations below 2483.5 MHz.

This contribution presents the evolution of the terrestrial 2.4 GHz band plan, testing and analysis conducted during the FCC regulatory process, E-UTRA to unlicensed co-existence simulations, and the justification behind FCC rules designed to protect Part 15 unlicensed operations below 2483.5 MHz.  A basic overview of co-existence regulations may be found in a companion TP to TR 36.791 (R4-1811133).

<<< Start of TP>>>

# 6	Special Co-Existence Considerations for Non-3GPP Unlicensed Operations in the 2.4 GHz ISM Band

## 6.1	Introduction

While Part 15 services in the 2.4 GHz band are not entitled to interference protection under FCC rules [4], unlicensed 2.4 GHz applications like 802.11 Wi-Fi and Bluetooth are of special significance.  This section outlines the regulatory history and technical basis for co-existence between the US E-UTRA 2.4 GHz TDD Band and non-3GPP unlicensed operation in the 2.4 GHz ISM Band.

## 6.2	2.4 GHz Terrestrial Band Plan Evolution

## The Globalstar satellite network operates in a downlink configuration from 2483.5 MHz to 2500 MHz.  Following the establishment of the ATC regulatory construct, 2483.5 MHz to 2495 MHz became available for terrestrial allocation.  The satellite downlink band from 2496 MHz to 2500 MHz was already allocated for terrestrial use as a portion of the high power BRS-1 service, which is now integrated into 3GPP Band 41.

More details concerning the FCC ruling on this specific band could be found in [3].

## 6.3	Ratified 2.4 GHz Terrestrial Band Relative to Common Unlicensed Applications

## 6.3.1	2.4 GHz Terrestrial Band Relative to Entire Unlicensed Band in the United States

This band is dedicated only for US usage. From regulatory perspective, FCC approved this band after balancing the needs of coexistent WLAN and BT access technologies in the ISM band.  More details could be found in [3].

For informative purposes, Figure 6.3.1-1 depicts a spectrogram taken between 2400 MHz and 2483.5 MHz in Boston, which reflects some of the challenges related to deploying this band in US band.  Overlaid atop the spectrogram are available 802.11 and Bluetooth channel configurations.  As expected, the three channel 802.11 system predominates, with significant activity on Channels 1, 6, and 11.  Intermediate (overlapping) 802.11 channel use and microwave oven activity contribute to a generally high noise and interference floor.  In this example, all three Bluetooth advertising channels are clearly evident.  In this informative example, the advertising BT channel at 2480 MHz appears to be not impacted by interference.

- Figure 6.3.1-1:  Informative Spectrogram of 2.4 GHz Band Spanning 2400 MHz to 2495 MHz (Boston, US)

## 6.3.2	2.4 GHz Terrestrial Band Relative to Unlicensed Band in the Rest of World (RoW)

It should be noted that the band plan and configuration of theis2.4 GHz Terrestrial Band is currently specific to the US. The US band many not be deployable outside of the US, due to regulatory restrictions concenring the ISM band, outside US.

## 6.4	FCC Specifications for the 2483.5-2495 LTE Band.

## 6.4.3	Special Emissions and Operations Rules to Protect Unlicensed Band

Conventional Part 15 unlicensed operations in the 2400 MHz to 2483.5 MHz band are subject to the conditions that no harmful interference is caused, and that interference must be accepted that may be caused by the operation of an authorized radio station.  The operator of a Part 15 unlicensed radio frequency device is required to cease operating the device upon notification by an FCC representative that the device is causing harmful interference.  Radiated out of band emissions from Part 15 unlicensed operation across 2400 to 2483.5 are limited to 500 microvolts/meter at a measurement distance of 3 meters.

In authorizing the 2.4 GHz Terrestrial Band, FCC regulations were adapted to ensure co-existence between licensed services in the band above 2483.5 MHz and unlicensed services in the band below 2483.5 MHz  To protect the coexistent Wi-Fi and Bluetooth services, special operating rules and conditions were integrated into Part 27 rules.  Summaries of key protective rules are listed below.

- Frequency Limits:  Commercial operations are limited solely to licensed spectrum above 2483.5 MHz, and Part 25 licensed spectrum above 2483.5 MHz may not be bonded with Part 15 unlicensed spectrum below 2483.5 MHz.

- Power Limits:  Power limits are identical to those in the unlicensed band with maximum permitted conducted power of 1 W (30 dBm) and EIRP of 6 dBW (36 dBm).

- Emissions Limits:  For a 10 MHz channel (maximum effective channel width), emissions at must be attenuated by a factor of 40 + 10 log (P) dB (-10 dBm) at 2483.5, by a factor of 43 + 10 log (P) dB (-13 dBm) at 2478.5 MHz, and 55 + 10 log (P) dB at 2473.5 MHz (-25 dBm).

- Network Operating System:  Operations must be under the control of a Network Operating System (NOS).  The NOS will have the ability to resolve interference issues related to commercial operations of the 2.4 GHz licensed service.

## 6.4.4	Frequency Limits and Prohibition Against Bonding and Aggregation below 2483.5 MHz

Frequency limits for the 2.4 GHz band are governed by Title 47 of the CFR, Part 25.149.  Terrestrial operation is limited to the licensed band from 2483.5 MHz to 2495 MHz.  Bonding or aggregation with unlicensed services below 2483.5 MHz is prohibited by Title 47 of the CFR, Parts 15.205, 15.209, 15.247 and 15.249.

This limitation of operational frequency is further clarified by FCC Report and Order 16-181, Paragraph 32.

## 6.4.5	Power Limits for Operations Between 2483.5 MHz and 2495 MHz that Equal Levels for Part 15 Services Below 2483.5 MHz

Power limits for the 2.4 GHz Terrestrial Band are governed by Title 47 of the CFR, Part 25.149(c)(4)(i)-(iv).  These power limits are identical to those found in Part 15 for unlicensed services authorized from 2400 MHz to 2483.5 MHz and they do not distinguish between base station and user equipment.

Power limits for the 2.4 GHz Terrestrial Band were justified in FCC Report and Order 16-181, Paragraph 22.

## 6.4.6	Emissions Limits Below 2483.5 MHz for Unlicensed Band Protection

Emissions limits for the 2.4 GHz Terrestrial Band are governed by Title 47 of the CFR, Part 25.149(c)(4)(v)-(vii).  Figure 6.4.6-1 is a graphical summary of emissions limits for the 2.4 GHz Terrestrial Band, assuming a 10 MHz emission bandwidth.  OOBE limits to protect unlicensed operations below 2483.5 MHz are justified by FCC Report and Order 16-181, Paragraphs 28-30.

- Figure 6.4.6-1:  FCC Mandated Emissions Limits for ISM Protection (RBW=1MHz).

## 6.4.7	Mandatory Deployment of Network Operating System

Special requirements for continuous centralized control of the 2.4 GHz band are governed by Title 47 of the CFR, Part 25.149(g)(2).  These provisions provide a unique mechanism for identification and resolutions of environmental interference issues associated with the 2.4 GHz Terrestrial Band.

## 6.5	Co-Existence Evaluation for 10 MHz LTE Emission within FCC Ratified Band Plan (2483.5 MHz to 2945 MHz)

## 6.5.1	UE Worst Case Emissions to ISM band

For uplink transmissions, the unwanted emissions that leak to the ISM band were simulated using RAN4 minimum RF performance assumptions. The power amplifier was calibrated so that after 4 dB post-PA losses, 22 dBm fully populated QPSK signal is output at the antenna connector, with the emissions profile just meeting 30 dB E-UTRA ACLR requirement. The model is based on measured data from an LTE PA, related to a 23 dBm UE. No filter selectivity has been assumed in order to evaluate the close-in emission margins to the regulatory limits. The carrier frequency is placed at the highest position within the band.

- Figure 6.5.1-1:  Worst Case UE TX Leakage to ISM Band (Global Star plot if the FCC mask).

Editorial Notes:

- 2483.5-2493.5 simulation to be added. Conclude that the higher allocation in the band alleviates slightly the impact upon coexistent access technologies.

- There is a slighter lower RF conducted power level due to the calibration and mismatches concerning the RF front-end.

It can be seen that the FCC emission mask is more relaxed than the LTE SEM, guaranteeing that the FCC limits are met. Also, the general spurious emission limit of -30 dBm/MHz is tighter than the FCC requirement on the ISM band, meaning that UEs have no trouble meeting the limits regardless of whether the general spurious limit applies.

To further improve the coexistence with ISM band services, a conservative emissions case assuming a TDD band filter have been studied. The assumptions for the filter design are as follows:

- Synchronization with Band 41 networks is assumed. This allows a relaxed response on the upper side of the band, and the lower side may be designed with a steep edge. Note that the LTE SEM should be sufficient to

meet the FCC unwanted emission limits above the band, without assuming any filter selectivity.

- Filter passband is 2485-2495 MHz, i.e. the transition band starts already inside the 2.4 GHz Terrestrial Band.

- Two different filter topologies have been considered. Both are based on publicly available data sheets for 2.4 GHz (ISM) and 2.3 and 2.5 GHz (LTE) acoustic wave filters, usually referenced by RAN4.

  - The first filter achieves 11 dB min., 13 dB typical attenuation at 12 MHz offset from the passband edge, with max. 3 dB insertion loss.

  - The second filter achieves 20 dB min., 29 dB typical attenuation at 18.5 MHz offset from the passband edge, with max. 2.5 dB insertion loss.

- Additional insertion loss is added due to RF Front end’s switches and mismatch losses amounting to a total of 4 dB on the RF passband filter.

The emission profiles are shown in Figure 6.5.1-2.

- Figure 6.5.1-2:  Worst Case UE TX Leakage to ISM Band Including Band Filter Minimum Response. The blue curves represent the baseline front end response without any pass band RF filter.

## 6.5.2	UE Typical Emissions to ISM Band

The previous section detailed worst case unwanted emissions at the RAN4 minimum RF performance level, which is not a typical design target for any equipment. The worst case conditions may occur across extreme temperature and operating conditions (e.g. voltage) and manufacturing tolerances, and the UE must still meet the regulatory requirements at these corners.

- Normally, e.g. 3-5 dB margin is targeted for ACLR in typical testing conditions, which indicates e.g. 5-7 dB improvement in the alternate adjacent channel (where 5th order intermodulation dominates the spectrum). This directly reduces the PA and antenna output unwanted emissions on the ISM band by a similar amount, depending on the offset from the channel edge.

- The band filter typical performance at the stopband edge depends heavily on temperature (due to frequency drift) and manufacturing tolerances. Across the stopband, it is normal to see 15-20 dB differences in the minimum vs. typical attenuation. Again, in typical conditions and across UE samples, the ISM band unwanted emissions will be lower than in the simulations shown above.

When fewer than the maximum amount of resource blocks are scheduled for a UE, the transmission bandwidth is narrower, and also the PA spectral regrowth does not reach as far as when transmitting full resource allocation. The emissions vs. resource allocation size was simulated, assuming minimum RF performance and Topology 1 filter (min. response, not typical). The results are shown in Figure 6.5.2-1 for QPSK modulation. In all cases, the transmit output power was set at maximum, i.e. 22 or 23 dBm depending on the allowed MPR.

- Figure 6.5.2-1:  Worst case UE TX leakage to ISM band including band filter minimum response

A conservative emissions case on Wi-Fi channel 11 (integrated over 2452-2472 MHz) spans from below -65 dBm/20MHz to below -40 dBm/20MHz, with only the largest uplink allocations creating emissions reaching towards -35 dBm/20MHz. Regarding emissions to the Bluetooth advertising channel at 2480 MHz, most of the worst case emissions are below -40 to -35 dBm/MHz, and in some rare cases the emissions may reach up to -20 or even -16 dBm/MHz. Modulation indexes 16-QAM and above will in practice show slightly lower emission profile.

Editorial Notes

- The edge PRB cases would require further analysis on how the impact upon WIFI and BT could be mititgated. This desentizes the surrounding STAs and BT clients by degrading the RX SINR of the surrounding STAs and BT clients

LTE also uses UL closed loop power control. In tbe cases when an UE doesn’t operate on cell edge conditions, it may not operate at maximum power. The unwanted emissions of practical UE transmitters would get reduced on average by at least 1:1 in dB scale, when the output power is lowered from maximum. To estimate this effect, a system simulation was performed, using Urban Macro and Urban Micro TD-LTE scenarios, and the cumulative distribution functions of the resulting output power is shown in figure 6.5.2-2. Since not all the assumptions behind the UL Total

TX power, presented below, are not available, this simulation is considered informative.

- Figure 6.5.2-2:  Informative System Simulation of Total UL TX Power CDF in Urban Macro and Micro Scenarios,.

The informative simulation results indicate that 50% of the UE transmissions occur at power levels below about 2-3 dBm in Urban Micro deployment. In indoor or small cell environments, typical transmit powers could be even lower. This presents a significant reduction in the leakage power towards the ISM band, compared to a conservative case figures shown above.

Concerning the typical emissions leaking to the ISM band we can summarize

- Worst case emissions with full UL allocation: -35 dBm/20MHz (Wi-Fi ch#11), -20 dBm/MHz (BT 2480 MHz)

- Effect of PA operation point: -5 to -7 dB (Wi-Fi ch#11), -3 to -5 dB (BT 2480 MHz)

- Effect of filter typical vs. minimum performance: -2 to -5 dB (Wi-Fi ch#11), 0 dB (BT 2480 MHz)

- Effect of typical UL resource allocation (conservative estimates): 0 to -20 dB (Wi-Fi ch#11), 0 to -5 dB (BT 2480 MHz)

- Effect of typical UL TX power (90th and 50th percentiles, conservative 1:1 estimate on emissions): -12 to -20 dB (both Wi-Fi and BT)

- Typical emissions in the order of: -54 to -87 dBm/20MHz (Wi-Fi ch#11), -31 to -50 dBm/MHz (BT 2480 MHz)

Editorial note: these conclussion to further reviewed

### 6.5.3	Base Station Worst Case Emissions to ISM Band

Subject to further study.

## 6.5.4	Base Station Typical Emissions to ISM Band

Subject to further study.

## 6.5.5	Analysis of Environmental Coexistence Impacts of 10 MHz LTE Emission Upon ISM Band Technologies

Subject to further study.

## 6.5.6	Analysis of In-Device Coexistence Impacts of 10 MHz LTE Emission Upon ISM Band Technologies

Subject to further study.

## 6.6 Conclusions

<<< End of TP>>>