3GPP TSG-RAN WG4 Meeting #88	R4-1811767

## Gothenburg, Sweden, 20 - 24 of August, 2018

Source: 	Huawei

Title:	TP to TS 38.141-2: OTA declarations clean-up (4.6)

Agenda Item:	7.9.4.3.5 (Declarations)

Document for:	Approval

# Introduction

In [2] the remaining issues on conducted declarations were proposed to be solved. Furthermore, new declarations related to test configurations generation were identified in [3].

Based on the above conducted declaration decisions, it is proposed to align also the OTA manufacturer declarations in TS 38.141-2 [1].

In the attached TP to TS 38.141-2 [1], modifications from [2] and proposal from [3] were mirrored into the OTA declarations. Additionally, conducted declarations re-numbering was performed.

For all the cross-references across corrections across the whole specification, it is proposed first to fix related part of the TS 38.141-1.

In this contribution, TP to TS 38.141-2 is provided on the OTA declarations clean-up, addressing the remaining issues for the conducted declarations.

# Conclusions

The following proposal is formulated:

Proposal 1: agree on the attached TP to TS 38.141-2 for the OTA NR manufacturer declarations clean-up.

# References

[1] 		Draft TS 38.141-1 v 0.4.0, shared after RAN4 AH-1807 on the RAN4 e-mail reflector

[2]		R4-1810815 	TP to TS 38.141-1: Remaining issues on conducted declarations (4.6)

[3]		R4-1810816 	New declarations related to test configurations generation

# TP to TS 38.141-2, v0.3.0

------------------------------ Modified section ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception"

[3]	3GPP TS 38.141-1: “NR, Base Station (BS) conformance testing, Part 1: Conducted conformance testing”

[4]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[6]	3GPP TR 37.842: "E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[7]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[8]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations"

[9] 		IEC 60 721: "Classification of environmental conditions"

[10]		IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[11]		IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[12]		IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[13]	Recommendation ITU-R M.328: "Spectra and bandwidth of emissions"

[14]	FCC publication number 662911: "Emissions Testing of Transmitters with Multiple Outputs in the Same Band".

[15]	ECC/DEC/(17)06: “The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)”

[16]	3GPP TS 36.104: “E-UTRA; Base Station (BS) radio transmission and reception”

------------------------------ Next modified section ------------------------------

## 4.6	Manufacturer’s declarations

The following BS manufacturer’s declarations listed in table 4.6-1 are required to be provided by the manufacturer for radiated requirements testing for BS type 1-H, BS type 1-O and BS type 2-O.

For the BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3], subclause 4.6.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
|  | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
|  | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
|  | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams, the minimum requirement to declare for conformance are the beams with the highest intend EIRP for each of the beams widths below: 1) Narrowest intended BeWθ, narrowest intended BeWϕ (possible when narrowest intended BeWθ) at the reference beam direction. 2) Narrowest intended BeWϕ, narrowest intended BeWθ (possible when narrowest intended BeWϕ) at the reference beam direction. 3) Widest intended BeWθ, widest intended BeWϕ (possible when widest intended BeWθ at the reference beam direction. 4) Widest intended BeWϕ, widest intended BeWθ (possible when widest intended BeWϕ) at the reference beam direction. 5) BeWθ and BeWϕ which provide highest intended EIRP of all possible beams at the reference beam direction. NOTE 3: Depending on the capability of the system some of these beams may be the same. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. | x | x | x |
|  | Operating band support | NR operating band(s) supported by the BS. Supported bands declared for every beam (). NOTE 4: These operating bands are related to their respective single-band RIBs. | x | x | x |
|  | BS requirements set | Declaration of one of the NR base station requirement’s set as defined for BS type 1-H, BS type 1-O, or BS type 2-O. | c | x | x |
|  | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. | c | x | x |
|  | NR BS channel band width and SCS support | NR BS channel bandwidth and SCS supported. Declared for each beam () and each operating band (). | c | x | x |
|  | Reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (). | x | x | x |
|  | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (). | x | x | x |
|  | Maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (). | x | x | x |
|  | Rated beam EIRP | The rated EIRP level per carrier (PRated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (), as well as the reference beam direction pair (). Declared for every beam (). | x | x | x |
|  | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (). | x | x | x |
|  | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (-) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
|  | Parallel beams | List of beams which have been declared equivalent () and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
|  | Number of carriers at maximum TRP | The number of carriers per operating band the NR BS is capable of generating at maximum TRP declared for every beam identified in D.x. | x | x | x |
|  | Multi-band transceiver units | Declared if an operating band is generated using transceiver units supporting operation in multiple operating bands through common active RF components. | x | x | x |
|  | Operating bands with multi-band dependencies | List operating bands which are generated by multi-band transceiver units. Declared for each operating band for which multi-band transceiver units () have been declared. | c | x | n/a |
|  | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band, declared for each supported operating band (). | c | x | x |
|  | Maximum radiated Base Station RF Bandwidth for contiguous operation | Maximum Base Station RF Bandwidth for contiguous spectrum operation, declared for each supported operating band (). | c | x | x |
|  | Maximum radiated Base Station RF Bandwidth for non- contiguous operation | Maximum Base Station RF Bandwidth for non-contiguous spectrum operation, declared for each supported operating band (). | c | x | x |
|  | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies () | c | x | n/a |
|  | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (). | c | x | x |
|  | Inter-band CA bands | Declared inter-band CA bands supported. | c | x | x |
|  | CA only operation | Declared of CA-only but not multiple carriers operation, declared per operating band (). | c | x | x |
|  | Total number of supported carriers for operating bands with multi-band dependencies | Total number of supported carriers for operating bands declared to have multi-band dependencies (). | c | x | x |
|  | Contiguous or non-contiguous spectrum operation support | Ability of BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. | c | x | x |
|  | Non-contiguous parameters | If non-contiguous operation is supported in operating band () and parameters (e.g. frequency range, maximum Base Station RF Bandwidth, rated total output power, etc.) differ from the contiguous spectrum operation, then this declaration provided parameters for the non-contiguous operation. Otherwise, parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same. | c | x | x |
|  | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
|  | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D.x. NOTE 5: As each identified OSDD has a declared minimum EIS value (), multiple operating band can only be declared if they have the same minimum EIS declaration. | x | x | n/a |
|  | OTA sensitivity supported NR BS channel bandwidth and SCS | The NR BS channel bandwidths and SCS supported by each OSDD. | x | x | n/a |
|  | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
|  | Minimum EIS for FR1 (EISREFSENS) | The minimum EISREFSENS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per NR supported channel BW for the OSDD (D.30). The lowest EIS value for all the declared OSDD’s is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. NOTE 6: If the BS type 1-H or BS type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. | x | x | n/a |
|  | Minimum EIS for FR2 (EISREFSENS_50M) | The EISREFSENS_50M level used for the derivation of the FR2 EISREFSENS for all channel bandwidths supported by BS type 2-O. The declared single integer value of EISREFSENS_50M shall be within the ranges specified in table below: BS class; EISREFSENS_50M range. Wide Area BS; -119 to -96 dBm. Medium Range BS; -114 to -91 dBm. Local Area BS; -109 to -86 dBm. NOTE 14: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that BS has to support 50MHz channel bandwidth. | n/a | n/a | x |
|  | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction () for each OSDD. | x | x | n/a |
|  | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. NOTE 7: Not applicable for BS type 2-O. | x | x | n/a |
|  | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (). NOTE 8: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. | x | x | n/a |
|  | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (). | x | x | n/a |
|  | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
|  | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. NOTE 9: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. | x | x | x |
|  | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (). NOTE 10: The OTA coverage reference direction may be the same as the Reference beam direction pair () but does not have to be. | x | x | x |
|  | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
|  | The rated carrier OTA BS power, PRated,c,TRP | PRated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. | n/a | x | x |
|  | Worst-case side of the BS on which the co-location test antenna is placed | Declare the worst-case side of the BS on which the co-location test antenna is placed and test will be done only on the declared side. | n/a | x | x |
|  | 64QAM power back-off | Additional power back-off declared for 64QAM EVM test requirement for BS type 2-O. NOTE 11: This manufacturer declaration is optional. | n/a | n/a | x |
|  | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | c | x | x |
|  | Geographic area support | The manufacturer shall declare the regions the BS may operate in. e.g. CEPT. | c | x | x |
|  | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. | c | x | x |
|  | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD and/or E-UTRA operating in another operating band. | c | x | n/a |
|  | Single-band RIB or multi-band RIB | List of single-band RIB and/or multi-band RIB resulting from the supported operating bands (), and operating bands with multi-band dependencies (). | c | x | x |
|  | Single or multiple carrier | BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RIB. | x | x | x |
|  | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RIB. | c | x | x |
|  | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. | c | x | x |
|  | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (), which have any impact on the test configuration generation. | c | x | n/a |
|  | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band. Declared per operating band (). | c | x | n/a |
|  | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (). | c | x | x |
|  | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (). | c | x | n/a |
|  | Operating band combination support | List of operating bands combinations supported by BS. Additionally, declares any other limitations under simultaneous operation in the declared band combinations for each multi-band connector which have any impact on the test configuration generation. Declared per multi-band RIB. | c | x | n/a |
|  | Inter-band CA | Declaration of operating band(s) combinations supporting inter-band CA. Declared per operating band combination (). | c | x | x |
|  | Intra-band contiguous CA | Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support. | c | x | x |
|  | Intra-band non-contiguous CA | Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support. | c | x | x |
|  | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | x | x | x |
|  | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (). | x | x | x |
|  | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | x | x | x |
|  | Band n20 support, operating in geographical areas allocated to broadcasting (DTT) | If the BS has single band connector(s) or multi-band connector(s) declared to support Band n20 the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). | c | x | n/a |
|  | Band n20 support, emission level for channel N (PEM,N) | If the BS has sin single band connector(s) or multi-band connector(s) declared to support Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the emission level for channel N (annex G of TS 36.104 [11]) shall be declared. | c | x | n/a |
|  | Band n20 support, Maximum output Power in [10] MHz (P10MHz) | If the BS has single band connector(s) or multi-band connector(s) declared to support Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the maximum output power in [10] MHz (annex G of TS 36.104 [16]) shall be declared. | c | x | n/a |
|  | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. | x | x | x |
|  | Rated beam EIRP at lower end of the fractional bandwidth (EIRPFBWlow) | The rated EIRP level per carrier at lower end of the fractional bandwidth (EIRPFBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (), as well as the reference beam direction pair (). Declared per beam for all supported frequency ranges in D.x. NOTE 12: if D.x is declared for certain frequency range (D.x), there shall be no “Rated beam EIRP” declaration (D.x) for the operating band containing that particular frequency range. | x | x | x |
|  | Rated beam EIRP at higher end of the fractional bandwidth (EIRPFBWhigh) | The rated EIRP level per carrier at higher end of the fractional bandwidth (EIRPFBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (), as well as the reference beam direction pair (). Declared per beam for all supported frequency ranges in D.x. NOTE 13: if D.x is declared for certain frequency range (D.x), there shall be no “Rated beam EIRP” declaration (D.x) for the operating band containing that particular frequency range. | x | x | x |
| NOTE 1: Manufacturer declarations applicable per BS requirement set were marked as “x”. Manufacturer declarations not applicable per BS requirement set were marked as “n/a”. NOTE 2: For BS type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3]. For declarations marked as ‘c’, related conducted declarations in TS 38.141-1 [3] apply. |  |  |  |  |  |

------------------------------ End of modified section ------------------------------