3GPP TSG-RAN WG4 Meeting #88bis	R4-1812051

Chengdu, CN, 8th – 12th Oct 2018

Source: 	Nokia

Title: 					n65 A-MPR study

Agenda Item: 			7.6.10.2

Document for:	Discussion

# 1	Introduction

This contribution presents our results for n65 to protect band 34.

# 2	Discussion

During email discussion on RAN4 reflector prior the meeting it was agreed to simulate 6 scenarios which are presented in Figure 1 and are taken from [1]. After doing the initial analysis we decided to simulate additional two positions to 5 MHz carrier, these positions are the next two lower frequency positions to the one indicated in Figure 1.

Figure 1: Simulated cases

Results in Table 1 and 2 are presented in same format as what was used for NS_05 to keep consistency. Only max A-MPR per modulation is reported and it is total backoff.

Table 1: Parameter

| CBW [MHz] | Offset from band edge [MHz] | Fc [MHz] | A-MPR |
| --- | --- | --- | --- |
| 5 | 15 | 1992,5 | A1 |
| 5 | 10 | 1997,5 | A2 |
| 5 | 5 | 2002,5 | A3 |
| 10 | 20 | 1985 | A4 |
| 10 | 10 | 1995 | A5 |
| 10 | 5 | 2000 | A6 |
| 15 | 15 | 1987,5 | A7 |
| 20 | 10 | 1990 | A8 |

Table 2: A-MPR

|  |  | A1 |  | A2 |  | A3 |  | A4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer |
| SC-FDMA | pi/2-BPSK | MPR | MPR | MPR | 2 | MPR | 8 | MPR | 1,5 |
|  | QPSK | MPR | MPR | MPR | 3,5 | 2 | 9 | MPR | 2,5 |
|  | 16QAM | MPR | MPR | MPR | 4 | 2 | 9,5 | MPR | 3 |
|  | 64QAM | MPR | MPR | MPR | 4 | MPR | 10 | MPR | 3 |
|  | 256QAM | MPR | MPR | MPR | 5 | 10 | 11 | MPR | MPR |
| OFDMA | QPSK | MPR | MPR | MPR | 4,5 | 4 | 11 | MPR | 4,5 |
|  | 16QAM | MPR | MPR | MPR | 4,5 | 3,5 | 11 | MPR | 4,5 |
|  | 64QAM | MPR | MPR | MPR | 5 | 4 | 10,5 | MPR | 4,5 |
|  | 256QAM | MPR | MPR | MPR | MPR | 10 | 11,5 | MPR | MPR |
|  |  | A5 |  | A6 |  | A7 |  | A8 |  |
|  |  | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer |
| SC-FDMA | pi/2-BPSK | MPR | 7 | 13 | 14,5 | MPR | 7 | 13 | 13 |
|  | QPSK | 1,5 | 8,5 | 13,5 | 15 | 1 | 8 | 14 | 14 |
|  | 16QAM | 2 | 9 | 15 | 15,5 | 1,5 | 8,5 | 15 | 14,5 |
|  | 64QAM | MPR | 9 | 15,5 | 15,5 | MPR | 8,5 | 15 | 14,5 |
|  | 256QAM | 5,5 | 9,5 | 16 | 16 | MPR | 9 | 16 | 15 |
| OFDMA | QPSK | 3,5 | 10,5 | 18 | 18 | 3,5 | 10 | 18 | 17 |
|  | 16QAM | 3,5 | 10 | 18 | 17,5 | 3,5 | 10 | 18 | 17,5 |
|  | 64QAM | MPR | 10,5 | 17,5 | 17,5 | MPR | 10 | 18,5 | 17,5 |
|  | 256QAM | MPR | 10,5 | 18,5 | 18 | MPR | 10 | 18,5 | 17,5 |

# 3	Conclusion

As a conclusion we can note that

- 5 MHz carrier benefits clearly when Fc is lowered (A2, A3) from 2002,5 MHz which was the only position in [1]

- Similarly 10 MHz carrier benefits when Fc is lowered (A5, A6) from 2000 MHz (A6).

- 15 MHz was only simulated in one position 1987,5 MHz A7 and has some inner allocations that do not require A-MPR

- 20 MHz was only simulated in one position 1990 MHz A8 and it requires substantial A-MPR

# 4	References

[1] R4-158247, Band 65 A-MPR Tables for Band 34 protection, Nokia Networks, RAN4#77