3GPP TSG-RAN WG4 Meeting #88bis                                                                     R4-1812123

# Chengdu, China, 8th – 12th October, 2018

Agenda Item:	7.11.7.1

Source:	Intel Corporation

Title:	Discussion about remaining issue about NR RLM

Document for:	Discussion

# Introduction

In this contribution we will continue to discuss RLM remaining issues.

# Evaluation period for CSI-RS RLM

The evaluation time for INS and OOS are defined for CSI-RS with density=3. In last meeting, we provide our simulation results and found that 10 samples and 20 samples for INS and OOS may not be sufficient for 24RB with D=3. From our simulation results, the OOS SNR point is -10dB after power boosting for NR RLM. Then the SNR3 and SNR2 should be -13dB and -7dB respectively. The SNR measurement accuracy should be accurate enough to distinguish SNR2 and SNR3.

From our simulation results, the SNR estimation error is 3.7dB and 1dB for SNR=-13dB and SNR=-7dB level by using 20 samples. The total SNR estimation error is around 3.7+1=4.7dB. When 2dB BB margin is considered, the total error is 6.7dB which is larger than SNR gap of 6dB. Therefore, the evaluation time is preferred to extend to 20/40 samples for INS and OOS respectively for CSI-RS with D=3. Another option is that if the evaluation time is not extended, 10/20 samples are only applied to 96RB case.

Proposal 1: For CSI-RS RLM with D=3, evaluation time of 10/20 samples are extended to 20/40 samples for INS and OOS respectively, or 10/20 samples are only applied to 96RB case.

For CSI-RS with D=1, since the reference signal density is very low, the measurement accuracy based on the reference signal will degrade a lot compared with that of D=3. We also provide simulation results in[].

Suppose both 40 samples are used for the SNR=-13dB and -7dB. For ETU case, the estimation error is 2.6dB and 1.9dB for SNR=-13dB and -7dB. Then the total error is 2.6+1.9=4.5dB. When 2dB BB margin is considered, the total error is 6.5dB which larger than SNR gap of 6dB. Therefore, it can’t achieve good evaluation performance when D=1 is used.

Observation 1: For CSI-RS RLM with D=1 of 96RB with extended evaluation time with 40 samples, the measurement performance is not good for ETU channel.

Proposal 2: For CSI-RS RLM with D=1, don’t define RLM evaluation time.

# Rx beam sweeping factor of CSI-RS for FR2

In last meeting, the Rx beam sweeping factor for SSB is defined to be 8. However, the Rx beam sweeping factor of CSI-RS for FR2 is not decide yet.

For Rx beam sweeping number in CSI-RS RLM, maxNumberRxBeam is one of the option. However, it is defined for beam management based on CSI-RS. It’s not clear whether it can be used for RLM or not. It needs RAN1 and RAN2 to clarify.

Proposal 3: maxNumberRxBeam can be used for rx beam sweeping factor of CSI-RS RLM.

# CORESET selection

If UE shall perform RLM when CSI-RS is not QCL-ed with any CORESET, which CORESET is used as reference, there are two options:

Option 1: Use the CORESET with lowest index among the QCL’ed CORESETs.

Option 2: UE is not expected to perform RLM based on this CSI-RS based RLM-RS.

We think option 2 is more preferred.

Proposal 4: When CSI-RS is not QCL-ed with any CORESET, UE is not expected to perform RLM based on this CSI-RS based RLM-RS.

# Second BLER pair

In current specification, only BLER pair of configuration 0 is defined. There is a LS from RAN1[2] to ask RAN4 to define the second BLER pair for VoIP service.

| Configuration | BLERout | BLERin |
| --- | --- | --- |
| 0 | 10% | 2% |
| 1 | TBD | TBD |

According to the quality of VoIP service, the BLERout and BLERin can be 35% and 10%. The PDCCH configuration parameter needs to be re-designed to make sure the SNR margin is large enough to distinguish configuration 0 and configuration 1.

Proposal 5: For second BLER pair, BLERout and BLERin can be 35% and 10% respectively.

# Conclusion

In this contribution the remaining issue about RLM are discussed. The following conclusion can be drawn:

Proposal 1: For CSI-RS RLM with D=3, evaluation time of 10/20 samples are extended to 20/40 samples for INS and OOS respectively, or 10/20 samples are only applied to 96RB case.

Proposal 2: For CSI-RS RLM with D=1, don’t define RLM evaluation time.

Proposal 3: maxNumberRxBeam can be used for rx beam sweeping factor of CSI-RS RLM.

Proposal 4: When CSI-RS is not QCL-ed with any CORESET, UE is not expected to perform RLM based on this CSI-RS based RLM-RS.

Proposal 5: For second BLER pair, BLERout and BLERin can be 35% and 10% respectively.

# References

[1] R4-1809880,” Discussion about evaluation time for CSI-RS based NR RLM”, Intel.

[2] R1-1809853,” LS on second BLER pair for RLM”.