3GPP TSG-RAN4 Meeting #88bis 	R4-1812142

# Chengdu, China, 8 – 12 Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on test cases for inter-frequency measurement in EN-DC with PScell in FR1 (section A.4.6.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test cases for inter-frequency measurement in EN-DC with PScell in FR1 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases for inter-frequency measurement in EN-DC with PScell in FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for inter-frequency measurement in EN-DC with PScell in FR1 are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### A.4.6.2	Inter-frequency Measurements

### A.4.6.2.1 	EN-DC event triggered reporting tests without SSB time index detection when DRX is not used

### A.4.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

The test parameters are given in Tables A.4.6.2.1.1-1 and A.4.6.2.1.1-2. In this test, there are three cells: LTE TDD cell 1 as PCell on E-UTRA RF channel 1, NR TDD cell 2 as PSCell in FR1 on NR RF channel 1 and NR TDD cell 3 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.4.6.2.1.1-1 is provided for UE who doesn’t support per-FR gap and in test 2 measurement gap pattern configuration #2 as defined in Table A.4.6.2.1.1-1 is provided for UE who supports per-FR gap. Test 1 is for UE who doesn’t support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR TDD cell 3.

The configuration of LTE TDD cell 1 is defined in table A.3.7.2.1-1.

Table A.4.6.2.1.1-1: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | 1, 2 |  | Two NR TDD carrier frequencies is used. NR RF channel number 1 is in FR1 and NR RF channel number 2 is in FR2. |
| Channel Bandwidth of LTE Cell 1 | MHz | 10 |  |  |
| Channel Bandwidth of NR Cell 2 | MHz | 40 |  |  |
| Channel Bandwidth of NR Cell 3 | MHz | 100 |  |  |
| Active cell |  | LTE TDD Cell 1 (PCell) and NR TDD cell 2 (PScell) |  | LTE TDD Cell 1 is on E-UTRA RF channel number 1. NR TDD Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | NR TDD cell 3 |  | NR TDD cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | 0 | 2 | As specified in TS 38.133 clause 9.1.2-1. |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | 3 s |  | Synchronous EN-DC |
| Time offset between Pcell/PScell and neighbour cell |  | 1.5 s |  | Synchronous cells to both PCell and PScell |
| T1 | s | 5 |  |  |
| T2 | s | TBD | TBD |  |

Table A.4.6.2.1.1-2: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 40 |  | 100 |  |
| Frequency range |  | 1 |  | 2 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | OP.1 |  | OP.1 |  |
| PDSCH parameters defined in A.3.1.1.2 |  | SR.2.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters defined in A.3.1.2.2 |  | CR.2.1 TDD |  | CR.3.1 TDD |  |
| SMTC configuration defined in A.3.2.1.1 and A.3.2.1.2 |  | SMTC pattern 2 in FR1 |  | SMTC pattern 1 in FR2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Note 3 | dBm/SCS | -98 |  |  |  |
| SS-RSRP Note 4 | dBm/SCS | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

### A.4.6.2.1.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.4.6.2.2 	EN-DC event triggered reporting tests without SSB time index detection when DRX is used

### A.4.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

The test parameters are given in Tables A.4.6.2.2.1-1 and A.4.6.2.2.1-2. In this test, there are three cells: LTE TDD cell 1 as PCell on E-UTRA RF channel 1, NR TDD cell 2 as PSCell in FR1 on NR RF channel 1 and NR TDD cell 3 as neighbour cell in FR2 on NR RF channel 2.

In test 1, measurement gap pattern configuration # 0 as defined in Table A.4.6.2.2.1-1 is provided for UE who doesn’t support per-FR gap; and in test 2, measurement gap pattern configuration #2 as defined in Table A.4.6.2.2.1-1 is provided for UE who supports per-FR gap. Test 1 is for UE who doesn’t support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR TDD cell 3.

The configuration of LTE TDD cell 1 is defined in table A.3.7.2.1-1.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.4.6.2.2.1-1: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | 1, 2 |  | Two NR TDD carrier frequencies is used. NR RF channel number 1 is in FR1 and NR RF channel number 2 is in FR2. |
| Channel Bandwidth of LTE Cell 1 | MHz | 10 |  |  |
| Channel Bandwidth of NR Cell 2 | MHz | 40 |  |  |
| Channel Bandwidth of NR Cell 3 | MHz | 100 |  |  |
| Active cell |  | LTE TDD Cell 1 (PCell) and NR TDD cell 2 (PScell) |  | LTE TDD Cell 1 is on E-UTRA RF channel number 1. NR TDD Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | NR TDD cell 3 |  | NR TDD cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | 0 | 2 | As specified in TS 38.133 clause 9.1.2-1. |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX on PCell |  | OFF |  | DRX is not used on PCell |
| DRX on PSCell |  | ON |  | DRX configuration is defined in A.4.6.2.2.1-3 |
| Time offset between PCell and PSCell |  | 3 s |  | Synchronous EN-DC |
| Time offset between Pcell/PScell and neighbour cell |  | 1.5 s |  | Synchronous cells to both PCell and PScell |
| T1 | s | 5 |  |  |
| T2 | s | TBD | TBD |  |

Table A.4.6.2.2.1-2: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 40 |  | 100 |  |
| Frequency range |  | 1 |  | 2 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | OP.1 |  | OP.1 |  |
| PDSCH parameters defined in A.3.1.1.2 |  | SR.2.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters defined in A.3.1.2.2 |  | CR.2.1 TDD |  | CR.3.1 TDD |  |
| SMTC configuration defined in A.3.2.1.1 and A.3.2.1.2 |  | SMTC pattern 2 in FR1 |  | SMTC pattern 1 in FR2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Note 3 | dBm/SCS | -98 |  |  |  |
| SS-RSRP Note 4 | dBm/SCS | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.4.6.2.2.1-3: DRX-Configuration for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| drx-onDurationTimer | ms1 | As specified in clause 6.3.2 in TS 38.331 |
| drx-InactivityTimer | ms1 |  |
| drx-RetransmissionTimerDL | sl1 |  |
| drx-RetransmissionTimerUL | sl1 |  |
| drx-LongCycleStartOffset | ms40 |  |
| shortDRX | disable |  |

Table A.4.6.2.2.1-4: TimeAlignmentTimer -Configuration EN-DC inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 |
| sr-ConfigIndex | TBD |  |

### A.4.6.2.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2, UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.4.6.2.3 	EN-DC event triggered reporting tests with SSB time index detection when DRX is not used

### A.4.6.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

The test parameters are given in Tables A.4.6.2.3.1-1 and A.4.6.2.3.1-2. In this test, there are three cells: LTE TDD cell 1 as PCell on E-UTRA RF channel 1, NR TDD cell 2 as PSCell in FR1 on NR RF channel 1 and NR TDD cell 3 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.4.6.2.3.1-1 is provided for UE who doesn’t support per-FR gap and in test 2 measurement gap pattern configuration #2 as defined in Table A.4.6.2.3.1-1 is provided for UE who supports per-FR gap. Test 1 is for UE who doesn’t support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR TDD cell 3.

The configuration of LTE TDD cell 1 is defined in table A.3.7.2.1-1.

Table A.4.6.2.3.1-1: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | 1, 2 |  | Two NR TDD carrier frequencies is used. NR RF channel number 1 is in FR1 and NR RF channel number 2 is in FR2. |
| Channel Bandwidth of LTE Cell 1 | MHz | 10 |  |  |
| Channel Bandwidth of NR Cell 2 | MHz | 40 |  |  |
| Channel Bandwidth of NR Cell 3 | MHz | 100 |  |  |
| Active cell |  | LTE TDD Cell 1 (PCell) and NR TDD cell 2 (PScell) |  | LTE TDD Cell 1 is on E-UTRA RF channel number 1. NR TDD Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | NR TDD cell 3 |  | NR TDD cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | 0 | 2 | As specified in TS 38.133 clause 9.1.2-1. |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | 3 s |  | Synchronous EN-DC |
| Time offset between Pcell/PScell and neighbour cell |  | 1.5 s |  | Synchronous cells to both PCell and PScell |
| T1 | s | 5 |  |  |
| T2 | s | TBD | TBD |  |

Table A.4.6.2.3.1-2: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 40 |  | 100 |  |
| Frequency range |  | 1 |  | 2 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | OP.1 |  | OP.1 |  |
| PDSCH parameters defined in A.3.1.1.2 |  | SR.2.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters defined in A.3.1.2.2 |  | CR.2.1 TDD |  | CR.3.1 TDD |  |
| SMTC configuration defined in A.3.2.1.1 and A.3.2.1.2 |  | SMTC pattern 2 in FR1 |  | SMTC pattern 1 in FR2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Note 3 | dBm/SCS | -98 |  |  |  |
| SS-RSRP Note 4 | dBm/SCS | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

### A.4.6.2.3.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index together with event triggered reporting.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.4.6.2.4 	EN-DC event triggered reporting tests with SSB time index detection when DRX is used

### A.4.6.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

The test parameters are given in Tables A.4.6.2.4.1-1 and A.4.6.2.4.1-2. In this test, there are three cells: LTE TDD cell 1 as PCell on E-UTRA RF channel 1, NR TDD cell 2 as PSCell in FR1 on NR RF channel 1 and NR TDD cell 3 as neighbour cell in FR2 on NR RF channel 2.

In test 1, measurement gap pattern configuration # 0 as defined in Table A.4.6.2.4.1-1 is provided for UE who doesn’t support per-FR gap; and in test 2, measurement gap pattern configuration #2 as defined in Table A.4.6.2.4.1-1 is provided for UE who supports per-FR gap. Test 1 is for UE who doesn’t support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR TDD cell 3.

The configuration of LTE TDD cell 1 is defined in table A.3.7.2.1-1.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.4.6.2.4.1-1: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | 1, 2 |  | Two NR TDD carrier frequencies is used. NR RF channel number 1 is in FR1 and NR RF channel number 2 is in FR2. |
| Channel Bandwidth of LTE Cell 1 | MHz | 10 |  |  |
| Channel Bandwidth of NR Cell 2 | MHz | 40 |  |  |
| Channel Bandwidth of NR Cell 3 | MHz | 100 |  |  |
| Active cell |  | LTE TDD Cell 1 (PCell) and NR TDD cell 2 (PScell) |  | LTE TDD Cell 1 is on E-UTRA RF channel number 1. NR TDD Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | NR TDD cell 3 |  | NR TDD cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | 0 | 2 | As specified in TS 38.133 clause 9.1.2-1. |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX on PCell |  | OFF |  | DRX is not used on PCell |
| DRX on PSCell |  | ON |  | DRX configuration is defined in A.4.6.2.4.1-3 |
| Time offset between PCell and PSCell |  | 3 s |  | Synchronous EN-DC |
| Time offset between Pcell/PScell and neighbour cell |  | 1.5 s |  | Synchronous cells to both PCell and PScell |
| T1 | s | 5 |  |  |
| T2 | s | TBD | TBD |  |

Table A.4.6.2.4.1-2: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 40 |  | 100 |  |
| Frequency range |  | 1 |  | 2 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | OP.1 |  | OP.1 |  |
| PDSCH parameters defined in A.3.1.1.2 |  | SR.2.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters defined in A.3.1.2.2 |  | CR.2.1 TDD |  | CR.3.1 TDD |  |
| SMTC configuration defined in A.3.2.1.1 and A.3.2.1.2 |  | SMTC pattern 2 in FR1 |  | SMTC pattern 1 in FR2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Note 3 | dBm/SCS | -98 |  |  |  |
| SS-RSRP Note 4 | dBm/SCS | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.4.6.2.4.1-3: DRX-Configuration for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| drx-onDurationTimer | ms1 | As specified in clause 6.3.2 in TS 38.331 |
| drx-InactivityTimer | ms1 |  |
| drx-RetransmissionTimerDL | sl1 |  |
| drx-RetransmissionTimerUL | sl1 |  |
| drx-LongCycleStartOffset | ms40 |  |
| shortDRX | disable |  |

Table A.4.6.2.4.1-4: TimeAlignmentTimer -Configuration EN-DC inter-frequency event triggered reporting with SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 |
| sr-ConfigIndex | TBD |  |

### A.4.6.2.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [TBD] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2, UE is required to report SSB time index together with event triggered reporting.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

End of Change 1