3GPP TSG-RAN WG4 Meeting #88bis	R4-1812174

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	7.12.6.11

Source: 		Intel Corporation

Title: 			On Candidate Beam Detection Requirements

Document for:	Discussion

# Introduction

In RAN4#88 candidate beam detection requirements were discussed in RAN4#88 and open issues are still to be resolved. In this paper we present our views on the open issues related to candidate beam detection.

# Discussion

The open issues related to candidate beam detection (CBD) are:

- Whether to define the same L1-RSRP measurement requirements for both candidate beam detection and beam reporting

- L1 averaging requirements for candidate beam detection

- The scaling factor N of L1 averaging period due to Rx beam sweeping in FR2

## L1-RSRP requirements for CBD

In RAN4#88 the L1-RSRP measurements requirements for CBD were discussed and the options were as follows

- Option 1: L1-RSRP measurement requirements for reporting beam reporting and candidate beam detection share the same requirements based on multiple samples.

- Option 2: Separate L1-RSRP measurement requirements will be introduced for candidate beam detection and reporting.

The L1-RSRP for candidate beam detection shall be based on multiple samples. In our paper [1] we propose that L1-RSRP for beam reporting be based on single shot measurement. Given that L1-RSRP for both CBD and beam reporting might not be based on multiple samples, we propose to introduce different requirements for CBD and beam reporting.

Proposal#1: Introduce separate requirements for L1-RSRP for candidate beam detection and beam reporting

## L1 averaging requirements for CBD

In RAN4#88 the averaging requirements for CBD were discussed. CBD can be based on SSB and/or CSI-RS.

For SSB based CBD, define the L1 averaging requirements as 3 samples

For CSI-RS based CBD, with CSI-RS configured with D=3, the number of samples for L1-averaging shall be defined as 3.

Proposal #2: For SSB based CBD define L1 averaging requirements as 3 samples

Proposal #3 For CSI-RS based BFD define L1 averaging requirements as 3 samples with CSI-RS configuration D=3

## Evaluation period in FR2 for CBD

In FR2 the evaluation period for CBD shall be scaled by N to account for Rx beam sweeping. This was discussed in RAN4#88 but no consensus was reached.

For the scale factor to evaluation period we propose the following:

For SSB based CBD, define

N=1, when Rx beam for BFD resource is known to UE

N=8, otherwise

For CSI-RS based CBD, define:

N=1, when Rx beam for BFD resource is known to UE

N=MaxNumberRxBeam, otherwise

The RRC parameter maxNumberRxBeam is used to indicate to the network how many repetitions of CSI-RS are needed to perform beam refinement on UE side. The use of MaxNumberRxBeam for defining the evaluation period for CSI-RS based CBD shall be confirmed by RAN2. In case MaxNumberRxBeam is not signalled, N=8 shall be used.

Proposal #4: For evaluation period in FR2 for SSB based candidate beam detection, define

N=1, when Rx beam for BFD resource is known to UE

N=8, otherwise

Proposal #5: For evaluation period in FR2 for CSI-RS based candidate beam detection, define

N=1, when Rx beam for BFD resource is known to UE

N= MaxNumberRxBeam or 8, otherwise

Proposal #6: The use of MaxNumberRxBeam for defining evaluation period for CSI-RS based candidate beam detection shall be confirmed by RAN2

# Conclusions

In this paper we present our views on requirements for candidate beam detection and have the following proposals:

Proposal#1: Introduce separate requirements for L1-RSRP for candidate beam detection and beam reporting

Proposal #2: For SSB based CBD define L1 averaging requirements as 3 samples

Proposal #3 For CSI-RS based BFD define L1 averaging requirements as 3 samples with CSI-RS configuration D=3

Proposal #4: For evaluation period in FR2 for SSB based candidate beam detection, define

N=1, when Rx beam for BFD resource is known to UE

N=8, otherwise

Proposal #5: For evaluation period in FR2 for CSI-RS based candidate beam detection, define

N=1, when Rx beam for BFD resource is known to UE

N= MaxNumberRxBeam or 8, otherwise

Proposal #6: The use of MaxNumberRxBeam for defining evaluation period for CSI-RS based candidate beam detection shall be confirmed by RAN2

# Reference

- R4-1812126, “Discussion about L1-RSRP measurement accuracy for beam reporting”, Intel Corporation