3GPP TSG-RAN4 Meeting #88bis	R4-1812256

# Chengdu, China, 8th Oct – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for interruptions at transitions in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-6 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for interruptions at transitions in EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following test cases: 1. E-UTRAN FDD – NR FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC with 15KHz SCS 2. E-UTRAN TDD – NR TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC with 15KHz SCS 3. E-UTRAN TDD – NR TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC with 30KHz SCS 4. E-UTRAN FDD – NR FDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC with 30KHz SCS 5. E-UTRAN TDD – NR TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC with 120KHz SCS 6. E-UTRAN FDD – NR TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC with 120KHz SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for interruptions at transitions in EN-DC will be missed in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.4.5.2.1.1, A.4.5.2.1.2, A.4.5.2.1.3, A.4.5.2.1.4, A.5.5.2.1.1 and A.5.5.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.4.5.2.1.1	E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC with 15KHz SCS

A.4.5.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.1.1-1 and A.4.5.2.1.1.1-2. The E-UTRAN PCell DRX configuration parameters are given in Table A.4.5.2.1.1.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the FDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR1 FDD secondary component (RF channel 2) with 15KHz SCS. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell has already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. CORESET indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.4.5.2.1.1.1-1: General test parameters for E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A.4.5.2.1.1.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.1.1-2: NR cell specific test parameters for E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Cell2 |
| --- | --- | --- |
|  |  | T1 |
| RF Channel Number |  | 2 |
| Duplex mode |  | FDD |
| BWchannel | MHz | 10MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.1.1 FDD |
| CORESET parametersNote1: |  | CR.1.1 FDD |
| OCNG PatternsNote1 |  | OP.1 |
| SMTC configuration |  | SMTC.1 FR1 |
| Note 2 | dBm/15 KHz | -104 |
|  | dB | 17 |
| Note 3 | dB | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| Time offset to cell1 Note 4 | s | 33 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |

Table A.4.5.2.1.1.1-3: E-UTRAN PCell DRX-Configuration for E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell1 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf160 |  |
| shortDRX | disable |  |
| Note 1: UE is continuously scheduled in NR PSCell |  |  |

A.4.5.2.1.1.2	Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR PSCell shall not exceed 0.5ms (1 slot) as defined in TS38.133 section 8. 2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.2.1.2	E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC with 15KHz SCS

A.4.5.2.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.2.1-1 and A.4.5.2.1.2.1-2. The E-UTRAN PCell DRX configuration parameters are given in Table A.4.5.2.1.2.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the TDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR1 TDD secondary component (RF channel 2) with 15KHz SCS. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell has already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.4.5.2.1.2.1-1: General test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A.4.5.2.1.3.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.2.1-2: NR cell specific test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Cell2 |
| --- | --- | --- |
|  |  | T1 |
| RF Channel Number |  | 2 |
| Duplex mode |  | TDD |
| BWchannel | MHz | 10MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.1.1 TDD |
| CORESET parametersNote1: |  | CR.1.1 TDD |
| OCNG PatternsNote1 |  | OP.1 |
| SMTC configuration |  | SMTC.1 FR1 |
| Note 2 | dBm/15 KHz | -104 |
|  | dB | 17 |
| Note 3 | dB | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| Time offset to cell1 Note 4 | s | 500 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |

Table A.4.5.2.1.2.1-3: E-UTRAN PCell DRX-Configuration for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Field | Cell1 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf160 |  |
| shortDRX | disable |  |
| Note 1: UE is continuously scheduled in NR PSCell |  |  |

A.4.5.2.1.2.2	Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR PSCell shall not exceed 1ms (2 slots) as defined in TS38.133 section 8. 2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.2.1.3	E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC with 30KHz SCS

A.4.5.2.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.3.1-1 and A.4.5.2.1.3.1-2. The E-UTRAN PCell DRX configuration parameters are given in Table A.4.5.2.1.3.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the TDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR1 TDD secondary component (RF channel 2) with 30KHz SCS. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell has already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.4.5.2.1.3.1-1: General test parameters for E-UTRAN TDD – NR FR1TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A.4.5.2.1.3.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.3.1-2: NR cell specific test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Cell2 |
| --- | --- | --- |
|  |  | T1 |
| RF Channel Number |  | 2 |
| Duplex mode |  | TDD |
| BWchannel | MHz | 40MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 TDD |
| CORESET parametersNote1: |  | CR.2.1 TDD |
| OCNG PatternsNote1 |  | OP.1 |
| SMTC configuration |  | SMTC.2 FR1 |
| Note 2 | dBm/15 KHz | -104 |
|  | dB | 17 |
| Note 3 | dB | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| Time offset to cell1 Note 4 | s | 33 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |

Table A.4.5.2.1.3.1-3: E-UTRAN PCell DRX-Configuration for E-UTRAN TDD- NR FR1 TDD interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell1 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf160 |  |
| shortDRX | disable |  |
| Note 1: UE is continuously scheduled in NR PSCell |  |  |

A.4.5.2.1.3.2		Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR PSCell shall not exceed 0.5ms (1 slot) as defined in TS38.133 section 8. 2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.2.1.4	E-UTRAN FDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC with 30KHz SCS

A.4.5.2.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.4.1-1 and A.4.5.2.1.4.1-2. The E-UTRAN PCell DRX configuration parameters are given in Table A.4.5.2.1.4.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the FDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR1 TDD secondary component (RF channel 2) with 30KHz SCS. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell has already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.4.5.2.1.4.1-1: General test parameters for E-UTRAN FDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A.4.5.2.1.4.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.4.1-2: NR cell specific test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Cell2 |
| --- | --- | --- |
|  |  | T1 |
| RF Channel Number |  | 2 |
| Duplex mode |  | TDD |
| BWchannel | MHz | 40MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 TDD |
| CORESET parametersNote1: |  | CR.2.1 TDD |
| OCNG PatternsNote1 |  | OP.1 |
| SMTC configuration |  | SMTC.2 FR1 |
| Note 2 | dBm/15 KHz | -104 |
|  | dB | 17 |
| Note 3 | dB | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| Time offset to cell1 Note 4 | s | 250 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |

Table A.4.5.2.1.4.1-3: E-UTRAN PCell DRX-Configuration for E-UTRAN FDD- NR FR1 TDD interruption at transitions between active and non-active during DRX in asynchronous DC

| Field | Cell1 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf160 |  |
| shortDRX | disable |  |
| Note 1: UE is continuously scheduled in NR PSCell |  |  |

A.4.5.2.1.4.2	Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR PSCell shall not exceed 1ms (2 slots) as defined in TS38.133 section 8. 2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Change #1>>

<< Start of Change #2>>

A.5.5.2.1.1	E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC with 120KHz SCS

A.5.5.2.1.1.1		Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.5.5.2.1.1.1-1 and A.5.5.2.1.1.1-2. The E-UTRAN PCell DRX configuration parameters are given in Table A.5.5.2.1.1.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-2. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the TDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR2 TDD secondary component (RF channel 2) with 120KHz SCS. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell have already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.5.5.2.1.1.1-1: General test parameters for E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| AoA number |  | 1 | Applicable to cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A.5.5.2.1.1.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |

Table A.5.5.2.1.1.1-2: NR cell specific test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Cell2 |
| --- | --- | --- |
|  |  | T1 |
| RF Channel Number |  | 2 |
| Duplex mode |  | TDD |
| BWchannel | MHz | 100MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.3.1 TDD |
| CORESET parametersNote1: |  | CR.3.1 TDD |
| OCNG PatternsNote1 |  | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 |
| Note 2 | dBm/15 KHz | -104 |
|  | dB | 17 |
| Note 3 | dB | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| Time offset to cell1 Note 4 | s | 33 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |

Table A.5.5.2.1.1.1-3: E-UTRAN PCell DRX-Configuration for E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Field | Cell1 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf160 |  |
| shortDRX | disable |  |
| Note 1: UE is continuously scheduled in NR PSCell |  |  |

A.5.5.2.1.1.2		Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR PSCell shall not exceed 0.625ms (5 slots) as defined in TS38.133 section 8. 2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.2.1.2	E-UTRAN FDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC with 120KHz SCS

A.5.5.2.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.5.5.2.1.2.1-1 and A.5.5.2.1.2.1-2. The E-UTRAN PCell DRX configuration parameters are given in Table A.5.5.2.1.2.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-2. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the FDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR2 TDD secondary component (RF channel 2) with 120KHz SCS. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell have already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.5.5.2.1.2.1-1: General test parameters for E-UTRAN FDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| AoA number |  | 1 | Applicable to cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A.5.5.2.1.2.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |

Table A.5.5.2.1.2.1-2: NR cell specific test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Cell2 |
| --- | --- | --- |
|  |  | T1 |
| RF Channel Number |  | 2 |
| Duplex mode |  | TDD |
| BWchannel | MHz | 100MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.3.1 TDD |
| CORESET parametersNote1: |  | CR.3.1 TDD |
| OCNG PatternsNote1 |  | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 |
| Note 2 | dBm/15 KHz | -104 |
|  | dB | 17 |
| Note 3 | dB | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| Time offset to cell1 Note 4 | s | 33 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |

Table A.5.5.2.1.2.1-3: E-UTRAN PCell DRX-Configuration for E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Field | Cell1 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf160 |  |
| shortDRX | disable |  |
| Note 1: UE is continuously scheduled in NR PSCell |  |  |

A.5.5.2.1.2.2		Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR PSCell shall not exceed 0.625ms (5 slots) as defined in TS38.133 section 8. 2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Change #2>>