3GPP TSG-RAN4 Meeting #88bis	R4-1812257

# Chengdu, China, 8th Oct – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for EN-DC interruptions due to deactivated SCell operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-6 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for EN-DC interruptions due to deactivated SCell operations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following test cases: 1. E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 15KHz SCS 2. E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 15KHz SCS 3. E-UTRAN TDD – NR TDD interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC with 30KHz SCS 4. E-UTRAN FDD – NR TDD interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC with 30KHz SCS 5. E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 120KHz SCS 6. E-UTRAN FDD – NR TDD interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC with 120KHz SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for EN-DC interruptions due to deactivated SCell operations will be missed in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.4.5.2.1.5, A.4.5.2.1.6, A.4.5.2.1.7, A.4.5.2.1.8, A.5.5.2.1.3 and A.5.5.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.4.5.2.1.5	E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 15KHz SCS

A.4.5.2.1.5.1	Test Purpose and Environment

The purpose of this test is to verify NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.5.1-1 and A.4.5.2.1.5.1-2 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1 Cell2 and Cell3. Cell1 is LTE PCell on the TDD primary component, Cell2 and Cell 3 is NR PSCell and NR deactivated SCell on the FR1 TDD secondary component. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. The point in time at which the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector, defines the start of time period T1. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.4.5.2.1.5.1-1: General test parameters for E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 15KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Active PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to cell1, cell 2 and cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.5.1-2: NR cell specific test parameters for E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 15KHz SCS

| Parameter | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 2 | 3 |
| Duplex mode |  | TDD | TDD |
| BWchannel | MHz | 10MHz | 10MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.1.1 TDD | - |
| CORESET parametersNote1: |  | CR.1.1 TDD | CR.1.1 TDD |
| OCNG PatternsNote1 |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR1 | SMTC.1 FR1 |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 17 | 17 |
| Note 3 | dB | 17 | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | 33 | 33 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

A.4.5.2.1.5.2	Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell. Each interruption on NR PSCell shall not exceed 1ms (1 slot) as defined in TS38.133 section 8.2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.2.1.6	E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 15KHz SCS

A.4.5.2.1.6.1	Test Purpose and Environment

The purpose of this test is to verify NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.6.1-1 and A.4.5.2.1.6.1-2 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1 Cell2 and Cell3. Cell1 is LTE PCell on the FDD primary component, Cell2 and Cell 3 is NR PSCell and NR deactivated SCell on the FR1 TDD secondary component. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. The point in time at which the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector, defines the start of time period T1. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.4.5.2.1.6.1-1: General test parameters for E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 15KHz SCS S

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to cell1, cell 2 and cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.6.1-2: NR cell specific test parameters for E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 15KHz SCS

| Parameter | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 2 | 3 |
| Duplex mode |  | TDD | TDD |
| BWchannel | MHz | 10MHz | 10MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.1.1 TDD | - |
| CORESET parametersNote1: |  | CR.1.1 TDD | CR.1.1 TDD |
| OCNG PatternsNote1 |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR1 | SMTC.1 FR1 |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 17 | 17 |
| Note 3 | dB | 17 | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | 500 | 500 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

A.4.5.2.1.6.2	Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell. Each interruption on NR PSCell shall not exceed 2ms (2 slot) as defined in TS38.133 section 8.2.1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.2.1.7	E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 30KHz SCS

A.4.5.2.1.7.1	Test Purpose and Environment

The purpose of this test is to verify NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.7.1-1 and A.4.5.2.1.7.1-2 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1 Cell2 and Cell3. Cell1 is LTE PCell on the TDD primary component, Cell2 and Cell 3 is NR PSCell and NR deactivated SCell on the FR1 TDD secondary component with 30KHz SCS. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. The point in time at which the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector, defines the start of time period T1. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.4.5.2.1.7.1-1: General test parameters for E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 30KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to cell1, cell 2 and cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.7.1-2: NR cell specific test parameters for E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 30KHz SCS

| Parameter | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 2 | 3 |
| Duplex mode |  | TDD | TDD |
| BWchannel | MHz | 10MHz | 10MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 TDD | - |
| CORESET parametersNote1: |  | CR.2.1 TDD | CR.2.1 TDD |
| OCNG PatternsNote1 |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.2 FR1 | SMTC.2 FR1 |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 17 | 17 |
| Note 3 | dB | 17 | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | 33 | 33 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

A.4.5.2.1.7.2	Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell. Each interruption on NR PSCell shall not exceed 0.5ms (1 slot) as defined in TS38.133 section 8.2.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.2.1.8	E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 30KHz SCS

A.4.5.2.1.8.1	Test Purpose and Environment

The purpose of this test is to verify NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.1.8.1-1 and A.4.5.2.1.8.1-2 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1 Cell2 and Cell3. Cell1 is LTE PCell on the FDD primary component, Cell2 and Cell 3 is NR PSCell and NR deactivated SCell on the FR1 TDD secondary component with 30KHz SCS. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. The point in time at which the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector, defines the start of time period T1. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.4.5.2.1.8.1-1: General test parameters for E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 30KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to cell1, cell 2 and cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 10 |  |

Table A.4.5.2.1.8.1-2: NR cell specific test parameters for E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 30KHz SCS

| Parameter | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 2 | 3 |
| Duplex mode |  | TDD | TDD |
| BWchannel | MHz | 10MHz | 10MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 TDD | - |
| CORESET parametersNote1: |  | CR.2.1 TDD | CR.2.1 TDD |
| OCNG PatternsNote1 |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.2 FR1 | SMTC.2 FR1 |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 17 | 17 |
| Note 3 | dB | 17 | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | 500 | 500 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

A.4.5.2.1.8.2	Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell. Each interruption on NR PSCell shall not exceed 2ms (2 slot) as defined in TS38.133 section 8.2.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

<< Start of Change #2>>

A.5.5.2.1.3	E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 120KHz SCS

A.5.5.2.1.3.1		Test Purpose and Environment

The purpose of this test is to verify NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.5.5.2.1.3.1-1 and A.5.5.2.1.3.1-2 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-2. In the test there are three cells: Cell1 Cell2 and Cell3. Cell1 is LTE PCell on the TDD primary component, Cell2 and Cell 3 is NR PSCell and NR deactivated SCell on the FR1 TDD secondary component with 120KHz SCS. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. The point in time at which the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector, defines the start of time period T1. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.5.5.2.1.3.1-1: General test parameters for E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 120KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to cell1, cell 2 and cell3 |
| AoA number |  | 1 | Applicable to cell2 and cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 10 |  |

Table A.5.5.2.1.3.1-2: NR cell specific test parameters for E-UTRAN TDD – NR TDD interruptions during measurements on deactivated NR SCC in synchronous EN-DC with 120KHz SCS

| Parameter | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 2 | 3 |
| Duplex mode |  | TDD | TDD |
| BWchannel | MHz | 100MHz | 100MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.3.1 TDD | - |
| CORESET parametersNote1: |  | CR.3.1 TDD | CR.3.1 TDD |
| OCNG PatternsNote1 |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 17 | 17 |
| Note 3 | dB | 17 | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | 33 | 33 |
| Time offset to cell2 Note 5 | s | - | 8 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: Receive time difference between slot boundaries of signals received from the two cells including time alignment error between the two cells. |  |  |  |

A.5.5.2.1.3.2		Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell. Each interruption on NR PSCell shall not exceed 0.625ms (5 slot) as defined in TS38.133 section 8.2.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.2.1.4	E-UTRAN FDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC with 120KHz SCS

A.5.5.2.1.4.1	Test Purpose and Environment

The purpose of this test is to verify NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The general test parameters and NR cell specific test parameters are given in Table A.5.5.2.1.4.1-1 and A.5.5.2.1.4.1-2 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-2. In the test there are three cells: Cell1 Cell2 and Cell3. Cell1 is LTE PCell on the FDD primary component, Cell2 and Cell 3 is NR PSCell and NR deactivated SCell on the FR1 TDD secondary component with 120KHz SCS. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. The point in time at which the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector, defines the start of time period T1. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.5.5.2.1.4.1-1: General test parameters for E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 120KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to cell1, cell 2 and cell3 |
| AoA number |  | 1 | Applicable to cell2 and cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 10 |  |

Table A.5.5.2.1.3.1-2: NR cell specific test parameters for E-UTRAN FDD – NR TDD interruptions during measurements on deactivated NR SCC in asynchronous EN-DC with 120KHz SCS

| Parameter | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 2 | 3 |
| Duplex mode |  | TDD | TDD |
| BWchannel | MHz | 100MHz | 100MHz |
| PSCell PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.3.1 TDD | - |
| CORESET parametersNote1: |  | CR.3.1 TDD | CR.3.1 TDD |
| OCNG PatternsNote1 |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 17 | 17 |
| Note 3 | dB | 17 | 17 |
| SS-RSRP Note 3 | dBm/15 KHz | -87 | -87 |
| SSB_RP Note 3 | dBm/15 KHz | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | 125 | 125 |
| Time offset to cell2 Note 5 | s | - | 8 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: Receive time difference between slot boundaries of signals received from the two cells including time alignment error between the two cells. |  |  |  |

A.5.5.2.1.3.2		Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell. Each interruption on NR PSCell shall not exceed 0.625ms (5 slot) as defined in TS38.133 section 8.2.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Change #2>>