3GPP TSG-RAN WG4 Meeting #88bis              	R4-1812303

Chengdu, CN, 8 Oct – 12 Oct 2018

Agenda item:	7.13.2.2

Source:	Samsung

Title:	Discussion and simulation results for NR PUSCH

Document for:	Discussion

# 1	Introduction

During RAN4# 88 meeting, the NR BS demodulation performance requirements are further discussed on the general issue and performance requirements test. Some general issue about NR PUSCH performance requirements are captured and agreed in the WF [1].

In this contribution, we provide our view on the remained issue of PUSCH performance requirement. Also, based on the agreed simulation assumption, the initial simulation results are provided for alignment purpose.

# 2	Discussion

## 2.1	Time domain resource allocation

NR defines two kinds of time domain resource allocation, type A and type B. The start symbol and symbol length can be configured in time domain as follows table 1:

Table 1: time domain resource allocation

|  | Start symbol | length |
| --- | --- | --- |
| Type A | 0 | {4,…,14} |
| Type B | {0,13} | {1,….14} |

Table 2: DMRS position with PUSCH type A and type B for different symbol length

| Duration in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| <4 | - | - | - | - |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 6 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 7 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 8 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 9 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 10 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 11 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 12 |  | , 9 | , 6, 9 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 13 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 14 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |

As indicated in the table 2, the number of DMRS and location are different with the length of scheduled data and PUSCH mapping type. For FR1, the duplex method of FDD, TDD and SUL are available. In terms of implementation point view, a front-loaded DMRS can help decoding latency by obtaining a fast channel estimate and performing sequential decoding of frequency-first mapped code blocks.  For the PUSCH mapping type A, the location of front-loaded DMRS L0 can be #2 or # 3.  For the PUSCH mapping type B, the first DMRS L0 is located within the first symbol with the related scheduled data, where the processing delay is less. In terms of channel estimation accuracy, DMRS location in Type A can achieve better estimation performance, considering the interpolation method in time domain can guarantee the better performance for edge symbols in scheduled PUSCH data.  Thus, our recommend is that the performance requirement of both PUSCH mapping type A and type B should be introduced in Rel-15 for FR1. However, considering the timeline of performance part, if both type A and type B included in Rel-15, the test cases will be duplicated. In terms of workload, we prefer type A with first priority. If time permits, we can introduce the limited number of test cases for type B. In case of slot based or not slot based method for type B, we prefer slot based with high priority.

Proposal 1: For FR1, the performance requirement of type A should be introduced in Rel-15 with high priority. If time permits, limited test cases can be introduced in Rel-15 for slot based transmission with type B.

For FR2, only available duplex method is TDD. Thus, non-slot based scheduling is preferred. As agreed in the last meeting, not-slot based transmission with type B is agreed. As for the number of data symbol, it depends on the detail configuration of TDD UL/DL configuration, the number of symbols for DL, and the number of SRS symbols. Normally, 12 and 10 UL data symbols are the typical scenarios for UL slot. For example, one symbol reserves for DCI , one symbol reserves for DL and UL switching, 12 symbols for UL data symbols, or two symbol reserves for DCI, one symbol reserves for DL and UL switching, 10 symbols for UL data, one symbol reserves for SRS. The default PUSCH time domain resource allocation A for normal CP can be indicated as Table 3.

Table 3, Default PUSCH time domain resource allocation A for normal CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

From this table, 12 or 10 UL data symbols are the majority cases.

As discussed in the last meeting, RAN4 group had listed many options for symbol length and symbol start position. Since the type B is agreed with FR2, in terms of performance, there is no different with different symbols start position.  Considering the reserved symbols for PDCCH and DL/UL switching, 2 is more reasonable. As for the symbol length, it depends on the through put and coding rate. Long symbol length can support high coding rate. Considering only 1 DMRS symbol is agreed, long symbol length can also check the impact of the channel estimation performance. Considering 3 symbols with reserved for PDCCH and DL/UL switching, 1 symbol reserved for SRS. We prefer the symbol length is 10.

Proposal 2: For FR2, not-slot transmission with PUSCH resource mapping type should be introduced in Rel-15.  Recommend symbol length is 10; data symbol index is 2 for performance requirement setting.

## 2.2	FRC table

Based on current agreement, we propose the FRC table as follows table 4 to table 6 for different BW, number of DMRS, respectively

Proposal 3: FRC Table for FR1 and FR2

Table 4, FRC table for FR1 with 1 DMRS

| CBW(MHz) | 10MHz | 10MHz | 10MHz | 40MHz | 40MHz | 40MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 30 | 30 | 30 |
| RB | 52 | 52 | 52 | 106 | 106 | 106 |
| Modulation order | 2 | 4 | 6 | 2 | 4 | 6 |
| MCS index | 2 | 16 | 20 | 2 | 16 | 20 |
| Code Rate | 1/5 | 2/3 | 1/5 | 1/5 | 2/3 | 1/5 |
| Num of DMRS | 1 | 1 | 1 | 1 | 1 | 1 |
| Channel bits | 16224 | 32448 | 48672 | 33072 | 66144 | 99216 |
| Final TBS (A) | 3104 | 21000 | 27144 | 6280 | 42016 | 55304 |
| CRC for TB (A>3824, 24; otherwise 16) | 16 | 24 | 24 | 24 | 24 | 24 |
| C (Num of CB) | 1 | 3 | 4 | 1 | 5 | 7 |
| Base Graph Type | BG2 | BG1 | BG1 | BG1 | BG1 | BG1 |

Table 5, FRC table for FR1 with 1 front-loaded DMRS and 1 additional DMRS

| CBW(MHz) | 10MHz | 10MHz | 10MHz | 40MHz | 40MHz | 40MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 30 | 30 | 30 |
| RB | 52 | 52 | 52 | 106 | 106 | 106 |
| Modulation order | 2 | 4 | 6 | 2 | 4 | 6 |
| MCS index | 2 | 16 | 20 | 2 | 16 | 20 |
| Code Rate | 1/5 | 2/3 | 1/5 | 1/5 | 2/3 | 1/5 |
| Num of DMRS | 1+1 | 1+1 | 1+1 | 1+1 | 1+1 | 1+1 |
| Channel bits | 14976 | 19952 | 44928 | 30528 | 61056 | 91584 |
| Final TBS (A) | 2856 | 19464 | 25104 | 5768 | 38936 | 50184 |
| CRC for TB (A>3824, 24; otherwise 16) | 16 | 24 | 24 | 24 | 24 | 24 |
| C (Num of CB) | 1 | 3 | 3 | 1 | 5 | 6 |
| Base Graph Type | BG2 | BG1 | BG1 | BG1 | BG1 | BG1 |

Table 6, FRC table for FR2 with 1 front-loaded DMRS

| CBW(MHz) | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 60 | 60 | 60 | 120 | 120 | 120 |
| RB | 132 | 132 | 132 | 66 | 66 | 66 |
| Modulation order | 2 | 4 | 6 | 2 | 4 | 6 |
| MCS index | 2 | 16 | 20 | 2 | 16 | 20 |
| Code Rate | 1/5 | 2/3 | 1/5 | 1/5 | 2/3 | 1/5 |
| Num of DMRS | 1 | 1 | 1 | 1 | 1 | 1 |
| Channel bits | 41184 | 82368 | 123552 | 20592 | 41184 | 61776 |
| Final TBS (A) | 7680 | 53288 | 67584 | 3848 | 26632 | 33816 |
| CRC for TB (A>3824, 24; otherwise 16) | 24 | 24 | 24 | 16 | 24 | 24 |
| C (Num of CB) | 1 | 7 | 9 | 1 | 4 | 5 |
| Base Graph Type | BG1 | BG1 | BG1 | BG1 | BG1 | BG1 |

# 3	Simulation Results

In this section, based on our preferred, the initial simulation results of PUSCH are provided as table 8, 9, 10. As agreed in WF [2] last meeting. Here, we just list the main simulation assumption as follows table 7.

Table 7: Simulation assumption for NR PUSCH for CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1,2 | 1 |
| Number of Rx | 2, 4,8 | 1,2 |
| Transmission scheme | Identity matrix (TMPI index 0) | n.a. |
| Number of layers | 1, 2 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1(3), 1+1(3,11) | 1 |
| symbols length | 14 | X |
| Time domain resource allocation type | type A | type B |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | TDL-A 30ns 10Hz | AWGN |
| SCS and BW | 15kHz: 10 30kHz: 40 | 60kHz: 100 120kHz: 100 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |

Based on the simulation assumption, the initial simulation can be indicated as the following tables, the remained will be provided later.

Table8: The target SNR performance for NR PUSCH for FR1 with 1 Tx with CP-OFDM

| Case | Tx/Rx | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) | Target SNR for MCS16 (dB) | Target SNR for MCS20 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T2Rx | 15 | 1 | 10 | -3.866 | 8.042 | 11.173 |
| 2 |  | 15 | 1+1 | 10 | -4.463 | 7.943 | 11.181 |
| 3 |  | 30 | 1 | 40 | -4.124 | 7.771 | 11.907 |
| 4 |  | 30 | 1+1 | 40 | -4.687 | 7.858 | 11.046 |
| 5 | 1Tx4Rx | 15 | 1 | 10 | -6.650 | 4.534 | 7.648 |
| 6 |  | 15 | 1+1 | 10 | -7.526 | 4.501 | 7.590 |
| 7 | 1Tx8Rx | 15 | 1 | 10 | -8.976 | 1.321 | 4.463 |
| 8 |  | 15 | 1+1 | 10 | -9.855 | 1.424 | 4.714 |

Table9: The target SNR performance for NR PUSCH for FR1 with 2 Tx with CP-OFDM (to be updated)

| Case | Tx/Rx | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) | Target SNR for MCS16 (dB) | Target SNR for MCS20 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2T2Rx | 15 | 1 | 10 |  |  |  |
| 2 |  | 15 | 1+1 | 10 |  |  |  |
| 3 | 2Tx4Rx | 15 | 1 | 10 |  |  |  |
| 4 |  | 15 | 1+1 | 10 |  |  |  |
| 5 | 2Tx8Rx | 15 | 1 | 10 |  |  |  |
| 6 |  | 15 | 1+1 | 10 |  |  |  |

Table10: The target SNR performance for NR PUSCH for FR1 with 1 Tx with DFT-OFDM

| Case | Tx/Rx | SCS | Number of DMRS symbol | BW | RB | Target SNR for MCS2 (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T2Rx | 30 | 1 | 40 | 50 | -3.823 |

# 4	Conclusion

In this contribution, based on the agreement of WF on the NR BS PUSCH demodulation, we provide our view about the remained issue of NR PUSCH demodulation requirement. Besides, based on our preferred and simulation assumption, the initial simulation results are provided for alignment purpose.

Proposal 1: For FR1, the performance requirement of type A should be introduced in Rel-15 with high priority. If time permits, limited test cases can be introduced in Rel-15 for slot based transmission with type B.

Proposal 2: For FR2, not-slot transmission with PUSCH resource mapping type should be introduced in Rel-15.  Recommend symbol length is 10; data symbol index is 2 for performance requirement setting.

Proposal 3: FRC Table for FR1 and FR2

# References

R4-1811694, WF on NR PUSCH demodulation requirements in Rel-15, Nokia