3GPP TSG-RAN WG4 Meeting #88bis               	R4-1812305

Chengdu, CN, 8 Oct – 12 Oct 2018

Agenda item:	7.13.2.4

Source:	Samsung

Title:	Discussion and simulation results for NR PRACH

Document for:	Discussion

# 1	Introduction

During RAN4 #88 meeting, the NR BS demodulation performance requirements are further discussed on the general issue and performance requirements test. Some general issue about NR PRACH performance requirements are captured and agreed in the WF [1] as follow:

- There are no test cases for restricted sets in Rel-15

- Preamble Format (For Rel-15)

  - For long sequence, requirements are defined for

- [Format 0]

  - For short sequence, requirements are

- [A1,A2,A3,B4,C0 and C2]

- SCS

  - Keep the previous agreement and cover 15kHz, 30kHz, 60kHz (FR2) and 120kHz for short sequence

  - Apply the performance requirements for short sequence based on BS declaration

- Timing offset

  - For conformance test, we reuse the LTE timing offset scheme for preamble transmission, i. e

- The timing offset base value is set to 50% of Ncs. This offset is increased within the loop, by adding in each step a value of [0.1us], until the end of the tested range, which is [0.9us]. Then the loop is being reset and the timing offset is set again to 50% of Ncs.

- Frequency offset

  - Frequency offset for FR1

- Option 1:500Hz

- Other options are not precluded

- Logical sequence index

  - Option 1:  22 for long sequence, 0 for short sequence

  - Other options not precluded

- Ncs

  - Long sequence: 13

  - Short sequence

- Option 1: 46  for FR1, 0 for FR2

- Option 2 : 23 for FR1, 69 for FR2

- Option 3:  0 for FR1, 0 for FR2

- Other options are not precluded

- Intension next meeting to narrow to one option for requirement

In this contribution, based on above agreement, we provide our view on the remained issues for the performance requirement of NR PRACH. Also, the initial simulation results are provided for alignment purpose.

# 2	Discussion

For PRACH format with short sequence, there are four SCS for each format. Considering that there are four different SCS scheme for each PRACH format with short sequence, we have at least about 40 formats with all the SCS and PRACH format combination. Aiming to reduce of payloads, our preference is that 15 KHz, for FR1, and 120 KHz for FR2.

According to previous agreements, preamble formats with SCS of 15/30 kHz are used only for below 6 GHz, and on the other hand preamble formats with SCS of 60/120 kHz are used only for above 6 GHz. 15KHz is using for LTE reframing. 120 KHz is most typical scenario, which will be deployed. In terms of receiver complexity, it may be beneficial that FFT processing for reception of RACH preamble is operated in a unit of OFDM symbol aligned with PUSCH or PUCCH.

Based on above analysis, our initial preferred subset for combination of preamble formats and SCS for FR1 and FR2 is provided in Table 1.

Proposal 1: Recommend combination of preamble formats and SCS for performance requirement in Rel-15

Table 1 Recommended combination of preamble formats and SCS

|  | Burst format | SCS(kHz) |
| --- | --- | --- |
| FR1 | 0 | 1.25 |
|  | A1 | 15 |
|  | C2 | 15 |
| FR2 | A1 | 120 |
|  | C2 | 120 |

In LTE RACH, each cell has its own unique set of preambles generated from a pre-defined set of 64 Zadoff-Chu sequences.  The set is first constructed from cyclic-shifts of a single root sequence, and then by cyclic shifts of consecutive roots if necessary. The cyclic shift offset NCS must be selected to allow unambiguous round-trip time estimation at the eNB in the worst case when the UE is located at the cell edge including the expected maximum delay spread, i.e.,

NCS≥2×CellRadius3×108+DelaySpread×NZC×ΔfRA+ng

where the PRACH subcarrier spacing  ΔfRA determines the sequence length TSEQ=1/ΔfRA and ng are additional guard samples due to the receiver pulse shaping filter.

Normally, larger cell size requires a larger cyclic shift in order to generate orthogonal sequences, and consequently, a higher number of ZC root sequences is necessary in order to provide the 64 required preambles. While for short sequence, the cell size is small than LTE, So, we prefer Ncs =0 for the short sequence, especially in FR2, considering the CP length is very short. For FR1, Ncs=23 is preferred.

Proposal 2: Recommend Ncs=0 for the performance requirement of PRACH with short sequence format in FR2, and Ncs=23 in FR1.

# 3	Simulation Results

In this section, the initial simulation results for NR PRACH are provided as shown in Table 2 and Table 3 for FR1 and FR2.

Table 2: Simulation assumption for NR PRACH in FR1

| Parameters | Format0 | Format A1 | Format A2 | Format A3 | Format B4 | Format C0 | FormatC2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz | 10MHz, 40MHz |
| SCS | 1.25KHz | 15KHz, 30KHz | 15KHz 30KHz | 15KHz 30KHz | 15KHz 30KHz | 15KHz 30KHz | 15KHz 30KHz |
| Rx | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Propagation channel | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Logical sequence index | 22 | 0 | 0 | 0 | 0 | 0 | 0 |
| Preamble index | 32 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ncs | 13 | 23 | 23 | 23 | 23 | 23 | 23 |
| Set type | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted |

Table 3: Simulation assumption for NR PRACH in FR2

| Parameters | Format A1 | Format A2 | Format A3 | Format B4 | Format C0 | FormatC2 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| SCS | 60KHz, 120KHz | 60KHz 120KHz | 60KHz 120KHz | 60KHz 120KHz | 60KHz 120KHz | 60KHz 120KHz |
| Rx | 2 | 2 | 2 | 2 | 2 | 2 |
| Propagation channel | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Logical sequence index | 0 | 0 | 0 | 0 | 0 | 0 |
| Preamble index | 0 | 0 | 0 | 0 | 0 | 0 |
| Ncs | 69 | 69 | 69 | 69 | 69 | 69 |
| Set type | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted | Unrestricted |

As for test metric, we follow the legacy way defined in LTE. Both the false alarm probability and missed detection probability are given as the performance requirement for NR PRACH. The initial simulation results can be referred as Table 4 and Table 5 in FR1 and FR2, respectively.

Table 4: Simulation results for NR PRACH in FR1 (to be updated)

| Format | SCS&BW | Missed detection probability (<=1%) |
| --- | --- | --- |
| Format 0 | 1.25KHz, 10MHz | -16.44 |
| Format 0 | 1.25KHz, 40MHz | -16.48 |
| Format A1 | 15KHz, 10MHz | -10.86 |
| Format A2 | 15KHz, 10MHz | -13.27 |
| Format A3 | 15KHz, 10MHz |  |
| Format B4 | 15KHz, 10MHz |  |
| Format C0 | 15KHz, 10MHz |  |
| Format C2 | 15KHz, 10MHz | -13.28 |
| Format A1 | 30KHz, 40MHz | -10.88 |
| Format A2 | 30KHz, 40MHz | -13.31 |
| Format A3 | 30KHz, 40MHz |  |
| Format B4 | 30KHz, 40MHz |  |
| Format C0 | 30KHz, 40MHz |  |
| Format C2 | 30KHz, 40MHz | -13.31 |

Table5: Simulation results for NR PRACH in FR2

| Format | SCS&BW | Missed detection probability (<=1%) |
| --- | --- | --- |
| Format A1 | 60KHz, 100MHz | -10.6 |
| Format A2 | 60KHz, 100MHz | -13.2 |
| Format A3 | 60KHz, 100MHz | -14.5 |
| Format B4 | 60KHz, 100MHz | -16.5 |
| Format C0 | 60KHz, 100MHz | -8.1 |
| Format C2 | 60KHz, 100MHz | -13.0 |
| Format A1 | 120KHz, 100MHz | -10.6 (Ncs =69) |
| Format A1 | 120KHz, 100MHz | -10.3 (Ncs = 0) |
| Format A2 | 120KHz, 100MHz | -13.2 |
| Format A3 | 120KHz, 100MHz | -14.5 |
| Format B4 | 120KHz, 100MHz | -16.5 |
| Format C0 | 120KHz, 100MHz | -8.1 |
| Format C2 | 120KHz, 100MHz | -13.0 (Ncs = 69) |
| Format C2 | 120KHz, 100MHz | -13.0 (Ncs =0) |

From both tables, Format B4 can achieve better performance compared with all the short sequence. Format C2 and A2 has the similar performance, C0 has the worst performance.

# 3	Conclusion

In this contrition, we provide our view about the remained issue on performance requirement of NR BS PRACH.

Proposal 1: Recommend combination of preamble formats and SCS for performance requirement in Rel-15

Table 1 Recommended combination of preamble formats and SCS

|  | Burst format | SCS(kHz) |
| --- | --- | --- |
| FR1 | 0 | 1.25 |
|  | A1 | 15 |
|  | C2 | 15 |
| FR2 | A1 | 120 |
|  | C2 | 120 |

Proposal 2: Recommend Ncs=0 for the performance requirement of PRACH with short sequence format in FR2, and Ncs=23 in FR1.

# References

R4-1811728, WF on NR PRACH performance requirement. Ericsson