3GPP TSG-RAN WG4 Meeting #88bis	R4-1812454

Chengdu, China, 8th – 12th Oct, 2018

Title:	LS on MaxNumberRxBeam

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

CC:	RAN WG 1

Contact Person:

#### Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N/A

1. Overall Description:

In RAN4, for defining the evaluation period in FR2 for Radio link monitoring, Beam failure detection and Candidate Beam detection, a scale factor of N is used for Rx beam sweeping.

For instance, the evaluation period for BFD in FR2 for non-DRX case is defined as

TEvaluate_BFD_CSI-RS = max([50], [MBFD *P*N] * TCSI-RS)

Where MBFD and P are defined in TS 38.133, N is still TBD.

RAN4 would like to suggest extending the scope of MaxNumberRxBeam to also indicate the Rx beam number of CSI-RS for Radio link monitoring, Beam failure detection and Candidate Beam detection and then RAN4 can finalize the corresponding requirement with scale factor N=MaxNumberRxBeam .

2. Actions:

To RAN WG2:

RAN4 respectfully asks RAN2 to confirm whether above suggestion from RAN4 is feasible or not.

3. Date of Next RAN4 Meetings:

3GPP RAN4 #89 		12 - 16 Nov 2018, 		Spokane, US

3GPP RAN4 #90 		25 Feb - 1 Mar 2019, 		Athens, GR