3GPP TSG-RAN WG4 Meeting #88bis	R4-1812455

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	7.13.1.3

Source: 	Intel Corporation

Title: 	Discussion on UE demodulation requirements for PDCCH

Document for:	Discussion

# Introduction

In RAN4#88 PDCCH demodulation requirements were discussed and the way forward and test cases for simulation alignment were agreed [1]. In [2] we provide simulation results for the agreed test cases. In this paper we provide our views on some of the open issues related to PDCCH test cases and propose test case for PDCCH demodulation.

# Discussion

In [2] we provide simulation results for alignment for the test cases discussed and agreed in RAN4#88 [1]. The open items that need to be addressed are:

- Test case with 1 symbol PDCCH

- Test cases with AL=16

- Test cases with 4Rx with interleaved CCE-to-REG mapping

## Test cases with 1 symbol PDCCH

In RAN4#88 it was agreed to introduce testcases with symbol PDCCH. Based on the results for the test cases used to evaluate performance with 1 symbol PDCCH, we propose the following test case:

Table 1: Proposed testcase for 1 symbol PDCCH

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config with 2Rx | Antenna config with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12a | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |

Proposal #1: Introduce 1 symbol PDCCH testcase with 15KHz SCS

## Test cases with AL 16

Table 2: PDCCH Results with AL=16

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | -3.99 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low | -6.64 |
| 16 | 120 | 1_0 | [60] | 40 | 2 | 16 | Interleaved | 2 | TDL-A, 30ns, 75Hz | 2x2 Low | -5.64 |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | -8.20 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low | -9.59 |

From the results with AL=16, we see that the operating SNR is very low, especially with 4Rx antenna. With 2 Rx antenna SNR conditions below -6dB SNR might trigger handover. Test #14 has a reasonable operating SNR and could be used to introduce test with AL=16 with 2Rx.

Proposal #2: Introduce testcase with reasonable operating SNR with AL=16 and 2Rx

## 4Rx Testcases with Interleaved CCE-to-REG mapping

In RAN4#88 4Rx testcases with Interleaved CCE-to-REG mapping were discussed. The REG bundle size for interleaved mapping can be {6, 2}, for 1 or 2 symbol CORESET. While REG bundling size of 2 gives more diversity in mapping, it also reduces the number of DMRS samples available for channel and noise estimation.  The receiver assumption for PDCCH is MMSE-IRC. For interference and noise limited conditions we evaluate the performance of MMSE-IRC with REG bundle size of 2 and 6. We also compare the performance with MMSE receiver.

In figures below we show the performance of test cases with 4Rx and interleaved mapping in noise and interference limited scenarios. For interference limited scenario the INR is 10dB.

| 30KHz SCS; 40MHz; TDL-A-30-10Hz; AL:2; 1x4 |  |
| --- | --- |

Figure 1: Test 6 Performance in Noise vs Interference limited conditions

| 30KHz SCS; 40MHz; TDL-C-300-100Hz; AL:4; 2x4 |  |
| --- | --- |

Figure 2: Test 7 Performance in Noise vs Interference limited conditions

Table 3: Results with 4Rx and Interleaved mapping

| Test No. | SCS(KHz) | Format | Payload | AL | Prop. condition | Antenna config | SNR @ 1% Pm-dsg In Noise limited |  |  | SINR @ 1% Pm-dsg In interference limited |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MMSE-IRC REG Bundle:2 | MMSE-IRC REG Bundle:6 | MMSE REG Bundle:2 | MMSE-IRC REG Bundle:2 | MMSE-IRC REG Bundle:6 | MMSE REG Bundle:2 |
| 6 | 30 | 1_0 | 41 | 2 | TDL-A, 30ns, 10Hz | 1x4 Low | 3.50 | -1.09 | -0.11 | 3.87 | -2.92 | 3.67 |
| 7 | 30 | 1_1 | 52 | 4 | TDL-C, 300ns, 100Hz | 1x4 Low | -0.09 | -4.20 | -3.32 | 2.05 | -1.73 | 1.31 |

In noise limited conditions we see that performance with 4Rx can be improved by either using a MMSE receiver or by using REG bundle size of 6. In interference limited scenario, the performance with MMSE receiver is also poor, since interference is not being mitigated. In field testing we cannot guarantee a noise free scenario and with MMSE-IRC receiver performance improvement is achieved with REG bundle size of 6. For 4Rx test cases, we recommend using REG bundle size of 6 for interleaved and non-interleaved CCE-to-REG mapping.

Proposal #3: Define PDCCH test cases with interleaved CCE-to-REG mapping for 4Rx with REG bundle size of 6

## PDCCH Testcases

Based on the discussion points above, we propose the following test cases for PDCCH:

Tests cases with 2Rx:

Table 4: PDCCH Test cases with 2Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x2 Low |
| 8 | 30 | 1_1 | 90 | 52 | 1 | 8 | Interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low |
| 9 | 120 | 1_0 | 60 | 40 | 1 | 2 | Interleaved | 2 | [TDL-A, 30ns, 75Hz] | 1x2 Low |
| 10 | 120 | 1_1 | 60 | 51 | 1 | 4 | Interleaved(1) | 6 | [TDL-A, 30ns, 300Hz] | 1x2 Low |
| 11 | 120 | 1_1 | 60 | 51 | 1 | 8 | Interleaved | 2 | [TDL-A, 30ns, 75Hz] | 2x2 Low |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low |
| 13 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low |
| Note1: Interleaver size=2 |  |  |  |  |  |  |  |  |  |  |

Test Cases with 4Rx

Table 5: PDCCH Test cases with 4Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low |
| 8 | 30 | 1_1 | 90 | 52 | 1 | 8 | Interleaved(1) | 6 | TDL-C, 300ns, 100Hz | 2x4 Low |
| 9 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| Note1: Interleaver size=2 |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution we present our views on the open items related to PDCCH testcases and propose PDCCH test cases for demodulation requirements. Our proposals are summarized below.

Proposal #1: Introduce 1 symbol PDCCH testcase with 15KHz SCS

Proposal #2: Introduce testcase with reasonable operating SNR with AL=16 and 2Rx

Proposal #3: Define PDCCH test cases with interleaved CCE-to-REG mapping for 4Rx with REG bundle size of 6

Proposal #4: Define the following testcases for PDCCH demodulation requirements with 2Rx:

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size |  |  | Prop. condition | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved |  | 6 | TDL-A, 30ns, 10Hz |  | 1x2 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved |  | 6 | TDL-C, 300ns, 100Hz |  | 2x2 Low |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved |  | 6 | TDL-C, 300ns, 100Hz |  | 1x2 Low |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved |  | 6 | TDL-A, 30ns, 10Hz |  | 1x2 Low |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved |  | 6 | TDL-C, 300ns, 100Hz |  | 2x2 Low |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved |  | 2 | TDL-A, 30ns, 10Hz |  | 1x2 Low |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved |  | 2 | TDL-C, 300ns, 100Hz |  | 1x2 Low |
| 8 | 30 | 1_1 | 90 | 52 | 1 | 8 | Interleaved |  | 6 | TDL-C, 300ns, 100Hz |  | 2x2 Low |
| 9 | 120 | 1_0 | 60 | 40 | 1 | 2 | Interleaved |  | 2 | [TDL-A, 30ns, 75Hz] |  | 1x2 Low |
| 10 | 120 | 1_1 | 60 | 51 | 1 | 4 | Interleaved(1) |  | 6 | [TDL-A, 30ns, 300Hz] |  | 1x2 Low |
| 11 | 120 | 1_1 | 60 | 51 | 1 | 8 | Interleaved |  | 2 | [TDL-A, 30ns, 75Hz] |  | 2x2 Low |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved |  | 6 | TDL-A, 30ns, 10Hz |  | 1x2 Low |
| 13 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved |  | 6 | TDL-A, 30ns, 10Hz |  | 1x2 Low |
| Note1: Interleaver size=2 |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal #5: Define the following testcases for PDCCH demodulation requirements with 4Rx:

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low |
| 8 | 30 | 1_1 | 90 | 52 | 1 | 8 | Interleaved(1) | 6 | TDL-C, 300ns, 100Hz | 2x4 Low |
| 9 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low |
| Note1: Interleaver size=2 |  |  |  |  |  |  |  |  |  |  |

# Reference

- R4-1811724, “Way forward on NR PDCCH demodulation requirements”, Ericsson

- R4-1812175, “Simulation Results for PDCCH”, Intel Corporation